3GPP TSG RAN WG4 Meeting #114-bis		R4-2504577

Wuhan, China, April 7th-11th, 2025

Agenda item:	7.14.3.1

Source:	Telecom Italia S.p.A., Vodafone

Title:	Proposals for Rel-19 TRP TRS requirements topic

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#114 meeting, discussion on Rel-19 TRP TRS requirements related to the WI [1] was held which outcomes and agreements have been captured in the WF [2]. This contribution provides proposals related to some of the issues captured in the WF.

# 2	1Tx FR1 TRP TRS requirements

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (size 2)

Way Forward:

  - RAN4 further discuss how to specify PDA hand related TRP/TRS requirements (size 2).

The following observations can be made:

Observation 1: The number of size 2 devices present in the market is quite low. To perform a measurement campaign that can provide the same statistical confidence of Rel-18 (i.e., 79 different size 1 models), would imply to measure all the size 2 devices currently available on the market. This approach is not feasible.

Observation 2: At the same time, performing the measurement campaign over a restricted number of size 2 devices will not guarantee the statistical confidence to derive the minimum performance requirements.

Observation 3: In the context of ETSI MSG TFES activities [3][4][5], LTE minimum performance requirements for size 1 devices have been derived from the size 2 devices through the offsets specified below [6] and which will be included in the ETSI EN 301-908-13 V13.4.1:

Based on the above observations, it is proposed the following:

Proposal 1: Define the size 2 devices minimum performance requirements based on an offset of the Rel-18 values.

Proposal 2: For TRP minimum performance requirements, the offset is proposed to be 0,5 for bands n1, n41, and n78, and 0,25 for band n28 (i.e., requirements are tightened). For TRS minimum performance requirements, the offset is proposed to be -0,5 for bands n1, n41, and n78, and -0,25 for band n28 (i.e., requirements are tightened).

Issue 4-2-3: band number for 1Tx requirements in Rel-19

Agreements:

  - RAN4 further discuss how to select 4 bands from n3, n5, n7, n8, and n20, encourage companies to provide feedback by RAN4 #114bis on available number of devices each band and provide devices to test labs for data collecting.

The following observations can be made:

Observation 4: Operators are investigating internally the number of devices that can be provided for the measurement campaign. A clearer view is expected to be provided at the RAN4#115 meeting in Malta.

Observation 5: European operators preferred bands (i.e., n3, n7, n8, and n20) were provide for the revision of the WID in December 2024 [1]. To accommodate also n5 from Asian operators, the bands proposed by European operators should be down selected to 3.

Observation 6: In the context of ETSI MSG TFES activities, as per ETSI EN 301-908-13 V13.3.1 [7], LTE minimum performance requirements for B20 and B28 have been defined with the same value.

Based on the above observations, it is proposed the following:

Proposal 3: Define the minimum performance requirements for n20 equal to the n28 values specified in TS 38.161 [8] (for TRP, the value that will be adopted in March 2026 applies). Define the minimum performance requirements for n3, n5, n7, and n8 through the measurement campaign.

Issue 4-2-5: How to specify 1Tx requirements in Rel-19

Way Forward:

- The working procedure in Rel-18 on how to specify the requirement can be a baseline.

The following observations can be made:

Observation 7: The working procedure of Rel-18 doesn’t provide any guidance on how to manage values of the data set that are technically questionable and that can derive from a wrong setup of the measurement environment and/or a faulty device sample.

Observation 8: In both cases, impact on the provided results that would affect the minimum performance requirements definition are possible since the CDF approach is negatively impacted by outlier (i.e., very low) values.

Based on the above observations, it is proposed the following:

Proposal 4: For anomalous measurements values, any 3GPP member has the rights to require that i) the device can be measured again (to remedy the case of a wrong measurement setup of the laboratory) and/or ii) replaced with another sample of the same model (to remedy the case of a faulty device). If such options will not be feasible within the time frame of the WI, the anomalous measurement values will be removed from the data set.

# 3	Conclusions

In the following, the observations and proposals of the contribution are summarized:

Observation 1: The number of size 2 devices present in the market is quite low. To perform a measurement campaign that can provide the same statistical confidence of Rel-18 (i.e., 79 different size 1 models), would imply to measure all the size 2 devices currently available on the market. This approach is not feasible.

Observation 2: At the same time, performing the measurement campaign over a restricted number of size 2 devices will not guarantee the statistical confidence to derive the minimum performance requirements.

Observation 3: In the context of ETSI MSG TFES activities [3][4][5], LTE minimum performance requirements for size 1 devices have been derived from the size 2 devices through the offsets specified below [6] and which will be included in the ETSI EN 301-908-13 V13.4.1:

Proposal 1: Define the size 2 devices minimum performance requirements based on an offset of the Rel-18 values.

Proposal 2: For TRP minimum performance requirements, the offset is proposed to be 0,5 for bands n1, n41, and n78, and 0,25 for band n28 (i.e., requirements are tightened). For TRS minimum performance requirements, the offset is proposed to be -0,5 for bands n1, n41, and n78, and -0,25 for band n28 (i.e., requirements are tightened).

Observation 4: Operators are investigating internally the number of devices that can be provided for the measurement campaign. A clearer view is expected to be provided at the RAN4#115 meeting in Malta.

Observation 5: European operators preferred bands (i.e., n3, n7, n8, and n20) were provide for the revision of the WID in December 2024 [1]. To accommodate also n5 from Asian operators, the bands proposed by European operators should be down selected to 3.

Observation 6: In the context of ETSI MSG TFES activities, as per ETSI EN 301-908-13 V13.3.1 [7], LTE minimum performance requirements for B20 and B28 have been defined with the same value.

Proposal 3: Define the minimum performance requirements for n20 equal to the n28 values specified in TS 38.161 [8] (for TRP, the value that will be adopted in March 2026 applies). Define the minimum performance requirements for n3, n5, n7, and n8 through the measurement campaign.

Observation 7: The working procedure of Rel-18 doesn’t provide any guidance on how to manage values of the data set that are technically questionable and that can derive from a wrong setup of the measurement environment and/or a faulty device sample.

Observation 8: In both cases, impact on the provided results that would affect the minimum performance requirements definition are possible since the CDF approach is negatively impacted by outlier (i.e., very low) values.

Proposal 4: For anomalous measurements values, any 3GPP member has the rights to require that i) the device can be measured again (to remedy the case of a wrong measurement setup of the laboratory) and/or ii) replaced with another sample of the same model (to remedy the case of a faulty device). If such options will not be feasible within the time frame of the WI, the anomalous measurement values will be removed from the data set.

# 4	References

- RP-243277, “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, December 2024

- R4-2502370, “Way Forward for [114][327] TRP_TRS_Ph3”, vivo, February 2025

- TFES(24)000003, “TRP analysis of Nordic mobile phone antenna performance 2023”, Huawei, February 2024

- TFES(24)000004, “The discussion on the LTE OTA parameters of device above 72mm”, vivo, February 2024

- TFES(24)002002, “OTA limit proposal for devices wider than 72mm”, Huawei, vivo, Honor, Xiaomi, March 2024

- TFES(24)080023r1, “UE LTE OTA parameters - status and agreements”, Chair, June 2024

- ETSI EN 301-908-13 V13.3.1, “Harmonised Standard for access to radio spectrum; Part 13: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE)”, October 2024

- 3GPP TS 38.161 V18.3.0, “User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios (Release 18)”, December 2024