3GPP TSG-RAN WG4 Meeting #114bis							         		R4-2504610

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Initial power boosting result for NR-NTN for a PC2 NR PA

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.10.2.2.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In R4#114 we proposed to study the power boosting options for NR-NTN based on a PC2 1Tx NR PA in [1]. A way forward [2] captured a number of agreements on NB-IoT NTN and eMTC-NTN HPUE work that are the basis of the present work. In this contribution, we provide preliminary measurement power boosting results on a PC2 1Tx PA. Further investigations will be needed as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow the maximum boosting and more power should be achievable.

# Discussion

## 2.1 Way forward input

The way forwards [2] captured an agreement on NR-NTN HPUE topics:

Issue 2-1: NR-NTN PC2

Agreement: Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible: 1Tx handheld and non-handheld, 2Tx handheld and non-handheld,

- If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19

Issue 2-3: NR-NTN PC1.5

Agreement: Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. FFS on other cases if time is allowed

We performed power boosting measurements on a PC2 1Tx Mid-band PA calibrated for 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveform. Further investigations will be needed as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow the maximum boosting and more power should be achievable.

## 2.2 Measurement Results

The measurement results are for a limited number of NR waveforms:

- 20MHz QPSK CP and DFT waveforms covering inner, outer, and inner third waveforms as discussed in [1].

- Transceiver impairments were set at the nominal NR 28dB and the 34dB 256QAM image and carrier leakage level.

- The power levels are at the antenna output assuming 4dB post PA losses as the PA is designed for PC2 1Tx NR power with up to 4dB post PA losses (30dBm 0dB MPR level at the PA output).

Table 1 provides the NR 31dB PC2 ACLR and SEM margins of the measured waveforms per output power, current increase vs MPR0 and for 28dB and 34dB levels of impairments:

- SEM0/1/2/3/4 stands for SEM regions with respective limits of -13dBm/1%CBW/-10dBm/MHz/-13dBm/MHz/-25dBm/MHz/-30dBm/MHz.

- Dark green corresponds to pass, light green equates to a just pass, orange is a just failed and red is failed.

- In the boost column, dark green is an all pass with margin and highest power. Light green a pass but with little margin or lower power.

Table 1: UTRA ACLR margin versus PA output power for various NB-IoT waveforms

Observations:

- All inner third waveforms (in blue) pass with good margin regardless of the impairment level.

- Small allocations like 4RB2 also passe with margin and have improved margin with improved impairments.

- Even 1RB edge allocation passes with improved impairments but it may not be the case with relaxed windowing filters.

- The power was limited due to external current limitation on the PA and it should be feasible to reach higher power once those limitations are controlled.

- Provided that in-band requirement do not create further limitations and assuming smaller post PA losses for large duplex gap NTN FDD case, PC1.5 power (29dBm) 0dB MPR might be reachable as some waveforms already reach 28.5dBm and still have margin.

Proposal: To start a full power boosting evaluation on a PC2 1Tx PA to achieve PC1.5 power it is important that we agree on key parameters:

- Image and carrier leakage: we propose to use the NR agreed level for 256QAM at 34dB to improve the power capability for outer waveforms.

- For highest power boosting we propose to focus on inner third waveforms with LCRB<NRB/3 and sitting at least NRB/3 from the edge.

- We also encourage agreeing upon a lower post PA loss for the NTN bands that have a large duplex gap.

# Conclusion

In this contribution, we share a preliminary set of NR-NTN HPUE results and make the following proposals to start a complete power boosting evaluation campaign based on a PC2 1Tx NR PA.

Proposal: To start a full power boosting evaluation on a PC2 1Tx PA to achieve PC1.5 power it is important that we agree on key parameters:

- Image and carrier leakage: we propose to use the NR agreed level for 256QAM at 34dB to improve the power capability for outer waveforms.

- For highest power boosting we propose to focus on inner third waveforms with LCRB<NRB/3 and sitting at least NRB/3 from the edge.

- We also encourage agreeing upon a lower post PA loss for the NTN bands that have a large duplex gap.

# References

[1] R4-2500090 Candidate TN PA for NR-NTN HPUE in smartphones, Skyworks Solutions Inc., R4#114

[2] R4-2502954 WF on HPUE requirements for NR-NTN, Samsung, R4#114