3GPP TSG-RAN WG4 Meeting #114-bis                                        R4-2504619

# Wuhan, China, 7th – 11th of April 2025

Title: 	UE RF requirements for NTN NB-IoT TDD

Agenda Item:	7.31.4 – UE RF requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# For information

As described in the approved WID RP-242415 for New WID on introduction of IoT-NTN TDD mode (Iridium, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024), the study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Decisions from previous RAN4#113 IoT_NTN_TDD WF:

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analysing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

Decisions from RAN plenary, December 2024, Madrid (WID update):

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

Current Work Plan from R4-2502291 (from RAN4#114):

| RAN#107 (Mar. 2025) | RAN1#120bis | 0.5 | - Discuss remaining aspects on the TDD support in NB IoT NTN - Discuss (if needed) related RRC parameters and send LS to RAN2 - Discuss Draft CRs |
| --- | --- | --- | --- |
|  | RAN2#129bis | 0.5 | - Further discuss and down-select solutions to support TDD for NB IoT NTN - Conclude on potential impacts on higher layers - Start the definition and configuration of signalling of the periodic pattern, and associated UE procedures (if any) |
|  | RAN4#114bis | 0.5+0.25 | - Discuss the UE AMPR, UE frequency error requirement, - Discuss repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN - Discuss (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration. - Discuss (if needed) the equations to be used to derive (at least) above mentioned requirements based on different FRC configurations discussed. - Discuss work split for UE and SAN RF part (TS 36.102, TS 36.108 and prepare TS 36.181 work split according to potential impacts) |
|  | RAN1#121 | 0.5 | - Conclude on the solutions for the TDD support in NB IoT NTN - Further discuss (if needed) related RRC parameters and send LS to RAN2 - Finalize RAN1 CRs |
|  | RAN2#130 | 0.5 | - Complete the definition and RRC configuration of signaling of the periodic pattern, and associated UE procedures (if any) - Conclude support of TDD mode in NB IoT NTN - Discuss stage-2 design of TDD support in NB IoT NTN - Discuss RAN2 CRs |
|  | RAN4#115 | 0.5+0.25 | - Continue discussion on the UE AMPR, UE frequency error requirement, - Continue discussion on repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN, - Continue discussion (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration, - Derive related values for the at least for previous mentioned requirements: REFSENS, in-channel selectivity, dynamic range, etc., - Start discussion on UE and SAN CRs for RF part and SAN conformance testing, - Start CR split to prepare Draft CRs for the Core requirements |

Previous agreements from WF R4-2502292 at RAN4#114 (“Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES) were:

Issue 2-1-1: Discussion on UE REFSENS for IoT TDD in TS 36.102

Agreement: Reuse REFSENS -108.2dBm for the new TDD NTN NB-IoT operating band.

Issue 2-1-2: Clause 7.3B Update - Reference sensitivity power level for UE category NB1 and NB2

Agreement: RAN4 to adopt the following modification in the TS 36.102:

## 7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 for FDD and TDD UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration.

Table 7.3B-1: Reference sensitivity for UE FDD and TDD category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

Issue 2-1-3: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss AMPR for 249. Proponent to clarify at next meeting which band has to be protected through this AMPR mechanism.

Issue 2-1-4: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss the potential update of frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot/segment.

Moderator note1: further clarify if UL slot is the LTE Tx slot or is for the Tx transmission time U=8. In both cases, the issue seems to be implementation-related.

Moderator note2: [Clause 6.4B.1of TS36.102] “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.”

Moderator note3: According to the 2nd note, it seems that TDD UE behaviour will be similar to FDD UE behaviour over different segment transmissions. Also, the current FRC in UL for FDD seem to indicate much higher transmissions times than 8ms (proposed for TDD frame).

Issue 2-1-5: Discussion on EVM clause for NB-IoT TDD NTN in TS 36.102

Agreement: Add for EVM clause “For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

The RAN4#114-bis meeting agenda is:

7.31	Introduction of IoT-NTN TDD mode	[IoT_NTN_TDD]

7.31.1	General aspects	[IoT_NTN_TDD]

7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)	[IoT_NTN_TDD-Core]

7.31.3	SAN RF requirements	[IoT_NTN_TDD-Core]

7.31.4	UE RF requirements	[IoT_NTN_TDD-Core]

7.31.5	RRM core requirements	[IoT_NTN_TDD-Core]

7.31.6	Moderator summary and conclusions

# UE Requirements for IoT_NTN_TDD

Please also recall the agreements from the previous WF at RAN#113:

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

## A-MPR discussion

Observation 1: For a better representation, the reader shall use the following figure for information purposes:

Figure 1: Current NTN NB-IoT L-band representation (NTN band 249)

Tx/Rx requirements have been also previously discussed:

Observation 2: Unless stated otherwise, reuse TS 36.102 NTN IoT 254 L-band Tx requirements also for 249 band in TS 36.102.

Observation 3: Unless stated otherwise, reuse TS 36.102 NTN IoT 255 L-band Rx requirements also for 249 band in TS 36.102.

The current 254 band definition does not include any A-MPR definition for NB1/NB2:

### 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise. For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1. Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE Network Signalling value; Requirements (subclause); E-UTRA Band; A-MPR (dB). NS_01; 6.5B.4.2; Table 5.2-1; N/A. NS_02N; 6.5B.4.4.2; 255; N/A. NS_03N; 6.5B.4.4.4; 254; N/A. NS_04N; 6.5B.4.4.5; 254; TBD. NS_05N; 6.5B.4.4.6; 254; TBD. NS_24; 6.5B.4.4.3; 256; PC3; PC5. ≤ 3.5; ≤ 0.5. ### 6.2B.4 Configured transmitted Power for category NB1 and NB2 For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply, wherein - The Maximum output power requirements are specified in subclause 6.2B.1 - The MPR requirements are specified in subclause 6.2B.2 - The A-MPR requirements are specified in subclause 6.2B.3.

With

##### 6.5B.4.4.4 Minimum requirement (network signalled value "NS_03N") When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz. ##### 6.5B.4.4.5 Minimum requirement (network signalled value "NS_04N" and "NS_05N") When "NS_04N" or "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz. ##### 6.5B.4.4.6 Void

And with:

##### 6.5A.4.4.4 Minimum requirement (network signalled value "NS_03N") When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth. Table 6.5.4.4.4-1: Additional out-of-band requirements for "NS_03N" Frequency band (MHz); Channel bandwidth / Spectrum emission limit1 (dBm); Measurement bandwidth; NOTE. BWchannel. 1559 ≤ f ≤ 1605; -50; 700 Hz; Discreet emissions averaged over any 2 millisecond active transmission interval. 1605 ≤ f ≤ 1610; -50 + 60/5 (f-1605); 700 Hz. 1559 ≤ f ≤ 1605; -40; 1MHz; Averaged over any 2 millisecond active transmission interval. 1605 ≤ f ≤ 1610; -40 + 60/5 (f-1605); 1MHz. NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna..

and

##### 6.5A.4.4.5 Minimum requirement (network signalled value "NS_04N" and "NS_05N") When "NS_04N" or "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth. Table 6.5A.4.4.5-1: Additional out-of-band requirements for "NS_04N" and "NS_05N" Frequency band (MHz); Channel bandwidth / Spectrum emission limit1 (dBm); Measurement bandwidth; NOTE. BWchannel. 1559 ≤ f ≤ 1605; -40; 1MHz; Averaged over any 2 millisecond active transmission interval. 1605 ≤ f ≤ 1610; -40 + 60/5 (f-1605); 1MHz. 1628.5 ≤ f ≤ 1631.5; -30; 30kHz. 1631.5 ≤ f ≤ 1636.5; -30; 100kHz. 1636.5 ≤ f ≤ 1646.5; -30; 300kHz. 1646.5 ≤ f ≤ 1666.5; -30; 1MHz. 1666.5 ≤ f ≤ 2200; -30; 3MHz. NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna..

However, remaining issues under the 254 band introduction in Rel-18 should not be part of the discussion in Rel-19 band 249.

Therefore, we propose the following set of proposals:

Proposal 1: Discussion of remaining issues from 254 band introduction should remain out of scope of the IoT_NTN_TDD WI.

Proposal 2: A-MPR requirement for 249 shall follow A-MPR requirements already defined for 254 in TS 36.102 since similar frequency range.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254, 249 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254, 249 | TBD |  |
| NS_05N | 6.5B.4.4.6 | 254, 249 | TBD |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Proposal 3: The A-MPR requirements for band 249 should not be more stringent than the ones already proposed for band 254.

As a matter of fact, studies under TR 36.764 already obtained the conclusion that additional MPR are not required for FCC regulations, while the following tables have been summarized for ETSI regulations with PC3 indicating values between 0.2-0.5 dB depending on the number of tones and tone position allocation:

Table 6.2.3.2-1: A-MPR for NS_04N

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2.3.2-2: A-MPR for NS_05N

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

And normally ETSI EN 301 441 and ETSI EN 301 681 have been already taken into account in TS 36.102 and TR 36.764. Moreover, the conclusions from TR 36.764 also indicate:

1/ For ETSI regulations: “For compliance with ETSI regulations specified in EN 301 681 for both in-band (Table 4) and out-of band (Table 3a) emissions, an A-MPR may not be needed, as the real PC3 device emissions presented in this report are expected to be either already within the values of the ETSI emission mask, or to be accommodated via the existing base MPR, which are thus sufficient.”

2/ For FCC regulations: “For compliance with FCC regulations detailed in [2], no A-MPR values are required to be specified, as the real PC3 device emissions levels are below the FCC mask with a 90 kHz frequency offset from the channel edge via the existing base MPR, which is thus sufficient. This is already reflected in the band 255 requirements, where no A-MPR was defined for NS_02N.”

As a matter of fact, current existent MPR tables can be used as such, without any specific Additional-MPR (A-MPR):

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-11 |  |  |  |
| MPR | 0 dB |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

Observation 4: The conclusions from TR 36.764 indicate:

1/ For ETSI regulations: “For compliance with ETSI regulations specified in EN 301 681 for both in-band (Table 4) and out-of band (Table 3a) emissions, an A-MPR may not be needed, as the real PC3 device emissions presented in this report are expected to be either already within the values of the ETSI emission mask, or to be accommodated via the existing base MPR, which are thus sufficient.”

2/ For FCC regulations: “For compliance with FCC regulations detailed in [2], no A-MPR values are required to be specified, as the real PC3 device emissions levels are below the FCC mask with a 90 kHz frequency offset from the channel edge via the existing base MPR, which is thus sufficient. This is already reflected in the band 255 requirements, where no A-MPR was defined for NS_02N.”

## Frequency error requirement discussion

With respect to the discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102, we have the following observations:

Observation 5: [Clause 6.4B.1 of TS36.102] “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.”

Observation 6: According to the note “When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission”, it seems that TDD UE behaviour will be similar to FDD UE behaviour over different segment transmissions. Also, the current FRC in UL for FDD seem to indicate much higher transmissions times than 8ms (proposed for TDD frame), which indicates that segmentation is required with a maximum 8ms segment duration.

Observation 7:

The current TS 36.213 indicates:

With the following explanation in TS 36.331:

NPUSCH-TxDuration-NB-r17 ::= SEQUENCE {

npusch-TxDuration-r17 ENUMERATED {ms2, ms4, ms8, ms16, ms32, ms64, ms128, ms256}

}

and definition:

npusch-TxDuration

Duration of NPUSCH segment transmission in NTN transmission, see TS 36.213 [23]. Unit in ms.

Value ms2 corresponds to 2 ms, value ms4 corresponds to 4 ms and so on.

Observation 8: The current specifications therefore allow segmentation below 8ms time, which would resolve the issue of the pre-compensation requirement being re-defined for NTN NB-IoT TDD 249 frequency band.

Moreover RAN1 is currently evaluating/discussing the following options:

Agreement

For segmented pre-compensation, RAN1 to further discuss at least the following options:

- Option 1: Dropped/postponed NPUSCH / NPRACH are counted for segment determination

- Option 2: A new pre-compensation segment is started at the beginning of each uplink burst

- Option 3: UL segment should only be defined in consecutive U subframes

- Option 4: Segmented pre-compensation does not apply for NB-IoT NTN TDD.

- FFS: How to handle the UL segment gap

Observation 9: RAN1 is currently discussing the postponement of the transmissions for the next transmission time if transmission time higher than 8ms.

Observation 10: If the pre-compensation is done at the beginning of each segment prior to transmission, or at the beginning of each 8ms transmission, the current frequency error requirement from TS 36.102 can be kept as it is.

Proposal 4: Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

## Other potential topics related to FDD/TDD

Proposal 5: Update the following phrase

## 5.2B Operating bands for category NB1 and NB2 Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1. Category NB1 and NB2 UE operate in HD-FDD duplex mode, except for 249 frequency band.

## Work Split

Proposal 6: Discuss Work Split for TS 36.102 (if any) during the meeting:

- Band definition (Clause 5.2)

- REFSENS update (Clause 7.3B)

- [EVM Annex]

- A-MPR

- [Frequency error requirement]

Note: Other identified Clauses are not precluded

# Conclusion

Observation 1: For a better representation, the reader shall use the following figure for information purposes:

Figure 1: Current NTN NB-IoT L-band representation (NTN band 249)

Observation 2: Unless stated otherwise, reuse TS 36.102 NTN IoT 254 L-band Tx requirements also for 249 band in TS 36.102.

Observation 3: Unless stated otherwise, reuse TS 36.102 NTN IoT 255 L-band Rx requirements also for 249 band in TS 36.102.

Proposal 1: Discussion of remaining issues from 254 band introduction should remain out of scope of the IoT_NTN_TDD WI.

Proposal 2: A-MPR requirement for 249 shall follow A-MPR requirements already defined for 254 in TS 36.102 since similar frequency range.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254, 249 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254, 249 | TBD |  |
| NS_05N | 6.5B.4.4.6 | 254, 249 | TBD |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Proposal 3: The A-MPR requirements for band 249 should not be more stringent than the ones already proposed for band 254.

Observation 4: The conclusions from TR 36.764 indicate:

1/ For ETSI regulations: “For compliance with ETSI regulations specified in EN 301 681 for both in-band (Table 4) and out-of band (Table 3a) emissions, an A-MPR may not be needed, as the real PC3 device emissions presented in this report are expected to be either already within the values of the ETSI emission mask, or to be accommodated via the existing base MPR, which are thus sufficient.”

2/ For FCC regulations: “For compliance with FCC regulations detailed in [2], no A-MPR values are required to be specified, as the real PC3 device emissions levels are below the FCC mask with a 90 kHz frequency offset from the channel edge via the existing base MPR, which is thus sufficient. This is already reflected in the band 255 requirements, where no A-MPR was defined for NS_02N.”

Observation 5: [Clause 6.4B.1 of TS36.102] “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.”

Observation 6: According to the note “When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission”, it seems that TDD UE behaviour will be similar to FDD UE behaviour over different segment transmissions. Also, the current FRC in UL for FDD seem to indicate much higher transmissions times than 8ms (proposed for TDD frame), which indicates that segmentation is required with a maximum 8ms segment duration.

Observation 7:

The current TS 36.213 indicates:

With the following explanation in TS 36.331:

NPUSCH-TxDuration-NB-r17 ::= SEQUENCE {

npusch-TxDuration-r17 ENUMERATED {ms2, ms4, ms8, ms16, ms32, ms64, ms128, ms256}

}

and definition:

npusch-TxDuration

Duration of NPUSCH segment transmission in NTN transmission, see TS 36.213 [23]. Unit in ms.

Value ms2 corresponds to 2 ms, value ms4 corresponds to 4 ms and so on.

Observation 8: The current specifications therefore allow segmentation below 8ms time, which would resolve the issue of the pre-compensation requirement being re-defined for NTN NB-IoT TDD 249 frequency band.

Observation 9: RAN1 is currently discussing the postponement of the transmissions for the next transmission time if transmission time higher than 8ms.

Observation 10: If the pre-compensation is done at the beginning of each segment prior to transmission, or at the beginning of each 8ms transmission, the current frequency error requirement from TS 36.102 can be kept as it is.

Proposal 4: Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

Proposal 5: Update the following phrase

## 5.2B Operating bands for category NB1 and NB2 Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1. Category NB1 and NB2 UE operate in HD-FDD duplex mode, except for 249 frequency band.

Proposal 6: Discuss Work Split for TS 36.102 (if any) during the meeting:

- Band definition (Clause 5.2)

- REFSENS update (Clause 7.3B)

- [EVM Annex]

- A-MPR

- [Frequency error requirement]

Note: Other identified Clauses are not precluded

# Annex – previous meeting agreements

4.1 Agreements from RAN4#113

The WF covers the contributions submitted under the following AI:

- 7.28.1 General aspects and work plan	[IoT_NTN_TDD]

- 7.28.2 Study of impact due to periodic pattern	[IoT_NTN_TDD-Core]

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

### Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

### Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

### Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demodulation aspect:

No observation in this meeting.

4.2 Agreements from RAN4#114

General requirements:

Issue 1-1-1: Work Plan presented under R4-2500136.

Agreement: Updated Work Plan to be agreed under R4-2502291 (see Annex for the updated work plan).

Issue 1-2-1: Band number

Agreement: Adopt [249] E-UTRA operating band number for the new TDD 1616-1626.5 MHz NTN NB-IoT frequency band introduction.

Issue 1-2-2: E-UTRA operating band

Agreement: Confirm the proposed band plan as follows:

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | - | 2010 MHz | 2170 MHz | - | 2200 MHz | FDD |
| 255 | 1626.5 MHz | - | 1660.5 MHz | 1525 MHz | - | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | 1616 MHz | - | 1626.5 MHz | 1616 MHz | - | 1626.5 MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. NOTE 3: Band 249 is defined only for NB-IoT operation. |  |  |  |  |  |  |  |

Issue 1-2-3: Gap between EARFCN FDD NTN and EARFCN TDD NTN

Agreement: Consider a gap between previous introduced EARFCN FDD NTN (starting from 2^18-1=262143 for NUL and 2^18-2^15-1=229375 for NDL) and new (first) introduced EARFCN TDD NTN.

Issue 1-2-4: EARFCN for TDD NTN

Agreement: For the NTN E-UTRA TDD EARFCN number, consider to start the NUL/NDL upper range from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL).

Issue 1-2-5: Channel raster for TDD NTN

Agreement: Use a channel raster of 100 kHz and a Step size of <1>.

Issue 1-3-1: TS 36.108 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update table as part of E-UTRA channel numbers in Section 5.4A.2, TS 36.108:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-3-2: TS 36.108 5.7.3 Clause

Agreement: As an initial working assumption, with respect to the definition of IoT-NTN TDD L-band, add the following information in 5.7.3 for the carrier frequency and EARFCN for DL and UL respectively:

- “The carrier frequency of NB-IoT in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

- “The carrier frequency of NB-IoT in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

Moderator Note: Companies to further check.

Issue 1-4-1: TS 36.102 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update TS 36.102 with:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-4-2: TS 36.102 5.4B Clause

Agreement: Update TS 36.102 with:

== Start Change == ## 5.4B Channel arrangement for category NB1 and NB2 ### 5.4B.1 Channel spacing Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. ### 5.4B.2 Channel raster, carrier frequency and EARFCN The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1. The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification. NOTE 2: For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0. == End Change ==

UE requirements:

Issue 2-1-1: Discussion on UE REFSENS for IoT TDD in TS 36.102

Agreement: Reuse REFSENS -108.2dBm for the new TDD NTN NB-IoT operating band.

Issue 2-1-2: Clause 7.3B Update - Reference sensitivity power level for UE category NB1 and NB2

Agreement: RAN4 to adopt the following modification in the TS 36.102:

## 7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 for FDD and TDD UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration.

Table 7.3B-1: Reference sensitivity for UE FDD and TDD category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

Issue 2-1-3: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss AMPR for 249. Proponent to clarify at next meeting which band has to be protected through this AMPR mechanism.

Issue 2-1-4: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss the potential update of frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot/segment.

Moderator note1: further clarify if UL slot is the LTE Tx slot or is for the Tx transmission time U=8. In both cases, the issue seems to be implementation-related.

Moderator note2: [Clause 6.4B.1of TS36.102] “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.”

Moderator note3: According to the 2nd note, it seems that TDD UE behaviour will be similar to FDD UE behaviour over different segment transmissions. Also, the current FRC in UL for FDD seem to indicate much higher transmissions times than 8ms (proposed for TDD frame).

Issue 2-1-5: Discussion on EVM clause for NB-IoT TDD NTN in TS 36.102

Agreement: Add for EVM clause “For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”