# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504628

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.13.6

Source:	Moderator (Huawei)

Title:	Topic summary for [114bis][303] NR_BS_RF_Part2_CLTA_SE

Document for:	Information

# Introduction

Focus of this thread is the co-location test antenna (CLTA) topic of the NR_BS_RF_req_evo work item, as covered in Topic #1.

Additionally, this thread covers discussion on the TX co-existence spurious emissions requirements re-evaluation in Topic #2.

Related objectives were extracted from the WID in RP-243270 are provided for reference:

| Core | OTA test enhancement - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition. Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz. |
| --- | --- |
| Perf | OTA test enhancement - OTA testing scope reduction: Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for TX IM, RX out of band blocking, for NR FR1. - CLTA: Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1. - TRP simplification: Simplify BS TRP estimation methods (e.g., Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods. Transmitter co-existence spurious emission requirements - Conformance testing corresponding to co-existence spurious emission requirements, if needed. |

# Topic #1: CLTA

## Companies’ contributions summary

The following CLTA-related contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503726 | Ericsson | Observation 1: Based on the work done so far in Rel-19 BS RF evolution WI, we can conclude that both measurement and simulation results indicate that 30 dB BS-to-BS isolation may not be a relevant assumption for co-location requirement derivation for bands in the upper region of FR1. Observation 2: To further evolve the co-location concept, it is essential to understand BS-to-BS isolation for relevant co-location situations and how to measure emission and inject an interferer properly. Both these aspects have been studied in Rel-19, proving a good start for follow up work in Rel-20. |
| R4-2504013 | Ericsson | Observation 1: The A-to-A coupling loss is lower for higher correlation. Observation 2: Considering AAS base station characteristics, choosing the minimum coupling loss represents an outlier case and is not fair. Maybe the 90, 95, or 99 percentile is more relevant. Observation 3: The spread in the coupling is lower in the A-SA compared to the A-A case. # Observation 4: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray will impact requirements. Observation 5: For the array to subarray coupling the difference between Urban Macro user-directed beams and 2D and 1D grids of beams and the 5 conformance Rx and Tx beams sets is small. Observation 6: The probe width has an impact on the measurement result in the near-field Observation 7: Correct averaging can provide a method that gives the correct average E-field. Proposal 1: Co-location of two systems at different bands should be further studied. Proposal 2: When deriving co-location requirements, we need to agree on what percentile to use in the coupling loss statistics. Proposal 3: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency. Proposal 4: For conformance testing a reduced set of beams can be used in co-location testing, especially for the antenna-subarray coupling. # Proposal 5: The capability of larger probes (wide band horns) to correctly capture an average field over a surface should be further studied. Proposal 6: The choice of test area needed to sample the average E-field should be further studied. The needed sampling grid also needs to be described. The dependent parameters could be EUT height and depth, test antenna aperture dimensions and/or beamwidths, distance between test antenna and EUT. |
| R4-2504191 | Nokia | Observation 1: SH400 and LB-2080 have a significant size difference. Observation 2: Wideband horn antenna receives different frequency bands in different part of the antenna. Observation 3: Wideband horn antenna results might be more comprehensive if antenna was moved forward to reflect the received surface to the radome of the DUT. Observation 4: Coupling results for 3.5GHz without beam steering are in the neighborhood of 35dB. Observation 5: Coupling results for 7GHz without beam steering are in the neighborhood of 55dB. Observation 6: When collecting data for beam steering scenarions, it makes sense to reuse CLTA arrangements. Observation 7: If specifications for AAS are loosened for frequencies >2.5GHz, there might be mismatch between the RRH requirements. Proposal 1: Consider beam steering range for the scenario analysis of +/- 60°. Proposal 2: Coupling loss studies need to be completed before any discussion about removing requirements. Proposal 3: Intra-system TX IMD requirements should not be removed. |
| R4-2504358 | ZTE Corporation, Sanechips | Observation 1: if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss between sub-array would be larger than 50dB with10cm separation distance. Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc. Proposal 1: if RAN4 agrees to focus on the MCL analysis within the same band in Rel-19, propose to consider Tx OFF power and Tx IMD requirement only. Observation 3: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level. Observation 4: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it. |
| R4-2504594 | Huawei, HiSilicon | Proposal 1: All beam steering cases are covered by the core requirement as such no change is required in the core. Proposal 2: Co-location Transmitter spurious emissions (co-location with other base stations) are no affected by DUT beam direction. Proposal 3: Transmitter intermodulation requirement is not affected by DUT beam direction. Proposal 4: The single column co-located aggressor for the Transmitter intermodulation requirement is confirmed as a worst case. Proposal 5: Beam forming direction is not an issue for the Tx OFF requirement. Proposal 6: Co-location blocking levels coupled to the Rx are not affected by the DUT Rx beam direction and the single column co-located aggressor is confirmed as the worst case. Beam steering does not affect the co-location blocking test. Proposal 7: TX IMD intra-system requirement is applicable to BS type 1-H, and therefore not affected by the discussion in this WI. Proposal 8: Use the following data as baseline (subject to further inputs) for the BS-BS isolation conclusions derivation. - No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); -54; -52; -56. Isolation in horizontal domain (dB); -31; -34 ÷ -46; -46. - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling. - Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 6.7 GHz --> 7 GHz. Isolation in vertical domain (dB). Isolation in horizontal domain (dB); A-to-SA: -55±20 dB; -77±25; A-to-SA: -70±20. Proposal 9: Considering timeline and progress so far, refrain from further WID objectives modifications. Aim to conclude the ongoing open issues. |
| R4-2504596 | Huawei, HiSilicon | Draft CR to TS 37.941: CLTA agreements |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 1.1.

### Sub-topic 1-1: BS-to-BS isolation analysis

Discussion on BS-to-BS isolation analyses were provided in multiple contributions.

- Proposals

  - Proposal 1: Co-location of two systems at different bands should be further studied. (R4-2504013, Ericsson)

  - Proposal 2: When deriving co-location requirements, we need to agree on what percentile to use in the coupling loss statistics. (R4-2504013, Ericsson)

  - Proposal 3: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency. (R4-2504013, Ericsson)

  - Proposal 4: For conformance testing a reduced set of beams can be used in co-location testing, especially for the antenna-subarray coupling. (R4-2504013, Ericsson)

- Moderator: WF in R4-2502279 has prioritized the following BS-to-BS isolation scenarios:

- Horizontal domain prioritized (vertical BS stacking analyses not precluded);

- Same-band analysis (inter-band analysis not precluded),

- Array-to-array case (array to sub-array not precluded);

- Same BS type for aggressor and victim (mixed cases of various BS types not precluded).

For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements.

- Recommended WF

  - Inter-band scenario: As the inter-band scenario was de-prioritized last meeting, no need to re-open the discussion.

  - Till end of Q3 2025, discussion focuses on Core aspects (conformance testing aspects to be handled after Core is concluded).

  - BS-to-BS isolation data collection: Update the baseline set of collected values as in tables below.  Companies are asked to fill in all any missing data based on the template agreed in WF (R4-2502279)

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (M, R4-2504358): -54 | ZTE (M, R4-2504358): -52 | ZTE (M, R4-2504358): -56 |  |
| Isolation in horizontal domain (dB) | ZTE (M, R4-2504358): -31 | ZTE (M, R4-2504358): -34 Huawei (R4-2504594, M): -46 | ZTE (M, R4-2504358): -46 |  |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 6.7 GHz --> 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | S: Ericsson (R4-2418398): A-to-SA: -55±20 dB |  | S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20 |

### Sub-topic 1-2: Beam stering consideration for co-location scenario

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: All beam steering cases are covered by the core requirement as such no change is required in the core.

  - Proposal 2: Co-location Transmitter spurious emissions (co-location with other base stations) are no affected by DUT beam direction.

  - Proposal 3: Transmitter intermodulation requirement is not affected by DUT beam direction.

  - Proposal 4: The single column co-located aggressor for the Transmitter intermodulation requirement is confirmed as a worst case.

  - Proposal 5: Beam forming direction is not an issue for the Tx OFF requirement.

  - Proposal 6: Co-location blocking levels coupled to the Rx are not affected by the DUT Rx beam direction and the single column co-located aggressor is confirmed as the worst case. Beam steering does not affect the co-location blocking test.

  - Proposal 7:  Consider beam steering range for the scenario analysis of +/- 60° (R4-2504191, Nokia)

- Moderator: During previous meeting, WF (R4-2502279) agreed beam steering range as “Beam steering range +/- [60] degrees”. Therefore, now the value can be confirmed.

- Recommended WF

  - Beam steering impact on RAN4 RF requirements: online discussion on requirement-specific proposals 1 – 6.

  - Beam steering simulation assumptions: Agree Proposal 7.

### Sub-topic 1-3: Impacted RF requirements

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: Intra-system TX IMD requirements should not be removed. (R4-2504191, Nokia)

  - Proposal 2: Coupling loss studies need to be completed before any discussion about removing requirements. (R4-2504191, Nokia)

  - Proposal 3: TX IMD intra-system requirement is applicable to BS type 1-H, and therefore not affected by the discussion in this WI. (R4-2504594, Huawei)

  - Proposal 4: if RAN4 agrees to focus on the MCL analysis within the same band in Rel-19, propose to consider Tx OFF power and Tx IMD requirement only. (R4-2504358, ZTE)

- Moderator:

  - During previous meetings it was proposed to remove TX IMD CLTA test for frequency bands above 4.9 GHz. (R4-2502135). There was a WF agreed (R4-2419797), saying “Decision on potential limitation of TX IMD co-location requirement applicability is postponed till conclusion on BS-to-BS isolation analysis is reached.”

- During previous meeting, baseline set of requirements for implementation of the BS-BS isolation conclusions was agreed in WF (R4-2502279) as: TX OFF power, co-location spur emission/Protection of the BS receiver of own or different BS, Tx IMD, co-located OOBB.

- Recommended WF

  - Conclusions on RF requirements impact: Follow previous WF (R4-2419797), which was saying that “Decision on potential limitation of TX IMD co-location requirement applicability is postponed till conclusion on BS-to-BS isolation analysis is reached.”

  - Impacted RF requirements: Prioritize analysis for Tx OFF power and Tx IMD co-location requirements only.

  - Intra-system TX IMD: Not to be affected by the discussion on OTA co-location requirements and BS-BS coupling.

### Sub-topic 1-4: CLTA alternatives: Horn antenna

Further discussion on horn antenna perfomance comparison with CLTA was provided (R4-2504191, Nokia), with multiple observations.

No formal proposals for horn antenna were formulated this time. Further analyses are needed.

### Sub-topic 1-5: OTA co-location requirements re-definition

Discussion and proposals related to re-defined approach to oc-location requirements were submitted (R4-2503726, R4-2504013, Ericsson).

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: The capability of larger probes (wide band horns) to correctly capture an average field over a surface should be further studied. (R4-2503726, Ericsson)

  - Proposal 2: The choice of test area needed to sample the average E-field should be further studied. The needed sampling grid also needs to be described. The dependent parameters could be EUT height and depth, test antenna aperture dimensions and/or beamwidths, distance between test antenna and EUT. (R4-2503726, Ericsson)

  - Proposal 3: Considering timeline and progress so far, refrain from further WID objectives modifications. Aim to conclude the ongoing open issues. (R4-2504594, Huawei)

- Moderator: While WID captures objectives on CLTA definition updates and current work includes investigation on the CLTA antenna alternatives, the above proposals 1 and 2 seem to propose new way of OTA co-location requirements testing (i.e., no CLTA needed). It needs to be clarified if this is actually withing the scope of this WID.

- Recommended WF

  - Online discussion needed.

# Topic #2: TX co-existence spurious emissions requirements re-evaluation

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2504192 | Nokia | Observation 1: Requirements cannot change between bands starting and ending frequency. Observation 2: n96 and n102 are unlicensed bands. Observation 3: There is a need to clarify the definition of minimum coupling loss. Proposal 1: Focus on band n104 as first priority. Proposal 2: Consider using system level simulations for MCL evaluation. Define different scenarios with specific path-loss equations, slow-fading impact, Tx and Rx beamforming etc. |
| R4-2504267 | Ericsson | Observation 1: It is a protection requirement. The spurious emission limits in TS 38.104 define the maximum emissions that an aggressor BS is allowed to emit towards the concerned band stated in each row of Table 6.6.5.2.3-1 of TS 38.104. Observation 2: The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive antennas with 17 dBi antenna gain. Observation 3: With the introduction of AAS BS in frequencies up to 7125 MHz, the impact of beams pointing towards the UE and considerably larger antenna gain, sub-array antenna structures and relevant propagation models should be considered in the coupling loss evaluation. Observation 4: The first preference should be to validate the existing spurious emission requirements with relevant AAS models for the different bands under considerations, and relevant propagation models. Observation 5: The requirement levels may differ, depending on the duplex method for BS-to-BS and BS-to-UE coupling. Observation 6: It can be observed that MCL equal to 53 dB relevant for Microcell environment is used for requirement derivation of requirements for Wide Area BS in TR 25.942. Observation 7: It can be noticed that the UE noise figure assumed for 2 GHz in TR 25.942 is not relevant for frequencies in the upper region of FR1. Observation 8: It can be observed that the requirement level for BS-to-BS co-existence spurious emission is based on the same link budget as co-location spurious emission but with a different assumption on coupling. Observation 9: It is essential to define re-evaluation methodology and agree on simulation parameters at an early stage of the work, since the Release-19 time schedule is limited. Observation 10: With the variables – High number of bands, Deployment scenarios, Duplexing mode, AAS/ Non-AAS antenna model assumptions, it is important to balance the scope of existence work with the available time to conclude on this work item. Observation 11: It is not clear upto what multiples of the fundamental frequency should be considered if they hit the victim frequency above 5.9 GHz. Observation 12: For the BS-to-UE coupling, the link is defined considering the beam points towards the served UE, but the coupling loss is calculated towards the victim UE Proposal 1: RAN4 to differentiate the spurious emission requirements based on BS classes as per the current BS RF specification. Proposal 2 : RAN4 to prioritize Macro deployment scenarios for the co-existence evaluation. Proposal 3 : RAN4 to prioritize 2 GHz and 3.5 GHz as Aggressor simulation frequency for the co-existence evaluation. Proposal 4 : RAN4 to prioritize 6 GHz as Victim simulation frequency for the co-existence evaluation. Proposal 5 : Define a Coupling Loss approach, similar to the legacy MCL approach, where we discuss the possible range on the 95th-99th percentile of the Coupling Loss CDF, instead of the minimum value. Proposal 6 : RAN4 to consider the following in BS-to-UE coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain. Proposal 7 : Considering the requirement levels must be defined based on co-existence evaluation and not co-location, range of 50% - 100% grid shift is a reasonable assumption for this work. Proposal 8 : RAN4 to consider the following in BS-to-BS coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain. Proposal 9 : RAN4 to consider the simulation assumptions in this clause and agree to use them as reference for the simulation evaluation work. |
| R4-2504298 | Huawei, HiSilicon | Observation 1: co-location emissions are generally required in the same bands as the co-existence in same geographical area emission requirements and co-location are significantly harder the necessity of relaxing the co-existence requirements may not be so motivated. Observation 2: The existing co-existence in same geographical area definitions should be maintained otherwise we are defining a new requirement. Observation 3: as the unwanted emissions are outside the operation band they will be uncorrelated and have no array gain Observation 4: As the element/sub-array pattern is fixed no statistical simulation is necessary and MCL approach is more suitable. Observation 5: Calculations indicate that with existing co-existence scenarios and conservative assumptions there is room to relax the same geographical area co-existence requirements at high FR1 frequencies. |
| R4-2504343 | Qualcomm Germany | Observation 1: Legacy co-existence Tx spurious emissions were based on low FR1 bands deployments and assumptions. Noticeably, passive antennas were assumed with fixed antenna gain. Proposal 1: RAN4 to consider system level simulations to derive the new emission requirements. Proposal 2: RAN4 to reuse BS and UE NF values that were used to derive legacy requirement (i.e., BS NF = 7 dB and UE NF = 9 dB). Proposal 3: RAN4 to reuse 0.5 dB degradation of victim receiver sensitivity to be used as a starting point. Proposal 4: RAN4 to consider both AAS and non-AAS at the BS side for low FR1 bands, while consider only AAS at the BS side for high FR1 bands. Proposal 5: RAN4 to consider no beamforming at the UE side (omnidirectional antenna model with 0 dBi). |
| R4-2504359 | ZTE Corporation, Sanechips | Proposal 1: reuse the same principle to specify the co-existence spurious emission requirement for the protection of BS receiver and UE receiver for BS operating at 5.9GHz. For FDD band: - For DL UE protection, reuse the 3db degradation; - For UL BS protection, reuse the 0.8dB degradation. For TDD bands: - For DL UE protection, reuse the 3db degradation; - For UL BS protection, reuse the 0.8dB degradation. - Further discuss how to pick up the appropriate value. Proposal 2: for the network layout assumption to evaluate the MCL between the aggressor BS and victim UE/BS, propose to use the following assumption as starting point. - for UE DL protection, victim UE is closest to the aggressor BS. - For UL BS2BS coexistence, 100% grid shift as starting point. Proposal 3: for U6GHz coexistence spurious emission requirement evaluation, propose to consider the following example spectrum, 700MHz, 1.8GHz, 2GHz, 3.5GHz, 4.9GHz. Proposal 4: for Tx gain assumption for spurious emission signal of U6GHz BS, propose to consider the following options for the further simulations. - Option 1: to consider the antenna element pattern only without any sub-array and beamforming gain considered - Option 2: to consider the sub-array pattern only without beamforming included. - Option 3: to consider AAS modelling with different correlation level similar as AAS SI phase. - Option 4: to consider the simulated antenna pattern for spurious emission [to be provided later on] Proposal 5: for Rx gain assumption of victim BS subject to spurious emission interference, propose to consider the following principle for the further simulations. - For low than 2GHz, both non-AAS antenna and AAS antenna should be considered since FDD massive MIMO is also supported. - To figure out the most stringent requirement between non-AAS and AAS antenna from victim perspective. - For above than 2GHz, AAS antenna should be considered - MCL at the sub-array connector should be considered. Proposal 6: for Tx antenna pattern and Rx antenna pattern, propose to use the antenna configuration in Table 1/2/3/4 for the initial simulation. For Tx gain at U6GHz, BS antenna gain could be assumed as 0dBi as starting point. |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clase 2.1.

### Sub-topic 2-1: Evaluation methodology

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: Consider using system level simulations for MCL evaluation. Define different scenarios with specific path-loss equations, slow-fading impact, Tx and Rx beamforming etc. (R4-2504192, Nokia)

  - Proposal 2: RAN4 to consider system level simulations to derive the new emission requirements. (R4-2504343, Qualcomm)

  - Proposal 3: Define a Coupling Loss approach, similar to the legacy MCL approach, where we discuss the possible range on the 95th-99th percentile of the Coupling Loss CDF, instead of the minimum value.  (R4-2504267, Ericsson)

- Moderator:

  - Considering timeline of this WI, is would be difficult to finalize system level simulation campaign before Rel-19 Core deadline. As we are touching on 25 years old assumptions, technical work shall be done properly, avoiding unnecessary rush.

  - While there was no firm deadline in the TFES LS request, RAN4 needs to discuss on the work handling, proceed within Rel-19, and then potentially shift the work to Rel-20 if needed.

  - While most companies suggest to proceed with the system level evaluations, this is not proposed by all companies. Discuss online on the system level vs. link level (MCL) evaluations pros and cons.

- Recommended WF:

  - Discuss online.

### Sub-topic 2-2: Network layout

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: for the network layout assumption to evaluate the MCL between the aggressor BS and victim UE/BS, propose to use the following assumption as starting point. (R4-2504359, ZTE)

    - For UE DL protection, victim UE is closest to the aggressor BS.

    - For UL BS2BS coexistence, 100% grid shift as starting point.

  - Proposal 2: Considering the requirement levels must be defined based on co-existence evaluation and not co-location, range of 50% - 100% grid shift is a reasonable assumption for this work. (R4-2504267, Ericsson)

- Recommended WF: Discuss online how to narrow down, or decide the value of the grid shift within the range of [50% - 100%].

### Sub-topic 2-3: Requirements structure

Open issues and candidate options before meeting:

- Proposals (non-exclusive)

  - Proposal 1: RAN4 to differentiate the spurious emission requirements based on BS classes as per the current BS RF specification. (R4-2504267, Ericsson)

  - Proposal 2: RAN4 to prioritize Macro deployment scenarios for the co-existence evaluation. (R4-2504267, Ericsson)

- Moderator:

  - For Proposal 1: As this work is ETSI-driven, we need to take care of the related European Commission preferences. More specifically – lack of sympathy to the manufacturer declaration-based requirements. As BS class is subject to manufacturer declaration, EC may not like to see declaration-dependant (i.e., BS class) requirement levels.

- Recommended WF:

  - Proposal 1: Discuss online, considering Moderator feedback above.

  - Proposal 2: Discuss online, subject to discussion on Evaluation methodology in sub-topic 2-1.

### Sub-topic 2-4: Frequency bands for co-existence work

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: Focus on band n104 as first priority. (R4-2504192, Nokia)

  - Proposal 2: for U6GHz coexistence spurious emission requirement evaluation, propose to consider the following example spectrum, 700MHz, 1.8GHz, 2GHz, 3.5GHz, 4.9GHz. (R4-2504359, ZTE)

  - Proposal 3	: RAN4 to prioritize 2 GHz and 3.5 GHz as Aggressor simulation frequency for the co-existence evaluation. (R4-2504267, Ericsson)

  - Proposal 4	: RAN4 to prioritize 6 GHz as Victim simulation frequency for the co-existence evaluation. (R4-2504267, Ericsson)

- Moderator: WID say to “focus on bands in the upper region of FR1 with priority above 5.9 GHz.”

- Recommended WF: Suggest to provide prioritization, while not limiting to certain bands, to allow proper evaluation of co-ex limits for upper region of FR1.

### Sub-topic 2-5: Antenna / array gain considerations

Open issues and candidate options before meeting:

- Proposals (non-exclusive)

  - Proposal 1: RAN4 to consider both AAS and non-AAS at the BS side for low FR1 bands, while consider only AAS at the BS side for high FR1 bands. (R4-2504343, Qualcomm), (R4-2504267, Ericsson)

  - Proposal 2: RAN4 to consider no beamforming at the UE side (omnidirectional antenna model with 0 dBi). (R4-2504343, Qualcomm), (R4-2504267, Ericsson)

  - Proposal 3: for Tx gain assumption for spurious emission signal of U6GHz BS, propose to consider the following options for the further simulations. (R4-2504359, ZTE)

    - Option 1: to consider the antenna element pattern only without any sub-array and beamforming gain considered

    - Option 2: to consider the sub-array pattern only without beamforming included.

    - Option 3: to consider AAS modelling with different correlation level similar as AAS SI phase.

    - Option 4: to consider the simulated antenna pattern for spurious emission [to be provided later on]

  - Proposal 4: for Rx gain assumption of victim BS subject to spurious emission interference, propose to consider the following principle for the further simulations. (R4-2504359, ZTE)

    - For low than 2GHz, both non-AAS antenna and AAS antenna should be considered since FDD massive MIMO is also supported.

      - To figure out the most stringent requirement between non-AAS and AAS antenna from victim perspective.

    - For above than 2GHz, AAS antenna should be considered

      - MCL at the sub-array connector should be considered.

  - Proposal 5: for Tx antenna pattern and Rx antenna pattern, propose to use the antenna configuration in Table 1/2/3/4 for the initial simulation. For Tx gain at U6GHz, BS antenna gain could be assumed as 0dBi as starting point. (R4-2504359, ZTE)

  - Proposal 6: RAN4 to consider the following in BS-to-UE coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain. (R4-2504267, Ericsson)

  - Proposal 7: RAN4 to consider the following in BS-to-BS coupling – For AAS, the gain at Tx to include both array gain and beamforming gain and for Non-AAS, the gain at Tx to only include the array gain. (R4-2504267, Ericsson)

- Recommended WF:

  - Discuss online.

  - Collect proposals in table format (WF).

  - Clarify how to consider frequency specific gains (see sub-topic 2-4).

### Sub-topic 2-6: Evaluation scenarios

Open issues and candidate options before meeting:

- Proposals (non-exclusive)

  - Proposal 1: RAN4 to reuse BS and UE NF values that were used to derive legacy requirement (i.e., BS NF = 7 dB and UE NF = 9 dB). (R4-2504343, Qualcomm)

  - Proposal 2: RAN4 to reuse 0.5 dB degradation of victim receiver sensitivity to be used as a starting point. (R4-2504343, Qualcomm)

  - Proposal 3: reuse the same principle to specify the co-existence spurious emission requirement for the protection of BS receiver and UE receiver for BS operating at 5.9GHz. (R4-2504359, ZTE)

    - For FDD band:

      - For DL UE protection, reuse the 3db degradation;

      - For UL BS protection, reuse the 0.8dB degradation.

    - For TDD bands:

      - For DL UE protection, reuse the 3db degradation;

      - For UL BS protection, reuse the 0.8dB degradation.

      - Further discuss how to pick up the appropriate value.

- Recommended WF:

  - Discuss online.

  - Collect proposals in table format (WF).

  - Clarify if there is need to consider frequency specific aspects (see topic 2-1).

### Sub-topic 2-7: System-level simulation assumptions

Open issues and candidate options before meeting:

- Proposals

  - Proposal 1: RAN4 to consider the simulation assumptions in this clause and agree to use them as reference for the simulation evaluation work. (R4-2504267, Ericsson)

- Recommended WF:

  - Handling depends on the Evaluation methodology discussion.

Table 3.3.1-1: AAS Macro BS antenna configuration

|  | AAS Urban Macro |  |  |
| --- | --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) | 6 GHz (Victim) |
| Parameter |  |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 2x8 | 4x8 | 8x16 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 6x1 | 3x1 | 3x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 28.1 | 28.25 | 22.2 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 | 6 |

Table 3.3.2-1: Non-AAS Macro BS antenna configuration

|  | Non- AAS Urban Macro |  |
| --- | --- | --- |
|  | 2 GHz (Aggressor) | 3.5 GHz (Aggressor) |
| Parameter |  |  |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) (Note 4) | 10x1 | 10x1 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1x1 | 1x1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | N/A |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 38 | 38 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 6 | 6 |