# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504701

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Way Forward for CLTA

Agenda Item:	7.13.6

Source: 	Huawei, HiSilicon

Document for:	Approval

## Sub-topic 1-1: TX IMD co-location

Way forward:

Intra-system TX IMD requirement is not within the scope of this WI. Therefore, no impact on Intra-system TX IMD requirement.

## Sub-topic 1-2: Beam steering

Way forward:

Beam steering impact on the RF requirements can be investigated case-by-case for the pre-selected requirements (as agreed in R4-2502279):

- Transmitter OFF power requirement,

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS,

- Tx IMD requirement,

- Co-located OOBB requirement.

No conclusion on exclusion of any of the above requirements from the beam steering coupling analysis.

## Sub-topic 1-3: Assumptions for the beam steering scenario analyses

Way forward: To improve simulation results alignment, the following assumptions are taken for the beam steering consideration (track changes showing delta compared to RAN4#114 agreements):

Data collection to prioritize the following scenarios:

- Beam steering range +/- 60 degrees,

- Reuse CLTA arrangements, e.g., same plane orientation; 10 cm offset enclosure to enclosure, etc.

## Sub-topic 1-4: Capturing conclusions on Core aspects

Way forward: Previous WF (R4-2502279, RAN4#114) on the timeline for BS-to-BS isolation data collection (i.e., ”conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting”) is amended as follows:

- Close BS-to-BS isolation data collection by RAN4#115 (May 2025) to allow drawing initial conclusions.

NOTE: Further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded.

- For RAN4#115 (May 2025) provide Draft CRs to TR 37.941 with the initial summary of the findings on the following areas (work-split proposed below):

  - Background and motivation for CLTA-alternatives discussion (Huawei),

  - BS-to-BS isolation analysis, including related assumptions (Ericsson),

  - Impact of BS-to-BS isolation analysis on the legacy co-location requirements (ZTE),

  - CLTA alternatives: horn antenna (Nokia).

## Sub-topic 1-5: BS-to-BS isolation analysis

Way forward: Current baseline for BS-to-BS isolation analysis is the measurement and simulation data as captured in the previous WF (i.e., R4-2502279, RAN4#114).