3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504751

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.23.4

Source:	Samsung

Title:	Way Forward for [114bis][306] NR_duplex_evo_General

Document for:	Approval

# Topic #1: General issues

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

    - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

      - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

      - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

##### Issue 1-2-3: Restriction on guardband size for SBFD

Agreement:

- Restriction on guardband size for SBFD:

  - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband.

    - The declaration of guardband size should ensure the benefits from SBFD operation.

      - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

##### Issue 1-2-4: Restriction on DL subband size

Agreement:

- If the SSB symbol is not overlapped with SBFD symbol,

  - There is no restriction on DL subband size.

- If the SSB symbol is overlapped with SBFD symbol,

  - FFS the impact from RAN4 perspective.

##### Issue 1-2-5: FR2-1 CBW support for SBFD DUD case

Agreement:

- If the SSB symbol is not overlapped with SBFD symbol,

  - 100MHz for DUD case is allowed for FR2-1.

- If the SSB symbol is overlapped with SBFD symbol,

  - In Rel-19, the BS channel bandwidth for DUD configuration shall be no less than 200MHz for FR2-1.

### Sub-topic 1-3: Requirement applicability for SBFD

##### Issue 1-3-1: Requirement set applicability table for SBFD-capable BS

Agreement:

- The following RAN4 requirement set applicability for SBFD-capable BS is agreed:

- Table 12.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6.2 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6.5 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | 12.3.7 |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.8 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.9 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7.2 | 12.5.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7.3 | 12.5.7.3 |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | 12.5.7.4 | 12.5.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7.5 | 12.5.7.5 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.9 | 12.5.9 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | 12.6.8 | 12.6.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.9 | 12.6.9 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.10 | 12.6.10 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

### Sub-topic 1-4: BS RF specification structure for SBFD requirements

##### Issue 1-4-1: BS RF specification structure for SBFD requirements

Agreement:

- The following clause structure is adopted for Chapter 12 for requirements for SBFD-capable BS:

12	Requirements for SBFD-capable BS	2

12.1	General	2

12.2	Conducted transmitter characteristics for SBFD-capable BS	3

12.2.1	General	3

12.2.2	Base station output power for SBFD	3

12.2.3	Output power dynamics for SBFD	3

12.2.4	Transmit ON/OFF power for SBFD	4

12.2.5	Transmitted signal quality for SBFD	4

12.2.6	Unwanted emissions for SBFD	4

12.2.7	Transmitter intermodulation for SBFD	4

12.2.8	Transient period between SBFD and non-SBFD	5

12.2.9	In-channel adjacent subband leakage for SBFD	5

12.3	Conducted receiver characteristics for SBFD-capable BS	5

12.3.1	General	5

12.3.2	Reference sensitivity level for SBFD	5

12.3.3	Dynamic range for SBFD	5

12.3.4	In-band selectivity and blocking for SBFD	5

12.3.5	Out-of-band blocking for SBFD	5

12.3.6	Receiver spurious emissions for SBFD	5

12.3.7	Receiver intermodulation for SBFD	5

12.3.8	In-channel selectivity for SBFD	5

12.3.9	In-channel adjacent subband blocking for SBFD	5

12.4          Conducted performance requirements for SBFD-capable BS

12.5	Radiated transmitter characteristics for SBFD-capable BS	5

12.5.1	General	5

12.5.2	Radiated transmit power for SBFD	5

12.5.3	OTA base station output power for SBFD	5

12.5.4	OTA output power dynamics for SBFD	5

12.5.5	OTA transmit ON/OFF power for SBFD	5

12.5.5.1	General	5

12.5.5.2	OTA transmitter OFF power for SBFD	5

12.5.5.3	OTA transient period for SBFD	5

12.5.6	OTA transmitted signal quality for SBFD	5

12.5.7	OTA unwanted emissions for SBFD	6

12.5.8	OTA transmitter intermodulation for SBFD	6

12.5.9	OTA transient period between SBFD and non-SBFD	6

12.5.10	OTA in-channel adjacent subband leakage for SBFD	6

12.6	Radiated receiver characteristics for SBFD-capable BS	6

12.6.1	General	6

12.6.2	OTA sensitivity for SBFD	6

12.6.3	OTA reference sensitivity level for SBFD	6

12.6.4	OTA dynamic range for SBFD	6

12.6.5	OTA in-band selectivity and blocking for SBFD	6

12.6.6	OTA out-of-band blocking for SBFD	6

12.6.7	OTA receiver spurious emissions for SBFD	6

12.6.8	OTA receiver intermodulation for SBFD	6

12.6.9	OTA in-channel selectivity for SBFD	6

12.6.10	OTA in-channel adjacent subband blocking for SBFD	6

12.7		Radiated performance requirements for SBFD-capable BS

### Sub-topic 1-5: Draft CR and work splitting

##### Issue 1-5-1: Draft CR and work splitting

Way forward:

- The following work splitting will be followed in RAN4#115 and #116, aiming at WI completion by Sep. 2025.

  - In the below work splitting, the same requirements for conducted and OTA counterparts are assigned into the same package.

| Work splitting packages | Relevant impacted clauses in TS38.104 | Responsible company |
| --- | --- | --- |
| Package-1 | Chapter 2 References Chapter 3 Definitions, symbols and abbreviations Chapter 5 Operating bands and channel arrangement | TBA |
| Package-2 | 12.2 Conducted transmitter characteristics for SBFD-capable BS 12.2.1 General 12.2.2 Base station output power for SBFD 12.2.3 Output power dynamics for SBFD 12.5 Radiated transmitter characteristics for SBFD-capable BS 12.5.1 General 12.5.2 Radiated transmit power for SBFD 12.5.3 OTA base station output power for SBFD 12.5.4 OTA output power dynamics for SBFD | TBA |
| Package-3 | 12.2.4 Transmit ON/OFF power for SBFD 12.2.5 Transmitted signal quality for SBFD 12.5.5 OTA transmit ON/OFF power for SBFD 12.5.6 OTA transmitted signal quality for SBFD | TBA |
| Package-4 | 12.2.6 Unwanted emissions for SBFD 12.2.7 Transmitter intermodulation for SBFD 12.5.7 OTA unwanted emissions for SBFD 12.5.8 OTA transmitter intermodulation for SBFD | TBA |
| Package-5 | 12.2.8 Transient period between SBFD and non-SBFD 12.2.9 In-channel adjacent subband leakage for SBFD 12.5.9 OTA transient period between SBFD and non-SBFD 12.5.10 OTA in-channel adjacent subband leakage for SBFD | TBA |
| Package-6 | 12.3 Conducted receiver characteristics for SBFD-capable BS 12.3.1 General 12.3.2 Reference sensitivity level for SBFD 12.3.3 Dynamic range for SBFD 12.6 Radiated receiver characteristics for SBFD-capable BS 12.6.1 General 12.6.2 OTA sensitivity for SBFD 12.6.3 OTA reference sensitivity level for SBFD 12.6.4 OTA dynamic range for SBFD | TBA |
| Package-7 | 12.3.4 In-band selectivity and blocking for SBFD 12.3.5 Out-of-band blocking for SBFD 12.6.5 OTA in-band selectivity and blocking for SBFD 12.6.6 OTA out-of-band blocking for SBFD | TBA |
| Package-8 | 12.3.6 Receiver spurious emissions for SBFD 12.3.7 Receiver intermodulation for SBFD 12.6.7 OTA receiver spurious emissions for SBFD 12.6.8 OTA receiver intermodulation for SBFD | TBA |
| Package-9 | 12.3.8 In-channel selectivity for SBFD 12.3.9 In-channel adjacent subband blocking for SBFD 12.6.9 OTA in-channel selectivity for SBFD 12.6.10 OTA in-channel adjacent subband blocking for SBFD | TBA |

### Sub-topic 1-7: Update for HPUE duty cycle evaluation

##### Issue 1-7-1: Update for HPUE duty cycle evaluation

Way forward:

- RAN4 has discussed the two following points:

  - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

  - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

- FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

# Topic #2: PUSCH with resource muting for CLI handling

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way forward:

- RAN4 has discussed this issue and will try to draft a reply LS to RAN1 based on existing results in next meeting.

  - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

# Topic #3: SBFD adjacent channel coexistence

### Sub-topic 3-2: SBFD-TDD co-existence in adjacent channel

##### Issue 3-2-1: Fair coexistence between SBFD and TDD networks

Way forward:

- RAN4 can further wait RAN1 conclusion on fair co-existence between SBFD and TDD networks.

# Topic #4: New BS RF requirements for SBFD

### Sub-topic 4-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 4-1-1: Location of transient period for Case-B (from SBFD symbol to UL symbol)

Agreement:

- For Case-B, guard symbol(s) should be reserved before UL symbols

  - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation.

##### Issue 4-1-2: Location of transient period for Case-C (from DL symbol to SBFD symbol)

Agreement:

- For the location of transient periods Case-C,

  - Both Option 1 (guard symbols reserved in non-SBFD DL symbol(s)) and Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission) shall be allowed, up to gNB implementation.

##### Issue 4-1-3: RAN4 requirement impact

Agreement:

- For the location of transient period between SBFD and non-SBFD symbols,

  - No need to be specified in RAN4 core requirement;

  - For conformance testing for relevant requirements,

    - FFS test configurations.

##### Issue 4-1-4: Text proposal for transient period requirement applicability on SBFD subbands

Way forward:

- Text proposal for transient period requirement applicability on SBFD subbands:

  - Discuss TP for new SBFD chapter directly.

### Sub-topic 4-2: LS to RAN1 on RAN4 agreement of transient period

##### Issue 4-2-1: LS to RAN1 on RAN4 agreement of transient period

Way forward:

- RAN4 prepares a LS to RAN1 by capturing the existing agreement on transient period.

### Sub-topic 4-3: In-channel adjacent subband leakage

Agreement:

- The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”).

##### Issue 4-3-2: Assumption for in-channel adjacent subband leakage requirement

Way forward:

- FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage.

##### Issue 4-3-3: Absolute limit for in-channel adjacent subband leakage

Way forward:

- Absolute limit for in-channel adjacent subband leakage:

  - FFS by considering the following exemplary cases by performing analysis including measurement or simulation:

    - FR1:

      - DU/UD: 80MHz (DL) – 20MHz (UL)

      - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL)

      - Other cases are not precluded

    - FR2-1:

      - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL)

      - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL)

      - Other cases are not precluded

  - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

##### Issue 4-3-4: Necessity of in-channel adjacent subband leakage ratio

Way forward:

- No need to further discuss the necessity of in-channel adjacent subband leakage ratio, based on agreement from RAN4#114.

### Sub-topic 4-4: In-channel adjacent subband blocking

##### Issue 4-4-1: In-channel adjacent subband blocking requirement

Way forward:

- New requirement for in-channel adjacent subband blocking:

  - The requirement can be derived by considering co-site inter-sector case and/or inter-site case

    - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification.

  - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

##### Issue 4-4-2: Assumption of gNB-to-gNB CLI handling

Way forward:

- FFS the assumption of gNB-to-gNB CLI handling for two cases:

  - Inter-site case:

    - Note: for the blocking requirement, RX digital beamforming cannot be considered.

  - Co-site inter-sector case:

    - Note: Inter-sector is near field interaction.