# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504766

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Way Forward for Tx spurious emissions co-existence limits re-evaluation

Agenda Item:	7.13.6

Source: 	Huawei, HiSilicon

Document for:	Approval

### Sub-topic 2-1: Evaluation methodology

Way forward:

- Consider MCL methodology, including AAS and non-AAS implementations.

  - TR 25.942 is used as reference for the methodology derivation.

- System level evaluations are not precluded.

- Aim Rel-19 conclusion.

### Sub-topic 2-2: Network layout

Way forward:

To determine coupling between the aggressor BS and victim UE/BS, the following is the starting point:

- For UL BS-BS coexistence, 100% grid shift as a starting point; evaluations of grid shift of 50% is not precluded.

### Sub-topic 2-3: BS classes consideration

Way forward:

While the legacy Tx spur co-existence limits are defined without distinctions of the BS class, further investigate whether and how to differentiate the spurious emission requirements based on BS classes.

For system level evaluations:

  - For the protection of BS receiver, to prioritize the Macro BS scenario;

  - For the protection of UE receiver, to start with Macro BS scenario; other BS class analyses are not precluded.

### Sub-topic 2-4: Frequency bands for co-existence work

Way forward:

- Aggressor carrier frequencies: prioritize 2 GHz, 3.5 GHz,

- Victim frequency bands: prioritize lower bound frequency of band n104, i.e, 6425 MHz.

### Sub-topic 2-5: Antenna model assumptions

Way forward:

- BS:

  - Aggressor BS: AAS and non-AAS;

  - Victim BS: AAS only.

  - Assumptions for antenna gains for OOB are FFS.

- UE: no beamforming; omnidirectional antenna model with 0 dBi.

Assumptions in sub-topics 2.5- to 2-7 are subject to consistency verification.

### Sub-topic 2-6:  Protection margin

Way forward:

- For DL (UE Rx) protection: reuse the 3 dB desensitisation assumption.

- For UL (BS Rx) protection: reuse the 0.8 dB desensitisation assumption.

### Sub-topic 2-7: System-level simulation assumptions

Way forward:

For AAS BS: follow TR 38.922 as baseline

For 2 and 3.5 GHz we reuse 4.4

For non-ASS: follow 37.840 as starting point.

For Propagation Model, use TR 38.901 as starting point.

#### Other simulation parameters

The following parameter value are considered as a staring point:

- ISD for Macro – 500m (Reference from TR 25.942);

- Macro BS antenna height – 25m

- BS to UE minimum distance – 35m

- UE outdoor – 100%