3GPP TSG-RAN WG4 Meeting #114bis	R4-2504901

Wuhan, China, April 7th – April 11th, 2025

Agenda item:			7.21.4

Source:	Moderator (Nokia)

Title:	WF on AI/ML Mobility

Document for:	Approval

# Introduction

- This WF captures the discussions and agreements carried out on AI/ML Mobility SI under [114bis][210] FS_NR_AIML_Mob thread.

# Agreements.

## RAN4 requirements for RRM Measurement prediction

### Agreements in the adhoc session

Issue 2-1-4: Need of Relative RSRP accuracy

Agreement:

RAN4 to consider both Absolute RSRP and Relative RSRP accuracy as the candidate metrics for RRM prediction use case.

Issue 2-1-1: Relative RSRP accuracy definition

Agreement:

- For intra-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - FFS if this definition will be needed.

- For inter-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

    - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

    - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

- FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

Issue 2-1-6: Impacts on beam/cell level measurements

Agreement:

- For Rel. 19 study:

- RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

Issue 2-1-8: Inter-frequency prediction scenario

Agreement:

- RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects:

  - the side condition of frequency prediction (e.g., EPRE difference)

  - the impact of correlation coefficient between measured and predicted frequency layers

  - impact of cluster approach, e.g.,

    - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

    - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

### Agreements in the RRM session

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1

Agreement:

During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase.

- At least capture the definition of the two Alternatives in the TR.

## RAN4 requirements for measurement event prediction

### Agreements in the RRM session

Issue 3-1-1: Prioritization of cases for event prediction

Agreement:

- RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization

Issue 3-1-2: Performance metric for event prediction use case

Agreement:

- Potential RRM requirement impact for event prediction use case - for indirect case:

- RAN4 to study the requirement for the predicted event triggered reporting

  - RAN4 to study the requirement for [delay and accuracy]

  - RAN4 to discuss the metric for the event prediction

- Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time],

  - Candidate options:

    - Option 1: RAN4 will define requirement for the predicted RSRPs

    - Option 2: RAN4 will not define requirement for the predicted RSRPs

## Testability and interoperability

### Agreements in the Coffee Break session

Issue 4-1-2: Prediction consistency in time domain

Agreement:

- To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories.

  - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation

  - RAN4 to prioritize discussion on FR1.

Issue 4-1-3: Multiple cells in the testing setup

Agreement:

- RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction.

- FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

# Way Forward.

## RAN4 requirements for RRM Measurement prediction

### Discussions in the adhoc session

Issue 2-1-7: Impacts of measurement error on prediction accuracy

WF:

Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.