3GPP TSG-RAN WG4 Meeting # 114bis											R4-2504902

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part1

Agenda Item:	7.22.4

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM requirements for UE-initiated/event-driven beam management >

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

      - the value of current beam and new beams are the same.

  - For M in L1_measurement periods:

    - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam

    - If timeRestrictionForChannelMeasurement is not configured,

      - Further discuss on:

        - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

        - Option 2: No requirements even the NW can configure it.

        - Option 3: FFS

  - For P (sharing factor) in L1_measurement periods:

    - For SSB based or CSI-RS based,

      - The definition is the same as legacy

      - The value can be the same or different for the current beam and new beam

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- The event triggered measurement reporting delay includes the Tfirst UL channel

  - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Xiaomi, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured.

- Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Option 3: (Nokia, Ericsson)

  - Wait for further RAN1 conclusion

- For how to specify the requirements in option 2 above:

  - Option 2a:

    - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

    - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

    - C: event instances threshold is configured by NW.

    - Ttime_window: time window is configured by NW.

    - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

  - Other options are not precluded

Issue 1-4: Whether/how to specify the UE behaviour if new L1-RSRP during the time period between first PUCCH and second PUSCH?

Agreement:

- No further discussion on this issue in RAN4.

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1:

  - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

- Option 2:

  - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Issue 1-6: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- For MAC-CE based DL/UL TCI state switch delay

  - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (ZTE)

    - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

  - Option 3: (MediaTek)

    - FFS on whether to remove Tfirst-SSB+TSSB-proc

- For DCI based DL/UL TCI state switch delay

  - Option 1:

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2:

    - Whether to further reduce the delay, wait for further RAN1 progress

Issue 1-7: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 2:

  - For such case, No RAN4 requirements.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Issue 1-9: Requirements for event-1 and event-7?

<Way forward >

- Option 1: When requirements for Event-2 are agreed, agree to reuse all requirements for Event-1 and Event-7.

- Option 2: others are not precluded

Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities.

- Option 1:

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2:

  - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

Issue 1-11: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Apple, Ericsson)

  - Discuss it later.

- Option 2: (CMCC, Huawei, Nokia)

  - Not preferred to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO

- Option 3: (MediaTek)

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

< Topic #2: Spec structure of Rel-19 MIMO Phase 5 >

Issue 2-1: Spec structure and work split of Rel-19 MIMO Phase 5

Agreement:

| Objective | Clauses | Companies |
| --- | --- | --- |
| UEIBM | 9.x1 event trigger Reporting based on L1-RSRP measurements 9.x1.1 Introduction 9.x1.2 Requirements applicability 9.x1.3 Measurement Reporting Requirements 9.x1.4 L1-RSRP measurement requirements 9.x1.4.1 SSB based L1-RSRP Reporting 9.x1.4.2 CSI-RS based L1-RSRP Reporting 9.x1.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement 9.x1.5.1 Measurement restriction for SSB based L1-RSRP 9.x1.5.2 Measurement restriction for CSI-RS based L1-RSRP 9.x1.6 Scheduling availability of UE during L1-RSRP measurement | Samsung |
|  | 9.x2 event trigger Reporting based on L1-RSRP measurements for a cell with different PCI from serving cell 9.x2.1 Introduction 9.x2.2 Requirements applicability 9.x2.3 Measurement Reporting Requirements 9.x2.4 L1-RSRP measurement requirements 9.x2.4.1 SSB based L1-RSRP Reporting 9.x2.5 Measurement restriction for SSB for L1-RSRP measurement 9.x2.5.1 Measurement restriction for SSB based L1-RSRP 9.x2.6 Scheduling availability of UE during L1-RSRP measurement | Apple |
|  | 8.15 Active downlink TCI state switching delay for unified TCI 8.16 Active uplink TCI state switching delay for unified TCI | Xiaomi |
| CJTC reporting | 9.x CJT calibration using delay/freq/[FFS on Phase] offset measurement for reporting 9.x.1 Introduction 9.x.2 Requirements applicability 9.x.3 Measurement Reporting Requirements | Ericsson |
|  | 9.x.4 measurement requirements 9.x.4.1 CSI-RS based Inter-TRP time/frequency/[FFS on Phase] misalignment Reporting | Huawei |
|  | 9.x.6 Scheduling availability of UE | Samsung |
| Asymmetric DL sTRP/UL mTRP deployment | 7 Timing 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment | CMCC |
|  | 7.5 Maximum Transmission Timing Difference 7.5.1 Introduction 7.5.7 Minimum requirements for multi-TRP | Nokia |
|  | 7.6 Maximum Receive Timing Difference 7.6.1 Introduction 7.6.8 Minimum requirements for Multi-TRPs | ZTE |

Note: For whether to use existing 9.5 and 9.13 for UEIBM or new sections, no consensus yet. RAN4 will decide in following meetings. Two companies can use this in the table as baseline for next meeting.