3GPP TSG-RAN WG4 Meeting #114-bis										                  R4-2504904

Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.23.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR_duplex_evo

Document for:	Approval

Agreements

# Topic #1: UE-to-UE CLI handling

    - Sub-topic 1-1: General issues

Issue 1-1-1: Rx beam

- Agreement

  - RAN4 to use a reference to RAN1 spec to reflect the Rx beam assumption for L1 UE-to-UE CLI measurements, which can be further discussed in CR drafting phase.

  - The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

Issue 1-1-2: Measurement reporting

- Agreement

  - Define reporting requirements for periodic reporting of L1-CLI-RSSI.

  - L1-RSRP measurement reporting requirement for periodic reporting as defined in clause 9.5.3.1 are re-used

Issue 1-1-3: Measurement capability

- Agreement

  - RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Issue 1-1-4: Report mapping

- Agreement (online session on Monday)

  - Confirm that the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI, including both absolute and differential reporting, given by RAN1 LS R1-2501620, are reasonable.

  - Note: In RAN4 understanding, the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP.

  - RAN4 to define measurement report mapping for L1-SRS-RSRP and L1-CLI-RSSI, including both absolute reporting and differential reporting.

Issue 1-1-6: Spec structure

- Agreement

  - Close the issue, discuss the CR work split directly.

    - Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-1: Measurement period

- Agreement

  - The number of samples (M) for measurement period:

    - For periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

    - For aperiodic SRS resources M=1

Issue 1-2-2: Scheduling restriction

- Agreement (online session on Monday with update)

  - For UL,

    - Scheduling restrictions always apply, for both FR1 and FR2.

    - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements.

    - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211.

  - For DL,

    - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies.

    - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements.

    - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

Issue 1-2-3: Measurement restriction

- Agreements

  - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

  - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options

    - Option 1: Prioritize L1-SRS-RSRP measurement

    - Option 2: Prioritize L1-CLI-RSSI measurement

    - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

    - Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-1: Measurement period

- Agreements (online session on Monday)

  - For DUD case with method #1 and resource type#2

    - In RAN4 understanding, if UE can support simultaneous PDSCH reception on the two DL subbnads for DUD, it can support simultaneous RSSI measurement on the two DL subbands based on 1 sample.

    - RAN4 to decide the number of samples for measurement period based on RAN1 progress on UE capability.

  - For method #3, the measurement period is based on 1 sample.

Issue 1-3-2: Scheduling restriction

- Agreements (adhoc session on Tuesday with addition)

  - Method #1:

    - For UL,

      - Scheduling restrictions always apply, for both FR1 and FR2.

      - The restricted symbols include symbols for RSSI resource.

      - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2

    - For DL,

      - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource

  - Method #3:

    - For UL,

      - Scheduling restrictions always apply, for both FR1 and FR2.

      - The restricted symbols include symbols for RSSI resource.

      - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2

    - For DL,

      - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource.

        - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH.

      - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

  - Generic assumption for defining RRM requirements, not intend to add it in the core requirement.

    - For all Method #1, Method #2 and Method #3,

      - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

Issue 1-3-3: Measurement restriction

- Agreements (adhoc session on Tuesday)

  - Collision between L1-CLI-RSSI measurements with different methods

    - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol.

  - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements

    - For Method #1& Method 3:

      - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

# Topic #2: SBFD operation impacts

    - Sub-topic 2-1: RRM impacts of SBFD operation

      - Issue 2-1-1a: CSI-RS L1 measurement – collision with UL

- Agreements (adhoc session on Thursday)

  - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement.

  - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach.

      - Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Background:

  - Different cases for DUD

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

    - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

  - RAN1 agreements related to CSI processing time

Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

- Agreements (adhoc session on Thursday with additions)

  - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD

  - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2

  - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements

    - Option 1: Add a lower bound of 8ms on top of existing delay requirements.

    - Option 2: other

  - For Case 3

    - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs

      - In the test case, not model the interfering UE.

      - Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

- Agreements

  - For L1-RSRP and L1-SINR

    - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network

    - For L1-RSRP and L1-SINR measurement period, further discuss the following options.

      - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

      - Option 2: other

  - For RLM/BFD/CBD, further discuss the following options

    - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type

    - Existing measurement period apply

    - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

      - Issue 2-1-4: Requirements for RACH requirements

- Agreements

  - Discuss whether and how RACH requirements are to be updated for SBFD operation based on RAN2 agreement.

      - Issue 2-1-6: Requirements for MG and BWP switch

- Agreements

  - Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements.

      - Issue 2-1-7: Requirements for UL resource muting

- Agreement

  - RAN4 to confirm there is no RRM impact due to UL resource muting

# Potential issues for further discussion

Topic 1: UE-to-UE CLI handling

- Sub-topic 1-1: General issues

Issue 1-1-5: Requirement applicability

Sub-topic 1-2: L1-SRS-RSRP

      - Issue 1-2-4: Side condition

Issue 1-2-5: Measurement accuracy

Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-4: Side conditions

Issue 1-3-5: Measurement accuracy

Issue 1-3-6: SCS for performing measurement

Topic #2: SBFD operation impacts

Sub-topic 2-1: RRM impacts of SBFD operation

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

Issue 2-1-3: Requirements for scheduling restriction

CR work split

| Objective | Impacted requirements and clauses | Contents | Company |
| --- | --- | --- | --- |
| General | 3.1 Definitions | Definition of SBFD symbol, UL and DL subband, etc. | Vivo |
|  | 3.3 Abbreviations | Abbreviations for SBFD |  |
| RRM requirements for UE-to-UE CLI handling | 9.x L1 CLI measurement (new clause) |  | Nokia |
|  | 9.x.1 L1-SRS-RSRP measurements |  |  |
|  | 9.x.1.1 Introduction | General description of the measurement: capability, configuration, measurement resource location and SCS, measurement timing, Rx beam determination, … |  |
|  | 9.x.1.2 Requirements applicability | Similar to 9.7.2.2 |  |
|  | 9.x.1.3 Measurement Reporting Requirements | Similar to 9.7.2.3 (only aperiodic reporting) |  |
|  | 9.x.1.4 Measurement capability | Similar to 9.7.2.4 (exact number TBD pending on RAN1 agreements) |  |
|  | 9.x.1.5 Measurement period | Including also measurement restriction | CMCC |
|  | 9.x.1.6 Scheduling availability | Similar to 9.7.4, for L1-SRS-RSRP only | ZTE |
|  | 9.x.2 L1-CLI-RSSI measurements |  | Samsung |
|  | 9.x.2.1 Introduction | General description of the measurement: capability, configuration, measurement resource location and SCS, measurement timing, Rx beam determination, … |  |
|  | 9.x.2.2 Requirements applicability | Similar to 9.7.3.2 |  |
|  | 9.x.2.3 Measurement Reporting Requirements | Similar to 9.7.3.3 (only periodic and aperiodic reporting) |  |
|  | 9.x.2.4 Measurement capability | Similar to 9.7.3.4 (exact number TBD pending on RAN1 agreements) |  |
|  | 9.x.2.5 Measurement period | Including also measurement restriction | LGE |
|  | 9.x.2.6 Scheduling availability | Similar to 9.7.4, for L1-CLI-RSSI only | Ericsson |
|  | Others, if identified |  | TBD |
| RRM impacts for SBFD operation | 8.1.3.2 Minimum requirement | Update CSI-RS based RLM evaluation period for impact of SBFD operation. Changes should be aligned with 8.5.3.2 and 8.5.6.2. | QC |
|  | 8.5.3.2 Minimum requirement | Update CSI-RS based BFD evaluation period for impact of SBFD operation. Changes should be aligned with 8.1.3.2 and 8.5.6.2. | CATT |
|  | 8.5.6.2 Minimum requirement | Update CSI-RS based CBD evaluation period for impact of SBFD operation. Changes should be aligned with 8.1.3.2 and 8.5.3.2. | CTC |
|  | 9.5.4.2 CSI-RS based L1-RSRP Reporting | Update CSI-RS based L1-RSRP measurement period for impact of SBFD operation. Changes should be aligned with 9.8.4. | OPPO |
|  | 9.8.4.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | Update CSI-RS based L1-SINR measurement period for impact of SBFD operation. Changes should be aligned with 9.5.4.2. | Huawei |
|  | 9.8.4.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured |  |  |
|  | 9.8.4.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured |  |  |
|  | 9.10.1 Introduction | Clarification of L3 CSI-RS measurement requirement. | MTK |
|  | 9.1.2 Measurement gap | Extend the term “UL slot” and “DL slot” to include SBFD slot | Apple |
|  | 8.6 Active BWP switch delay |  |  |
|  | Others, if identified | Based on discussions so far, possible impacts could be for - Inter-frequency SSB measurement without gap - RACH requirements | TBD |