3GPP TSG-RAN WG4 Meeting #114bis	R4-2505107

Wuhan, China, 7th ‒ 11th April, 2025

Title: 	WF on AI ML for air interface study

Agenda Item:	7.20.3

Source: 	Ericsson

Document for:	Approval

# Topic 1: CSI compression: Simulation analysis for options 3 and 4

## Datasets

Endorsed preliminary observations

- Datasets consist of separate training data and separate test data.

- All companies have produced datasets based on the same simulation assumptions, however differences can be observed between performance with different companies datasets.

- The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets

- The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s).

## Option 3 track 1

Endorsed preliminary observations

- The investigation on option 3 is based on using the agreed structure for the decoder and encoder

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset

- The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet.

  - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2)

- For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets

- Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered

Further simulation activity for option 3 track 1

No further simulation activity is needed for option 3 track 1.

## Option 3 track 2

Endorsed preliminary observations

- There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset

  - Around 1% variation in SGCS; see results spreadsheet for more details

  - Preliminary observation; further results may be added

- It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional

- When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far

  - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details

  - Preliminary observation; further results may be added

- Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered

Further simulation activity for option 3 track 2

- Definitions

  - Training Dataset 1 is the dataset used for training of the frozen decoder.

  - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder.

- For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset.

- Additional results 1 for RAN4#115:

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset.

- Additional results 2 for RAN4#115:

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset.

- Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder

  - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset)

  - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset.

  - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

## Option 4

Endorsed preliminary observations

- For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used

- For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b)

Further simulation activity for option 4

- All interested companies are requested to upload their “own” decoders to the server as soon as possible.

  - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset.

- Definitions

  - Dataset 1 = Dataset used for training the frozen encoder

  - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder)

  - Dataset 3 = Dataset used for training the own encoder (using the own decoder)

- First results priority results for RAN4#115:

  - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset

  - Test dataset = Mixed test dataset

  - Provide results for

    - Your company’s “own” decoder and “own encoder”

    - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders)

- Second priority results for RAN4#115:

  - Dataset 1 = Dataset 2 = Mixed training dataset

  - Dataset 3 = Company own dataset

  - Test Dataset: Mixed Dataset, also consider own test dataset if interested.

  - Provide results for

    - Your company’s “own” decoder and “own encoder”

    - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders)

- Results for consideration of lower complexity encoder for RAN4#115:

  - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset

  - Test dataset = Mixed test dataset

  - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156]

  - Provide results for

    - Your company’s “own” decoder and “own encoder”

    - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders)

- For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset.

## Reduction of simulation results volume

In order to simplify simulations, the following was agreed;

- No more results are expected Option 3 track 1

- No more results are expected Unquantized model results

- For option 3 track 2, use the Samsung decoder as the frozen decoder. Lower priority for Nokia decoder.

- For option 4, use the Samsung encoder as the frozen encoder. Lower priority for Nokia encoder.

# Topic 2: CSI compression: Non-simulation issues

## What needs to be captured or specified for Option 3

Agreements

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

For further discussion:

The following topics should be further discussed:

- For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured.

- Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available

  - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments

- Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

## What needs to be captured or specified for Option 4

Agreements

Agreement:

- Dataset is specified for option 4a.

- FFS on dataset is captured for option 4b.

- Assume reference decoder is captured.

- FFS on reference encoder for option 4a

- Test decoder structure is (i) fully specified or (ii) partially specified

- NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase.

For further discussion:

The above FFS; i.e.

- Whether a dataset is captured for option 4b

- Whether a reference encoder is specified for option 4a

## Relation between RAN1 option 3 and RAN1 inter-vendor collaboration direction C

Agreements

None

For further discussion:

- From the online discussion: FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions.

The relation between the RAN4 options and RAN1 inter-vendor interoperability directions should be further clarified.

# Topic 3: LCM requirements

## Delay requirements for reporting / monitoring

Agreement:

- Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner

      - FFS on which use cases the requirement will be defined for

      - FFS on the details for each use case in WID

## Delay requirements for activation/switching/fallback

Agreement:

- RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2:

      - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference

      - functionality deactivation refers to the process of network deactivating an active functionality

Agreement:

- FFS on whether the deactivation requirements will be introduced

      - The starting point is when the UE receives the signalling to commence deactivation

      - FFS when the endpoint for deactivation is

      - FFS whether fallback delay is the same as deactivation delay or has some other components

Agreement:

- For beam management use case

      - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML

      - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML

      - FFS on the definition of first reporting

# Topic 4: Generalization

## Relation between generalization testing and additional conditions signalling

Agreements

- There shall be consistency between applicable conditions signalling and testing

- No test will be defined that implies a change of network condition or associated ID during the test.

## Non-static scenarios

Agreement:

- Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing.

- FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario.

# Topic 4: Other

## Post-deployment

Agreement:

- Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing”

- Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

## Discussion on general issues

Further discussion will take place on general issues (LCM requirements, post-deployment testing, generalization etc.) will take place at RAN4#115 and further contributions welcome.