3GPP TSG-RAN WG4 Meeting # 114bis	R4-2505114

Wuhan, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	WF on HPUE RF requirements for IoT-NTN

Agenda Item:	7.10.5

Document for:	Approval

- Tx Requirements

- Tx requirements

Issue 1-1-1: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC2

- WF (Ad-hoc agreement):

  - Tentative PA Calibration points

    - Option 1:

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc

      - CIM3: 60 dBc

      - [NR ACLR: 31dBc]

    - Option 2:

      - DFT-s-OFDM 1RE (Single Tone)

      - [Single Tone] [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

      - [2] or [3] dB post PA loss

      - 0dB MPR

      - Carrier leakage 28dBc

      - IQ image:28 dBc

      - CIM3: 60 dBc

  - Both options can be considered

  - Both Testing and simulation may be considered.

Issue 1-1-2: Power boosting for NB-Iot based Iot-NTN

Agreement:

- Not consider PC1.5 and power boosting for Iot-NTN HPUE.

Issue 1-1-3: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1

- WF (Ad-hoc agreement):

  - Tentative PA Calibration points

    - Option 1:

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc

      - CIM3: 60 dBc

      - [NR ACLR: 31dBc]

    - Option 2:

      - DFT-s-OFDM 1RE (Single Tone)

      - Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

      - [2] or [3] dB post PA loss

      - 0dB MPR

      - Carrier leakage 28dBc

      - IQ image:28 dBc

      - CIM3: 60 dBc

  - For PC1, Option 2 as baseline. Option 1 may be considered by interested parties.

  - Both Testing and simulation may be considered.

Issue 1-1-4: ACLR for eMTC based Iot-NTN for PC2;

Agreement:

- 31dBc ACLR

Issue 1-1-6: MPR assumptions for eMTC based Iot-NTN for PC2

Agreement:

- Reuse eMTC TN PC2 MPR requirements (same to PC3)

Issue 1-1-7: MPR assumptions for eMTC based Iot-NTN for PC1

Agreement:

- Use TN PC2 requirements for Category M1 as starting point

Issue 1-1-8: Basic MOP tolerance (Including eMTC and NB-Iot based) for PC2

- Proposals

  - For PC2:

    - Option 1: +2 / -3dB

      - Aligned with NR-NTN

    - Option 2: +2 / -2dB

      - Aligned with LTE, and more adaptable for narrowband

- WF (Ad-hoc agreement):

  - Use Option 2 as basic MOP tolerance for PC2, including eMTC and NB-IoT based NTN.

  - Foe PC2 NB-Iot based Iot-NTN, further boosting discussed in issue 1-1-10 may have an impact on tolerance based on the basic MOP tolerance

Issue 1-1-9: Basic MOP tolerance (Including eMTC and NB-Iot based) for PC1

- WF (Ad-hoc agreement):

  - For PC1: +2 / -3dB

Issue 1-1-10: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation.

- Proposals

- Option 1: Relax TX MOP tolerance upper limitation for NTN HD-FDD (MTK)

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD. (MTK)

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26x | +2, -2 | 23x | +2, -2 |
| 255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 3: Power tolerances are applied uniformly to HD-FDD and FD-FDD. (Qualcomm)

- WF (Ad-hoc agreement):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - To down-select in next meeting between:

    - Option 1: Increase [1] dB MOP upper tolerance

    - Option 2: Increase [1] dB MOP nominal power (optional with capability signalling)

Issue 1-1-11: PC1 Nominal MOP for NB-Iot based Iot-NTN

- WF (Ad-hoc agreement):

Define the nominal output power for NB-IoT based Iot-NTN PC1 as 31dBm

- Feasibility & Requirements Application

Issue 1-2-1: For NB-Iot based Iot-NTN, PC1 requirements can be applied to:

Agreement:

- For NB-Iot based Iot-NTN, PC1 requirements can be applied to both handheld and non-handheld.

  - FFS whether to limit to the use case with single tone and a limited number of tones

Issue 1-2-2: For eMTC based with half duplex, PC1 requirements can be applied to:

- WF (Ad-hoc agreement)

  - Non-handheld only

Current Status: (For Information)

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | Both* *Note: Tones limitation FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

Other (For information):

- Interesting companies can provide inputs on UE RF components capabilities for FDD and HD-FDD especially for handheld HPUE, including NR-NTN and Iot-NTN in next meeting.

- SAR Related issues

- SAR Related issues

Issue 2-1-2: Whether and how to introduce duty cycle related method.

- Proposals (Not necessarily mutually exclusive):

  - Scheme 1: (vivo)

Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

  - Scheme 2: (oppo)

UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2.

  - Scheme 3: (Samsung)

Observation 3-1: Legacy dutycycle method does not fit the purpose of keeping as much power as possible for NTN scenario, as legacy solution requires UE to fall back 3/6 dBs, which is the power of entire power class.

Observation 3-2: In Rel-19, NTN would not introduce any beam service ratio in time domain in NTN phase3 WI in parallel.

Observation 4-1: To let UE drop certain ratio of uplink opportunities will face the same challenge as other duty cycle method, because there is no NW uplink ratio in time domain in a FDD NTN band. It means if we follow this way, there is no way we could specify or to guide UE behaviors, like how much ratio of uplink should UE drop, and for how long.

Observation 4-2: To let UE go into a Tx UL Gap also faces same challenges as above.

Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method, considering following:

1) P-MPR had been working well and leave space for implementation;

2) without uplink duty ratio from NW side, current solutions on the table (including legacy tdd duty cycle solution) cannot be optimized and clear enough for UE implementation;

3) There are many non-3GPP technologies already implemented in actual SAR compliance, and it helped to maintain power level well enough.

  - Scheme 4: (Qualcomm)

        - A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

        - For X and Y, consider value range from 100ms to 5s with 100ms granularity.

        - Starting time for the indicated values is the transmission time of the report.

        - Send an LS to RAN2 to specify energy headroom reporting.

        - Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2 and 2Tx PC1.5. Capture this allowance into specification.

        - Further evaluate potential specification impact from filter power handling for PC1.

  - Scheme 5: (Ericsson, Sony, Nokia)

Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network.

Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN.

Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony)

    - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

      - DPC = ‘00’: the criteria to report DPC is not met

      - DPC = ‘01’: ΔPPowerClass = 0 dB

      - DPC = ‘10’: ΔPPowerClass = 3 dB

      - DPC = ‘11’: ΔPPowerClass = 6 dB

    - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

      - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

      - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

      - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

      - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

  - Scheme 6: (ViaSat, Inmarsat)

Observation 4: In a non-terrestrial network, if the UE is using a UL duty cycle significant enough to exceed SAR limits, it is likely at the edge of coverage (using a low MCS with lots of parity), in which case, transmit power backoff is not a preferred option as it will almost invariably lead to radio link failure.

Proposal 3: For NTN, extend the existing TN signalling mechanism for UL duty cycle reporting to inform the RAN that it is close to the SAR limit, then the RAN will reduce the number of UL slots allocated to the UE. When the SAR limit has fallen, the UE can signal the improved condition back to the RAN so that the UL slots are picked back up.

- WF

  - Further discuss the proposed schemes related to duty cycle related SAR scheme.

  - Other schemes or no duty cycle related scheme are not precluded.

- Rx Requirments

- RSD (for full duplex eMTC based)

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - Reference sensitivity values

    - 256:

      - 0 dB (MTK, Samsung, Ericsson)

      - 1.1dB (Qualcomm)

    - 255:

      - 0.8dB (MTK)

      - 1.2dB (Qualcomm)

  - No differentiation of handheld and non-handheld for RSD

- WF (Ad-hoc agreement):

  - Reference sensitivity values

    - 256: [0.5] dB

    - 255: 1 dB

- Issues not discussed in this meeting

Issue 1-1-5: ACLR for eMTC based Iot-NTN for PC1;

- Proposals

  - Option 1: Re-use PC3 ACLR 30dBc. (xiaomi, vivo)

  - Option 2: 31dBc ACLR (MTK, Qualcomm)

  - Option 3: 37dBc ACLR (OPPO)

- Recommended WF

  - FFS.

Issue 2-1-1: P-MPR and other SAR solution applicability for UE-types

- Proposals

  - Not explicitly limit the applicability of P-MPR and or any other agreed SAR solutions in the spec for NR-NTN/IoT-NTN to certain UE types (e.g. handheld or non-handheld) (Samsung, Xiaomi, vivo)

- Recommended WF

  - Yes

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for 254/255/256 (oppo, Ericsson)

  - Option 2: 0dB for at least band 256 (Samsung)

  - Option 3: non-zero value as following: (MediaTek)

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | [3.7] |
| 255 | [4.7] |
| 254 | [3.7] |

- Recommended WF

  - TBD. Further explanation from proponents of option 3 needed