3GPP TSG-RAN WG4 Meeting #114bis		R4-2505124

Wuhan, Hubei, China, Apr. 7th – 11th, 2025

Title:	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Response to:	R2-2401572

Release:	Release 19

Work Item:	NR_CADC_SUL_R19

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:

Name:	Aijun CAO

E-mail Address:	ajcao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 thanks RAN2 for the reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx and sharing RAN2 agreements and questions on for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL.

For carrier aggregation involving a low-low band combination where the UL of one band partially overlaps with the DL of the other band, such as CA_n5-n8, RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.:

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

- Option 1: Only support 1UL/2DL CA. Single UL in n5

- Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

2) Dedicated RF filters implementation with partial frequency range

- Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

In particular, Option 1 has already been specified in Rel-18, while RAN4 has decided to specify Option 2 to support full band frequency range of each band in Rel-19. A UE supporting Option 3 enables simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions. However, option 3 is excluded in Rel19, and RAN4 hasn’t achieved the agreements on how to introduce option 3in Rel-20 or a later release.

Moreover, standardizing Option 2 in Rel-19 may require a future-proof signaling design to support that the capabilities for the simultaneous operation in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced, as requested in the original LS R4-2420410.

Additionally, with the new inputs to RAN4#114bis, several new issues and proposals are raised and discussed with the following technical notes for further consideration:

- New issues raised by some companies in this meeting for Option 2 - Interrupt needed for RF front end configuration change may be is required for some implementation - Interrupt needed for RFFE configuration change PA ON/OFF: PA On/Off as the switching between the case1 (n5+n8DL/n5UL) and case2 (n5DL/n5+n8UL) cannot be done with keeping the PAs ON while switching - For the newly raised issues, two solutions are proposed and discussed: - Solution #1 – switching by activation and deactivation: - If UE reports support of 2UL for CA_n5-n8, network only activate either n5DL or n8UL at a time - Change of cases is done in such a way that to no extra interrupts/switching time is needed - Staying in Case 1 longer than Case 2: - FFS: impact on RRM interrupt requirements - If RRM requirement impacts are identified, drop Option 2 in Rel-19 - RAN2 capability signaling is required - Solution #2 - New capability: introduce a UE capability of switching gap (i.e., switching period and location) to cover necessary switching period including RFFE configuration switching, PA On/Off etc., - Can be the same for both directions (n5DL -> n8UL, n8UL->n5DL), or one for each direction - Network accommodates the switching gap by scheduling - Cross-carrier scheduling might be needed (e.g., from Case 2 with only 1 DL n8DL to Case 1 with two DL) - No RAN1 spec change is required - Further improvement, e.g., via L-L CA via switching is not precluded in Rel-20 or later - Companies are encouraged to evaluate if solution #1 or solution #2 is feasible to be used to specify n5-n8 2UL/2DL CA in Rel-19

Regarding the questions raised, in-line answers could be found below:

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

Answer 1:      If only Option 2 is assumed, it is impossible to implement simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, and also it is impossible for a UE to support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands.

However,as stated in the above, it’s possible for a UE implemented with Option 3 to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or  simultaneous Rx/Tx in some part(s) of the overlapping FDD bands.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

Answer 2:      Even if Option 3 may be specified in a later release to enable a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations), RAN4 does not identify any additional UE capabilities for the support/requirement of other features. For example, frequency gap/guard band is not considered as a solution to enable DL_CA_n5A-n8A with UL CA_n5A-n8A. Instead, RAN4 will introduce a new NOTE to pre-define 2UL/2DL CA_n5-n8 in the Rel-19 TS38.101-1 specification, which is: NOTE 15: Uplink is only in n5 for one uplink CA_n5-n8. 2UL/2DL CA_n5A-8A is supported using non-concurrent n5 DL and n8 UL.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into account targeting a future-proof signalling design for supporting CA_n5-n8 with 2UL/2DL.

# 3	Dates of next TSG-RAN WG2 meetings

3GPP TSG RAN WG4#115	May 19th – 23rd, 2025			Malta