3GPP RAN4 Meeting #114bis		R4-2505147

Wuhan, China, April 7 – 11, 2025

Agenda item:	7.7.1

Source:	Apple

Title:	TP to TR38.768 on operator provided scenarios for low band aggregation via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The discussion to clarify scope of the NR low band carrier aggregation via switching work item was initiated by the Telus contribution to RAN #104 [1] and was updated and revised in several motivation papers [2-4].  The most recent work item description revision was approved in RAN #107 [5].  The series of motivation papers provide useful background of operator-proposed scenarios.

This contribution provides a text proposal to TR38.768 [6] with three changes:  a reflection of the study item scope (Clause 1), an update of the references (Clause 2), and a summary of the operator-provided scenarios (Clause 4).

# 3	References

[1]	RP-241537, “Switching based low-low carrier aggregation,” TELUS, 3GPP RAN #104

[2]	R4-2411175, “Motivation for UL Tx switching for FR1 intra-band non-contiguous UL CA in Rel-19,” Apple, 3GPP RAN4 #112

[3]	R4-2417690, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19,” AT&T, Apple, Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc, Boost Mobile Network, Murata, Samsung, Ericsson, Qorvo, Nokia, Huawei, HiSilicon

[4]	RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19,” AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE

[5]	RP-250247, “Revised WID on Low NR band carrier aggregation via switching,” AT&T, 3GPP RAN #107

[6]	R4-2502875, “Skeleton TR 38.768 on low NR band carrier aggregation via switching v0.0.2,” Apple, 3GPP RAN4 #114

# 2	Text Proposal

<< begin change 1 >>

# 1	Scope

This Technical Report captures the outcomes from the Work Item on NR low band carrier aggregation via switching.  The scope of the Work Item is to introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

<< end change 1 >>

<< begin change 2 >>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

<< end change 2 >>

<< begin change 3 >>

# 4	Low NR band aggregation scenarios

Operator interest for aggregating low band spectrum has always been high during the entire history of the carrier aggregation feature, spanning LTE and NR specifications.  Considering the practicalities of handset RF front end architectures available in the market today, the operators have requested 3GPP to consider specifying a solution based on a switching scheme which can potentially accommodate the related limitations.

The following observations motivate the effort undertaken by this Work Item:

-	The amount of mid-band spectrum an operator holds is typically 10-20 times greater than their low-band spectrum holdings.

-	Mid-band spectrum is more effective closer to cell sites, while low-band spectrum propagates farther, making it more useful at greater distances from sites.

-	Low-band spectrum carries significant traffic volumes in urban (indoor) and rural areas, leading to congestion that severely degrades customer experience.

-	This issue is exacerbated by the fact that low-band SDL bands, which reach most poor coverage areas, cannot be paired with an UL, rendering them useless.

-	Utilization of SDL bands via Low-Low band CA, where one low band serves as an SDL, could potentially solve this problem. However, this solution doesn't exist due to OEM challenges in supporting it.

In the context of the scope of this Work Item, Table 4-1 below illustrates the applicable operator-provided scenarios.

Table 4-1: Operator-provided scenarios for NR low band aggregation via switching

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n5A-n29A |  | (1), (2) |
| CA_n12A-n29A |  | (3) |
| CA_n14A-n29A |  |  |
| CA_n28A-n67A |  | (4), (5) |
| CA_n29A-n71A |  | (6), (7) |
| NOTE 1: CA_n5A-n29A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 2: Fractional bandwidth of a single antenna to support this combination is 22% and represents a practical implementation challenge NOTE 3: There is no incumbent narrowband service in the band gap between n29 and n12 DL (728~729 MHz) in Canada NOTE 4: In Europe band n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz DL NOTE 5: 3GPP will introduce requirements for band n28 assuming full band duplexer architecture in Rel-19 NOTE 6: CA_n29A-n71A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 7: Fractional bandwidth of a single antenna to support this combination is 16.5% and represents a practical implementation challenge |  |  |

<< end change 3 >>