3GPP TSG-RAN WG4 Meeting #114bis                                                       	 R4-2505229

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.19.5

Source:	OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

Title:	Updated simulation assumptions for AIML CSI prediction

Document for:	Approval

- Introduction

In this contribution, according to companies’ inputs in RAN4 #114bis meeting, updated simulation assumptions for AI/ML based CSI prediction are proposed.

- Updated simulation assumptions for AIML based CSI prediction

For information

Based on the current 1st round simulation results (as shown in Annex), RAN4 still needs to address two issues:

- How to reduce the discrepancies in simulation results among different companies during the "step-1" phase.

- How to conduct simulations for the "step-2" phase.

Regarding the discrepancies in simulation results, encourage companies to further check the following aspects:

  - the dataset variations across companies are evident, e.g., the results obtained using "sample and hold" already show notable differences.

  - potential overfitting issues may arise from the division of training and test sets. Different assumptions may yield different evaluation outcomes, e.g., when the training and test sets are sampled from the same time-varying channel (at different time instants), compared to scenarios where the training and test sets are independently generated through same simulation assumptions.

  - the use of different models by companies undoubtedly contributes to the misalignment in results.

Regarding updates to the simulation assumptions for AIML-based CSI prediction (for step-2):

Based on the current progress of RAN4’s CSI prediction discussion and proposals for AI-based CSI prediction simulation assumptions in RAN4 #114bis, following aspects should be considered for updating the simulation assumptions:

  - Codebook configuration for Rel-18 eTypeII-Doppler, e.g., N4 = 1.

  - CSI feedback overhead, e.g., PC7 as baseline.

  - CSI-RS configuration, where proposals in R4-2504539 could be adopted to initiate subsequent simulations.

  - Practical channel estimation should be utilized in “step-2”.

  - The original “step-2” will be split into updated “step-2” and “step-3”.

  - Use SGCS as the KPI for updated “step-2” to check whether alignment could be achieved (with Ideal DL channel estimation),

  - Use TPs as the KPI for updated “step-3” to check whether alignment could be achieved (with practical DL channel estimation)

Way forward for next step simulation for CSI prediction

  - For step-1,

    - Interested companies are encouraged to provide the 2nd round of CSI prediction simulation results in RAN4#115 meeting.

  - For step-2,

    - Simulation assumptions for step-2 are updated in RAN4#114bis meeting.

    - Interested companies are also encouraged to provide the 1st round of CSI prediction simulation results (SGCS as KPI, SGCS between the eigenvectors of predicted PMI derived from the ideal DL channel estimation and ground-truth eigenvectors), in RAN4#115 meeting.

  - For step-3,

    - CSI prediction simulation (TPs as KPI, with practical DL channel estimation) is postponed to RAN4#116 meeting.

    - Note: Simulation assumptions for CSI prediction (TPs as KPI, with practical DL channel estimation) will be updated further according to the progress of RAN1&RAN4 in RAN4#115. Companies are encouraged to check the updated assumptions and input suggestions/comments in RAN4#115 meeting.

  - Companies are encouraged to provide the “sample and hold” results for above simulations for steps 1 and 2.

  - Companies can also provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models.

Table 1: Updated simulation assumptions for AIML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | Option1:32(N1,N2)=(4,4) as defined in Rel-15 type1 Option2:16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | FFS: following CSI-RS configuration: - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). - For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 FFS: following test setup (timeline): For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| SNR assumption | Only for step-3 [FFS, e.g., Option1: claimed by companies for step-2 first round simulation(TP as KPI) Option2: Example: [-6:2:10] dB Option3: other suggestions ] |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS: - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-2 or performance requirements) |
| KPI | Step-1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Step-2: - check SGCS after CSI feedback Step-3: - check TPs after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads. |

Table2: (For information) Test setup for FDD test setup with Periodic  CSI-RS configuration (based on R.PDSCH.1-24.1 FDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 16 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layer |  | 2 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot | Bits | 24072 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot | CBs | 3 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for FDD test setup with Periodic CSI-RS configuration |  |  |  |  |  |  |

Table3: (For information) Test setup for TDD test setup with Periodic  CSI-RS configuration (based on R.PDSCH.2-38.1 TDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 24 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layer |  | 2 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Non CSI-RS Slot i | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Non CSI-RS Slot | CBs | 6 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for TDD test setup with Periodic CSI-RS configuration |  |  |  |  |  |  |

Annex: First round simulation results(step-1) for AIML based CSI prediction

Table 1. First round simulation results(step-1) for AIML based CSI prediction

|  | gNB TX antennas | 20Hz |  |  |  |  | 50Hz |  |  |  |  | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI |
| Qualcomm | 16 |  |  |  |  |  |  | 0.93725 | 0.9572 | 0.9173 | 0.2430 for Rank2-ave 0.3127 for Rank2-L1 0.1734 for Rank2-L2 |  |  |  |  |  |
| MediaTek | 16 |  | 0.96285 | 0.9731 | 0.9526 |  |  | 0.92855 | 0.9496 | 0.9075 |  |  | 0.81235 | 0.8636 | 0.7611 |  |
|  | 32 |  | 0.96625 | 0.9774 | 0.9551 |  |  | 0.92935 | 0.9525 | 0.9062 |  |  | 0.7921 | 0.8501 | 0.7341 |  |
| Apple | 16 | 0.98 |  |  |  | 0.85 | 0.87 |  |  |  | 0.53 | 0.45 |  |  |  | 0.45 |
| CMCC | 16 |  |  |  |  |  | 0.9249 |  |  |  |  |  |  |  |  |  |
| ZTE | 16 | 0.96 |  |  |  | 0.82 |  |  |  |  |  | 0.94 |  |  |  | 0.81 |
| vivo | 32 | 0.993 |  |  |  |  | 0.902 |  |  |  |  | 0.298 |  |  |  |  |
| OPPO | 32 | 0.9994 |  |  |  |  | 0.9032 |  |  |  |  |  | 0.2846 |  |  |  |
| Huawei, HiSilicon | 16 | 0.9985 | 0.998 |  |  |  | 0.8673 | 0.8166 |  |  |  | 0.3011 | 0.2334 |  |  |  |
| Ericsson | 32 | 0.92 | 0.905 | 0.92 | 0.89 |  | 0.78 | 0.72 | 0.78 | 0.66 |  | 0.31 | 0.24 | 0.31 | 0.17 |  |
| Samsung | 16 | 0.996 | 0.993 | 0.996 | 0.99 | 0.807 for Rank1 0.7365 for Rank2-ave 0.807 for Rank2-L1 0.666 for Rank2-L2 | 0.995 | 0.992 | 0.995 | 0.989 | 0.363 for Rank1 0.2835 for Rank2-ave 0.363 for Rank2-L1 0.204 for Rank2-L2 | 0.995 | 0.992 | 0.995 | 0.989 | 0.185 for Rank1 0.1635 for Rank2-ave 0.185 for Rank2-L1 0.142 for Rank2-L2 |
|  | 32 | 1 | 1 | 1 | 1 | 0.874 for Rank1 0.8525 for Rank2-ave 0.874 for Rank2-L1 0.831 for Rank2-L2 | 1 | 1 | 1 | 1 | 0.589 for Rank1 0.5545 for Rank2-ave 0.589 for Rank2-L1 0.520 for Rank2-L2 | 1 | 1 | 1 | 1 | 0.467 for Rank1 0.44 for Rank2-ave 0.467 for Rank2-L1 0.413 for Rank2-L2 |