3GPP TSG-RAN4 Meeting #114bis					R4-2505231

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	WF on PC2 RedCap UE RF requirements

Agenda item:			7.6.2

Document for:	Approval

Background

This WF captures the agreements for RF Rx requirements for PC2 RedCap UE in FDD bands.

Wayforward

Issue 1-1-1: REFSENS for 1Rx in FD-FDD

Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD

- Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges

  - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB.

  - Companies to evaluate REFSENS for other bands.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

- Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions

  - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1.

  - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A.

  - The format in which the REFSENS is shown in the specs is TBD pending evaluations.

Way forward: It is encouraged for companies to provide analysis for option 1 and 2.

Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.