3GPP TSG-RAN WG4 Meeting #114bis	R4-2505232

# Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:			5.1.1

Source:	Huawei, HiSilicon

Title:	WF on Introduction FR2 Power Class 8

Document for:	Approval

Way forward

Issue 1: The assumed UE RF architecture

- Way Forward:

  - Option 1: One panel with 1x2 antenna array

  - Option 2: Two panels with 1x2 antenna array for each panel

Issue 2: The UE baseband processing capability

- Way Forward:

  - Same as PC3, i.e. support 200MHz CBW, CA and UL-MIMO

Issue 3: The UE RRM capability

- Way Forward:

  - Same as PC3

Issue 3: UE capability and applicable UE types

- Way forward:

  - UE capability: define UE capability signalling of support FR2 PC8

  - Applicable UE type:

    - Option 1; FFS in RAN4 specification. RAN4 can review it in the future to decide the applicable UE type based on the millimetre wave industry need.

    - Option 2: No restriction in RAN4 specification.

Issue 3: Introduction of UE FR2 Power Class 8

- Way Forward:

  - Option 1: To make the PC8 proposal more palatable to all stakeholders, RAN4 to consider adopting the following requirements for n257/n258/n261:

    - min pk. EIRP: 16.4 dBm

    - EIRP spherical coverage requirement:

      - if Option 1 in Issue 1 is adopted

        - Option 1a: 5.5dBm @ 50th% ile direction

      - if Option 2 in Issue 1 is adopted

        - Option 2a: 7.5dBm @ 50th% ile direction

        - Option 2b: 11.5 dBm @ 50th% ile direction

  - Option 2: Introduce FR2 power class 8 with the access control scheme and with the min peak EIRP = 16.4 dBm and EIRP spherical coverage@50%-ile CDF = 5.5 dBm

    - intraFreqReselectionFR2PC8

    - cellBarredFR2PC8

  - Option 3: Option 1 + Option 2

  - Option 4: PC8 is not introduced

- Reference

- R4-2417183, WF on FR2 Power Class 8, RAN4#112bis, Huawei, HiSilicon

- R4-2414420, WF on introduction of new FR2 Power Class 8, RAN4#112, Huawei, HiSilicon

- R4-2415629, Updated system simulation assumption for introduction of FR2 PC8, Huawei, HiSilicon