3GPP TSG-RAN WG4 #114bis	R4-2503124

Wuhan, China, April 7-11, 2025

Agenda item:	7.25.2.1		On-demand SSB SCell operation

Source: 	Qualcomm Incorporated

Title: 	Views on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. Several agreements have been made for on-demand SSB, and additional open issues have been identified in [2].

In this paper we provide our views for the open issues identified in the way forward in [2] in RAN4 #114.

# Discussion

Good progress was made in RAN4 #114 on several aspects of OD-SSB [2]. In the following subsections we address the open topics.

## OD-SSB based deactivated SCell L3 Measurement

In the previous meetings it was already agreed that there will be a fast measurement window (FMW) for doing measurements based on OD-SSB periodicity. After the fast measurement window, the UE shall meet legacy deactivated SCell measurement requirements. Regarding the length of the fast measurement window, the following agreement has been made in RAN4 #114.

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

The length of the FMW depends on whether the cell was already detected or is still to be detected. Figure 2-1 and Figure 2-2 show two scenarios for Case #1 when only OD-SSB is present but no always-on SSB depending on how the OD-SSB is triggered by the network. Two scenarios can be distinguished:

- Scenario A: OD-SSB is triggered once but never deactivated

- Scenario B: OD-SSB is triggered for a finite duration with a gap in between

For Case #1 after the SCell is added, the OD-SSB is activated by MAC CE. In this case, the UE should follow OD-SSB periodicity. Since in this case the cell has not yet been detected, the cell identification period should be taken into account according to the agreement above. However, once the report is sent to the network, there is no need any more to measure in fast measurement mode. Afterwards the UE should fall back to legacy measurement mode to avoid increase in UE power consumption. This is in particular important in scenario A to avoid that the UE always stays in fast measurement mode.

Observation 1: If one valid measurement report in fast mode is sent to the network there is no need any more to continue measurements in the fast mode. This is important when the network activates the OD-SSB but does not deactivate it again.

Once the measurement report has been sent to the network, the SCell is known. Or equivalently, in case that the SCell is already known to the network, a valid L3 measurement report has already been sent. This is option 2 for the start point of T2: ‘Option 2: the latter point of L3 measurement reporting for FMW and Tend‘ in [2]. Then additional measurement reports with faster measurement period are not needed anymore and the legacy requirements based on measCycleScell can be reused. For scenario A that means that the UE would never do fast measurements anymore although OD-SSB are continuously sent by the network.

Observation 2: For a known deactivated SCell, no faster measurements are required by the network

Proposal 1: Once the UE has sent a valid measurement report, the UE should be allowed to ignore the OD-SSB periodicity and fall back to legacy measurement mode. This is option 2: the latter point of L3 measurement reporting for FMW and Tend in [2].

Figure 2-1: Scenario A – OD-SSB in between two FMW

Figure 2-2: Scenario B – No OD-SSB in between two FMW

If the OD-SSB in scenario A is not deactivated by the network again, it becomes in its behaviour like an always-on OD-SSB. This configuration possibility is allowed based on the RAN1 agreement. In this case there is no need any more to measure again in the fast mode and OD-SSB periodicity should not be followed for measurements.

Of course, the network can also send a deactivation command or activate the OD-SSB for a finite duration as it is shown in scenario B. In this scenario, the UE sends a valid L3 measurement report at a certain point in time although still some OD-SSB are sent by the network afterwards. Those OD-SSB should not be measured in fast mode. Afterwards in this scenario B there is a time gap without any OD-SSB before OD-SSB is activated again by the network. If the SCell is known, the network should not be allowed to trigger another measurement report in fast measurement mode within 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) after a measurement report has been sent. Figure 2-2 illustrates the triggering of the fast measurement for the case that the SCell is already detected. 5*max(measCycleSCell, DRX cycle) is the minimum time between L3 reports that allows to keep the SCell known. There is no need to trigger the fast measurement mode even faster since the SCell is already known.

Proposal 2: The network should not be allowed to trigger another measurement in fast measurement mode within 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) if the SCell is known (sub-issue 1-2 minimal time of T2).

Another open point related to L3 measurements for deactivated SCell is how to handle scenarios with multiple carriers. It has been decided in RAN4 #114 that RAN4 should prioritize scenarios with a single OD-SSB carrier and multiple legacy SSB carriers in deactivated SCell measurement. The open question for this scenario is how to determine the CSSF, i.e. either to prioritize the OD-SSB carrier or to treat OD-SSB carriers and the carriers with legacy SSB equally.

Agreement:

RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement.

- FFS multiple OD-SSB carriers

Since the default assumption in RAN4 is that only two searchers / measurement entities are available and one is reserved for PCC, prioritizing the OD-SSB carrier comes at the expense of measurement delay of the SCells with legacy SSB, independent whether they are deactivated or activated. In addition, it needs to be taken into account that there is a non-trivial impact to implementation and specification when the OD-SSB carrier shall get higher priority in the measurement. The scheduling of measurements in the UE for legacy SSB happens X ms before the legacy SSB is received over the air. If now OD-SSB on another carrier is triggered Y ms < X ms before over the air reception of legacy SSB, either priority of OD-SSB measurements cannot be granted anymore or an already scheduled measurement based on legacy SSB needs to be cancelled again. Since cancellation cannot happen infinitely fast, cancellation is only feasible for some Y < X but not for all. These corner cases need to be identified by RAN4 and excluded from any requirement.

Observation 3: Since the default assumption in RAN4 is that only two searchers / measurement entities are available and one is reserved for PCC, prioritizing the OD-SSB carrier comes at the expense of measurement delay of the carriers with legacy SSB, independent whether they are deactivated or activated.

Observation 4: It is technically not feasible that OD-SSB gets higher priority than legacy SSB measurements in all possible scenarios. There are corner cases that need to be identified by RAN4 to ensure that potential requirements do not apply for them.

Hence, we propose that measurements based on OD-SSB and legacy SSB on other carriers are treated equally, and no higher priority is given for the OD-SSB carrier. This is option 2 in Sub-issue 3-2 in [2].

Proposal 3: Measurements based on OD-SSB and legacy SSB on other carriers should be treated equally, and no higher priority is given for the OD-SSB carrier. This is option 2 in Sub-issue 3-2 in [2].

Another open issue is how to handle L3 measurement in deactivated SCell for Case #2 when both OD-SSB and always-on SSB are present. In RAN4 #114, the following two options were identified:

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

Agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

In RAN4 #114, RAN4 agreed on the following scenarios to start with for Case #2. In both scenarios, the center frequency is identical between OD-SSB and always-on SSB.

As we already mentioned above, the main purpose of fast L3 measurements is to send a measurement report to the network as quickly as possible. That allows the network to consider the cell to be known. If the cell is already known, then it makes sense that the UE follows legacy requirements based on measCycleSCell. Hence, option 2 above is preferred. Option 1 allows the network to trigger fast measurements in the UE even if the cell is known. We do not think that this additional acceleration of sending the measurement report should be supported since it comes at the expense of considerable increase in UE power consumption if this approach is triggered very often by the network.

Observation 5: ‘Option 1: The same approach in Case 1 can be reused’ allows the network to trigger fast measurements in the UE even if the cell is known. This comes at the expense of considerable increase in UE power consumption if this approach is triggered very often by the network.

Proposal 4: RAN4 should agree for deactivated SCell measurements for Case #2 to Option 2: If the serving cell is unknown, the same approach in Case 1 can be reused. If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

The following scenarios for Case #2 have been agreed so far in RAN4.

Agreement:

RAN4 to start from the following scenarios:

- Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

- The scenarios can be updated based on other WGs’ progress.

In addition, in RAN4 #114 it was further agreed that the measurement requirement is based on the effective measurement periodicity. This agreement was only reached for the case of scenario 4 if the effective measurement periodicity is half of the OD-SSB periodicity.

Agreement:

- In scenario 4, the measurement requirement is defined based on the effective measurement periodicity.

  - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - The effective measurement periodicity is half of OD-SSB periodicity

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

This agreement essentially means that RAN4 will define measurement requirements for Case #2 that are based on the combination of OD-SSB and always-on SSB for scenario 4 in case the effective measurement periodicity is half of the OD-SSB periodicity. However, this does not necessarily imply that the UE has to combine measurements based on OD-SSB and always-on SSB, but it is left to UE implementation whether any combining is needed in order to meet the accuracy requirements.

Observation 6: RAN4 agreed in RAN4 #114 that measurement requirements for Case #2 will be based on the combination of OD-SSB and always-on SSB in case the effective measurement periodicity is half of the OD-SSB periodicity.

Observation 7: RAN4 has not agreed that the UE has to combine measurements based on OD-SSB and always-on SSB, but it is left to UE implementation whether any combining is needed in order to meet the accuracy requirements.

In RAN1 #120, RAN1 discussed the relation in terms of time domain location between OD-SSB and always-on SSB and sent a LS to RAN4 [5]. We first focus on the case that the center frequencies are identical. For this case, the LS provides the following information below.

Alt Time-C1 is an extension of the previous RAN4 agreement that the effective measurement periodicity is half of the OD-SSB periodicity. Now in Alt Time-C1, the union of always-on SSB and OD-SSB continues to have a periodic effective time domain pattern. Hence, the requirement can be based on the combining of both always-on SSB and OD-SSB and follow the new effective periodicity.

Observation 8: Alt Time-C1 is an extension of the previous RAN4 agreement that the effective measurement periodicity is half of the OD-SSB periodicity.

Proposal 5: The requirement for Alt Time-C1 can be based on the combining of both always-on SSB and OD-SSB and follow the new effective periodicity.

Alt Time-C2 results in a non-periodic time domain pattern of always-on SSB and OD-SSB. We do not see a strong use case for such a configuration. It can result into an effective SSB burst periodicity less than 5 ms which is not a supported periodicity by always-on SSB, OD-SSB and SSB adaptation. Hence, we prefer not to define requirements for Alt Time-C2.

From RAN1 LS:

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Proposal 6: RAN4 should not define requirements for Case #2 when the union of always-on SSB and OD-SSB yields to a non-periodic time domain pattern (Alt Time-C2).

Finally, RAN1 agreed that always-on SSB and OD-SSB can also be located on different non-overlapping frequencies within the same BWP. This is the case when always-on SSB is a CD-SSB on the synchronization raster. In this case OD-SSB is not on the sync-raster. RAN1 further agreed in RAN1 #120 that this scenario is subject to a separate UE capability and the UE is not required to measure both always-on SSB and OD-SSB:

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Observation 9: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, RAN1 has agreed the UE is not required to measure both SSBs.

In this case the UE should follow Proposal 4 for deactivated SCell L3 measurements and requirements should not be defined based on the combination of measurements.

Proposal 7: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, Proposal 4 can be applied as well, and requirements should not be defined based on the combination of measurements.

## MAC CE Timeline

In [3], RAN4 replied to a RAN1 LS regarding the MAC CE timeline as follows:

- RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective.

  - Note 1: This does not imply UE shall be ready to process the OD-SSB from time instance A from RAN4 requirement perspective.

- Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

Figure 2-3 illustrates the MAC CE timeline. A MAC CE triggered OD-SSB is received in slot #n and the corresponding PDSCH is acknowledged in slot #(n+m). In this slot, the MAC CE is processed and the information contained in the MAC CE is available in the UE. This delay is unrelated to the OD-SSB and applies to each MAC CE. In case MAC CE triggers OD-SSB, the delay of m slots till the HARQ feedback is sent to the network can remain unchanged in the timeline for MAC CE triggered OD-SSB. According to the regular MAC CE timeline, additional 3 ms are left to the UE to get ready for the reception of the OD-SSB. Hence, Tmin is the minimal time when the UE needs to be ready to process the OD-SSB. These additional 3 ms apply so far in 3GPP for A-TRS and aperiodic CSI-RS. However, the RAN1 LS extends the time of 3 ms to SSB. Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side are usually used for very different purposes by the UE, it cannot be easily assumed that the same timeline can be applied for SSB as well.

Figure 2-3: MAC CE Timeline for OD-SSB

It is reflected in the Reply LS to RAN1 that the UE may need more time to be ready to process OD_SSB. RAN4 should discuss this additional processing/preparation time for OD-SSB [3].

Observation 10: The UE may need more time than given by the regular MAC CE timeline for aperiodic signals to be ready to process OD-SSB.

Proposal 8: RAN4 should discuss this additional processing/preparation time for OD-SSB reception further.

The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB. For example, in the Reply LS by RAN1 [4] there is the following comment:

Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?):

- …….

- …….

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

That means potentially there are up to 64 different OD-SSBs with different beams within an OD-SSB burst. In this case the UE needs to prepare for up to 64 different Rx beams. This may take more time than preparing for the reception of an OD_SSB burst with only 4 different beams. This scenario with only four beams in FR2 may take again more preparation time than for FR1 when no analog Rx beam needs to be prepared. RAN4 should discuss whether a single margin is introduced that allows to cover the worst-case preparation time or whether multiple margins should be introduced that reflect different configuration scenarios.

Observation 11: The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

Proposal 9: RAN4 should discuss whether a single margin is introduced that allows to cover the worst-case preparation time or whether multiple margins are introduced that reflect different configuration scenarios.

The discussion above applies for the MAC CE that activates OD-SSB. In addition, RAN1 has agreed that also a MAC CE is introduced that allows deactivating the OD-SSB. For this MAC CE a timeline applies as well till when the UE can stop processing the OD-SSB. If the given timeline is too short, the UE may still try to process an OD-SSB that is not sent by the network anymore. Depending on the purpose of the OD-SSB processing, the impact can be severe if the UE makes use of a measurement based on a signal that is not present. Generally, the timeline for deactivating and activating OD-SSB may not be identical. However, for simplicity it can be assumed that both timelines are identical.

Observation 12: For a MAC CE deactivating OD-SSB a timeline applies till when the UE can stop processing the OD-SSB as well. Generally, the timeline for deactivating and activating OD-SSB may not be identical.

Proposal 10: To keep the specification simple, RAN4 should consider defining the same timeline for activating and deactivating OD-SSB.

## OD-SSB based SCell Activation

In RAN4 #114, the following agreements were achieved regarding L1 measurements before, during and after SCell activation. Before SCell activation, no L1 measurements are supported, but during and after SCell activation, L1 measurements are supported. In addition, it has been agreed that the L1 measurements are based on OD-SSB periodicity during SCell activation and after SCell activation when no DRX is configured.

The remaining open question is how to handle L1 measurements after SCell activation when DRX is configured. The way forward has been captured as shown below.

Agreement:

- RAN4 not to support OD-SSB based L1 measurements for deactivated Scell.

Issue 4-2: L1 measurement after SCell activation

Agreement:

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

- RAN4 to support OD-SSB L1 measurement during SCell activation.

  - The requirement when DRX is configured.

    - Based on OD-SSB periodicity

Wayforward:

- RAN4 to support OD-SSB L1 measurement after SCell activation.

  - FFS the requirement when DRX is configured.

    - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi)

    - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung)

    - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT)

    - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

Option 2 says that L1 measurement is always based on OD-SSB periodicity. We do not think that Option 2 is preferred since it requires the UE to measure based on OD-SSB periodicity even if DRX is configured. This increases the power consumption of the UE significantly. Instead, the legacy requirement can be reused where TSSB refers to the OD-SSB periodicity.

Observation 13: Option 2 is not preferred since it may require the UE to measure based on OD-SSB periodicity even if DRX is configured. This potentially increases the power consumption of the UE.

Proposal 11: L1 measurements after SCell activation should follow the legacy requirements based on DRX where TSSB refers to the OD-SSB periodicity.

In RAN4 #114, it was agreed that RAN4 will define requirements for direct SCell activation at SCell addition. This will be handled during the CR phase.

Agreement:

- RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

In our view, RAN4 should also focus on multiple SCell activation since multiple SCell activation is a mandatory feature. The other Scell activation approaches are optional features in Rel-16, Rel-17 and Rel-18 with less relevance in our view given current deployment status. RAN4 should also distinguish in those features whether OD-SSB is used for deactivated SCell measurement or acceleration of SCell activation. For Rel-17 Fast SCell activation, an aperiodic TRS is applied. For that feature, OD-SSB should only be used for deactivated SCell measurement.

Proposal 12: RAN4 should focus next on OD-SSB based SCell activation delay requirements for multiple SCell activation since multiple SCell activation is a mandatory feature.

Proposal 13: OD-SSB based SCell activation delay requirements should not be defined for Rel-17 Fast SCell Activation.

## Miscellaneous

TS 38.133 allows interruptions on PCell or activated SCell during measurements on deactivated SCell when the configured measCycleSCell is 640 ms or longer. RAN4 #114 identified an open issue that it should be investigated whether OD-SSB has impact on the allowed interruption duration.

Issue 6-6: Interruption

Open Issue:

- RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated Scell.

For a long measCycleSCell, an interruption on PCell and activated SCell are accepted in TS 38.133 when the RF chain for deactivated SCell is brought up/down to allow power savings on the UE side. We do not see that additional interruptions are needed or should be allowed, and the requirement can be reused for OD-SSB. Probably, the wording in section 8.2 of TS 38.133 needs to be updated since measCycleSCell may not be meaningful all deployment scenarios of OD-SSB anymore.

Observation 14: Additional interruptions on PCell or activated SCell during measurements on deactivated SCell are not needed for OD-SSB.

Proposal 14: The wording in section 8.2 of TS 38.133 should be updated since measCycleSCell may not be meaningful for all deployment scenarios of OD-SSB anymore.

Regarding OD-SSB based neighbor cell measurements it has been agreed in Issue 7-2 in RAN4 #114 that for FR1 legacy requirements can be followed and no change in specification is needed. For FR2, it has been identified that further discussion is needed.

Issue 7-2: OD-SSB based Neighbour cell measurement

    - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell

Agreement:

  - For Scenario 1a: No RAN4 spec. change.

    - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell

Agreement:

  - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB.

    - The agreement can be revisited if not aligned with the further progress in other WGs.

  - Further discuss for FR2.

    - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell

Agreement:

  - For FR1, follow legacy requirement for neighbour cell measurement.

    - The agreement can be revisited if not aligned with the further progress in other WGs.

  - Further discuss for FR2.

In our opinion, neighbor cell measurements for FR2 should follow legacy requirements as well and no change in Rx beam sweeping is required.

Proposal 15: Neighbor cell measurements for FR2 should follow legacy requirements as well and no change in Rx beam sweeping is required.

Regarding the Issue 7-3, we do not prefer to introduce a separatee L3 measurement report for OD-SSB. Instead, legacy measurement reporting should be reused.

Issue 7-3: OD-SSB based L3 reporting

Wayforward:

- RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.

Proposal 16: No OD-SSB based L3 measurement reporting should be introduced.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114. Our observations and proposals are summarized as follows:

Observation 1: If one valid measurement report in fast mode is sent to the network there is no need any more to continue measurements in the fast mode. This is important when the network activates the OD-SSB but does not deactivate it again.

Observation 2: For a known deactivated SCell, no faster measurements are required by the network

Proposal 1: Once the UE has sent a valid measurement report, the UE should be allowed to ignore the OD-SSB periodicity and fall back to legacy measurement mode. This is option 2: the latter point of L3 measurement reporting for FMW and Tend in [2].

Proposal 2: The network should not be allowed to trigger another measurement in fast measurement mode within 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) if the SCell is known (sub-issue 1-2 minimal time of T2).

Observation 3: Since the default assumption in RAN4 is that only two searchers / measurement entities are available and one is reserved for PCC, prioritizing the OD-SSB carrier comes at the expense of measurement delay of the carriers with legacy SSB, independent whether they are deactivated or activated.

Observation 4: It is technically not feasible that OD-SSB gets higher priority than legacy SSB measurements in all possible scenarios. There are corner cases that need to be identified by RAN4 to ensure that potential requirements do not apply for them.

Proposal 3: Measurements based on OD-SSB and legacy SSB on other carriers should be treated equally, and no higher priority is given for the OD-SSB carrier. This is option 2 in Sub-issue 3-2 in [2].

Observation 5: ‘Option 1: The same approach in Case 1 can be reused’ allows the network to trigger fast measurements in the UE even if the cell is known. This comes at the expense of considerable increase in UE power consumption if this approach is triggered very often by the network.

Proposal 4: RAN4 should agree for deactivated SCell measurements for Case #2 to Option 2: If the serving cell is unknown, the same approach in Case 1 can be reused. If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

Observation 6: RAN4 agreed in RAN4 #114 that measurement requirements for Case #2 will be based on the combination of OD-SSB and always-on SSB in case the effective measurement periodicity is half of the OD-SSB periodicity.

Observation 7: RAN4 has not agreed that the UE has to combine measurements based on OD-SSB and always-on SSB, but it is left to UE implementation whether any combining is needed in order to meet the accuracy requirements.

Observation 8: Alt Time-C1 is an extension of the previous RAN4 agreement that the effective measurement periodicity is half of the OD-SSB periodicity.

Proposal 5: The requirement for Alt Time-C1 can be based on the combining of both always-on SSB and OD-SSB and follow the new effective periodicity.

Proposal 6: RAN4 should not define requirements for Case #2 when the union of always-on SSB and OD-SSB yields to a non-periodic time domain pattern (Alt Time-C2).

Observation 9: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, RAN1 has agreed the UE is not required to measure both SSBs.

Proposal 7: For the case that always-on SSB and OD-SSB are on different frequencies within the same BWP, Proposal 4 can be applied as well, and requirements should not be defined based on the combination of measurements.

Observation 10: The UE may need more time than given by the regular MAC CE timeline for aperiodic signals to be ready to process OD-SSB.

Proposal 8: RAN4 should discuss this additional processing/preparation time for OD-SSB reception further.

Observation 11: The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

Proposal 9: RAN4 should discuss whether a single margin is introduced that allows to cover the worst-case preparation time or whether multiple margins are introduced that reflect different configuration scenarios.

Observation 12: For a MAC CE deactivating OD-SSB a timeline applies till when the UE can stop processing the OD-SSB as well. Generally, the timeline for deactivating and activating OD-SSB may not be identical.

Proposal 10: To keep the specification simple, RAN4 should consider defining the same timeline for activating and deactivating OD-SSB.

Observation 13: Option 2 is not preferred since it may require the UE to measure based on OD-SSB periodicity even if DRX is configured. This potentially increases the power consumption of the UE.

Proposal 11: L1 measurements after SCell activation should follow the legacy requirements based on DRX where TSSB refers to the OD-SSB periodicity.

Proposal 12: RAN4 should focus next on OD-SSB based SCell activation delay requirements for multiple SCell activation since multiple SCell activation is a mandatory feature.

Proposal 13: OD-SSB based SCell activation delay requirements should not be defined for Rel-17 Fast SCell Activation.

Observation 14: Additional interruptions on PCell or activated SCell during measurements on deactivated SCell are not needed for OD-SSB.

Proposal 14: The wording in section 8.2 of TS 38.133 should be updated since measCycleSCell may not be meaningful for all deployment scenarios of OD-SSB anymore.

Proposal 15: Neighbor cell measurements for FR2 should follow legacy requirements as well and no change in Rx beam sweeping is required.

Proposal 16: No OD-SSB based L3 measurement reporting should be introduced.

# References

- R4-2112508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #114, February 2025

- R4-2420192, Reply LS on timeline for On-demand SSB operation on SCell, RAN4 #113, November 2024

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN1 #119, November 2024

- R1-2501633, LS on time location of on-demand SSB for SCell, RAN1 #120, February 2025

- R2-2501484, Reply LS on SSB adaptation, RAN2 #129, February 2025