3GPP TSG-RAN WG4 Meeting #114-bis									  				  R4-2503129

Wuhan, CN 7th - 11th April 2025

Agenda item:			7.11.6

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for Ku bands for NR NTN

Document for:	Discussion

# Introduction

This paper presents our views on the remaining issues of the RRM requirements for Ku bands for NR NTN as outlined in WF Error! Reference source not found.. The agreements are as follows:

Issue 1-1: RRM scope for NR NTN on Ku bands

Agreement:

- For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session.

- Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE:

  - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology

  - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology

  - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology

  - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

Issue 1-3: UL transmit timing requirement

Agreement:

  - RAN4 to define timing accuracy requirements for case 1, 2:

    - For FR1-NTN numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used.

      - For the applicable SCS, follow the agreement in BDaT session.

  - RAN4 to define timing accuracy requirements for case 3:

    - For FR2-NTN numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

      - For the applicable SCS, follow the agreement in BDaT session.

      - The endorsed CR with 7.5 Ts under the same side conditions as Rel-18 are considered.

  - RAN4 to define timing accuracy requirements for case 4:

    - For FR2-NTN numerology, FFS existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

      - For the applicable SCS, follow the agreement in BDaT session.

      - The endorsed CR with 7.5 Ts is considered. FFS whether the same or different side condition from Rel-18 can be used.

# RRM requirement for Ku band

For Ku band RRM requirement, it can leverage the existing Ka band RRM requirements as the starting point, as UE RX beam needs to be considered in Ku band as well. The current Ka band RRM requirements are summarized in Table 1.

Table 1 Ka band RRM requirements

| RRM requirements | Type 1: Electronic |  | Type 2: Mechanical |  |
| --- | --- | --- | --- | --- |
|  | intra-sat | inter-sat | intra-sat | inter-sat |
| UE uplink timing accuracy | Y* |  |  |  |
| RRC IDLE and INACTIVE mobility | Y | N | Y | N |
| (Conditional/blind) Handover | Y | Y* | Y | Y* |
| RRC Re-establishment | Y | N | Y | N |
| RRC Connection Release with Redirection | N |  |  |  |
| Radio Link Monitoring | Y |  |  |  |
| Link Recovery procedure (BFD/CBD) | N |  |  |  |
| Active TCI switching | N |  |  |  |
| L1-RSRP | Y |  |  |  |
| L3 Measurement Procedure (delay) | Y | N | Y | N |
| L3 Measurement Performance (accuracy) | Y | N | Y | N |

| N | unspecified |
| --- | --- |
| Y | Based on FR1 requirement |
| Y* | Impacted Requirement |

According to the current Ka band RRM requirements, it can also be observed that, for intra-Satellite RRM requirement, most Ka band RRM requirements are based on FR1, by assuming same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite. (i.e. no Rx beam sweeping). The same approach can be used for Ku-band.

Proposal 1: For intra-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point.

However, for inter-Satellite Handover requirement, the interruption time differs upon UE beam steering type (i.e. ‘Electronic’ or ‘Mechanical’), as in 6.1C.1.3.2, excerpted as below, RAN4 will need to discuss whether to reuse the same value of T_sat_beam as in Ka band for Ku band.

Proposal 2: For inter-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point. Besides, RAN4 to discuss whether to reuse the same value of T_sat_beam for Ku band Handover requirement.

Specifically, the following are our comments on the RRM core requirements, including suggestions for reused unmodified requirements, editorial revisions, and new technical work.

Table 2 RRM Core Requirements list for Ku band

| Section Number | Section/ sub-section title | Comments |
| --- | --- | --- |
| RRM Core Requirements |  |  |
| 4.2.2.12 | Measurements of inter-frequency NR cells with NTN carrier | Not applicable for Ku band (it is for FR1 NTN) |
| 4.2C | Cell Re-selection for NR UE for Satellite Access |  |
| 4.2C.2 | Requirements |  |
| 4.2C.2.1 | UE measurement capability | reused unmodified |
| 4.2C.2.2 | Measurement and evaluation of serving cell | reused unmodified (for intra-satellite) |
| 4.2C.2.3 | Measurements of intra-frequency NR cells | editorial revisions to include Ku band (for intra-satellite) |
| 4.2C.2.4 | Measurements of inter-frequency NR cells | editorial revisions to include Ku band (for intra-satellite) |
| 6.1C | Handover for SAN |  |
| 6.1C.1.2 | NR SAN FR1 – NR SAN FR1 Handover | Not applicable for Ku band (it is for FR1 NTN) |
| 6.1C.1.3 | NR SAN FR2-NTN – NR SAN FR2-NTN Handover | [technical] needed to confirm whether to reuse Tsat_beam |
| 6.1C.2.4 | NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover | editorial revisions to include Ku band (for intra-satellite) |
| 6.1C.3 | NR SAN Satellite switching with re-synchronization |  |
| 6.1C.3.2 | NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization | Not applicable for Ku band (it is for FR1 NTN) |
| 6.2C.1 | SA: RRC Re-establishment for Satellite Access |  |
| 6.2C.1.2.2 | UE Re-establishment delay requirement for VSAT | reused unmodified |
| 6.2C.2 | Random access for satellite access | reused unmodified |
| 6.2C.3 | SA: RRC Connection Release with Redirection for Satellite Access | No requirements for Ka/Ku |
| 7.1C | UE transmit timing for Satellite Access |  |
| 7.1C.1 | Introduction |  |
| 7.1C.2 | Requirements |  |
|  | Table 7.1C.2-1: Te_NTN Timing Error Limit | [technical] pending on applicable SCS |
|  | Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO | [technical] pending on applicable SCS |
|  | Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO | [technical] pending on applicable SCS |
|  | Table 7.1C.2-4: The Value of NTAoffset for VSAT in FR2-NTN | [technical] To decide the default is 25600 as in FR1 or 0 as in FR2-NTN |
| 7.1C.2.1 | Gradual timing adjustment |  |
|  | Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate | editorial revisions to include Ku band (for intra-satellite) |
| 7.2C | UE timer accuracy for satellite access | reused unmodified |
| 7.3C | Timing advance for satellite access |  |
| 7.3C.2.2 | Table 7.3C.2.2-1: UE Timing Advance adjustment accuracy | [technical] pending on applicable SCS |
|  | Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN | [technical] pending on applicable SCS |
| 8.1C | Radio Link Monitoring for Satellite Access |  |
|  | Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.2 | Requirements for SSB based radio link monitoring | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.2.3 | Measurement restrictions for SSB based RLM | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.3 | Requirements for CSI-RS based radio link monitoring | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.3.2 | Minimum requirement | editorial revisions to include Ku band (for intra-satellite) |
|  | Table 8.1C.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for [FR2-NTN] | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.3.3 | Measurement restrictions for CSI-RS based RLM | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.7 | Scheduling availability of UE during radio link monitoring |  |
| 8.1C.7.1 | Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1 and [FR2-NTN] | editorial revisions to include Ku band (for intra-satellite) |
| 8.1C.7.2 | Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1 and [FR2-NTN] | editorial revisions to include Ku band (for intra-satellite) |
| 9.1C.2 | Measurement gap |  |
|  | Table 9.1C.2-2 | [technical] pending on applicable SCS |
|  | Table 9.1C.2-2a | [technical] pending on applicable SCS |
| 9.2C.2 | Requirements applicability | editorial revisions to include Ku band (for intra-satellite) |
| 9.2C.3 | Number of cells and number of SSB | reused unmodified |
| 9.2C.4 | Measurement Reporting Requirements |  |
| 9.2C.4.1 | Periodic Reporting | editorial revisions to include Ku band (for intra-satellite) |
| 9.2C.4.2 | Event-triggered Periodic Reporting | editorial revisions to include Ku band (for intra-satellite) |
| 9.2C.4.3 | Event Triggered Reporting | editorial revisions to include Ku band (for intra-satellite) |
| 9.3C | NR inter-frequency measurements for SAN |  |
| 9.3C.2 | Requirements applicability | editorial revisions to include Ku band (for intra-satellite) |
| 9.3C.3 | Number of cells and number of SSB | [technical] discuss whether to introduce this requirement for Ku/Ka |
| 9.3C.3.1 | Requirements for FR1 | Not applicable for Ku band (it is for FR1 NTN) |
| 9.5C.7.1 | L1-RSRP measurement requirements for NTN band above 10 GHz | reused unmodified |

Proposal 3: Discuss RRM requirement list for Ku-band (as in provided Table 2).

# Summary

The observations and proposals are summarized as the following:

Proposal 1: For intra-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point.

Proposal 2: For inter-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point. Besides, RAN4 to discuss whether to reuse the same value of T_sat_beam for Ku band Handover requirement.

Proposal 3: Discuss RRM requirement list for Ku-band (as in provided Table 2).

# Reference

- R4-2503129, WF on RRM requirements for NR_NTN_Ku_bands, Feb. 2025.