3GPP TSG-RAN WG4 Meeting #114-bis									    R4-2503130

Wuhan, CN 7th - 11th April 2025

Agenda Item:	7.25.2.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for OD-SSB for network energy saving

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

The remaining issues on On-demand SSB(OD-SSB) requirements [1] are discussed in this paper.

# 1 OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Agreement:

RAN4 to define measurement requirement based on T2.

Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

When NW re-activates the OD-SSB within the T2

- Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

- Option 2: UE is required to perform “fast measurement” without cell search.

  - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

- Option 3: UE is required to perform FMW after T2.

Option 2 may not be necessary, as it appears to be already covered by the current clause 9.2.4.3 in TS38.133.

A cell is detectable only if at least one SSBs measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

Proposal 1: When NW re-activates the OD-SSB within the T2, either Option 1 or Option 3 should be considered, if the OD-SSB is available.

The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

The T2 between multiple OD-SSB transmissions was discussed to limit the UE measurement complexity and prevent the UE from remining in FWM. Intuitively, the two fast modes should not be too close to each other.

Considering that the network has no intention to trigger frequent OD-SSB transmissions from network power saving perspective, a simple and clear definition of T2 is preferred. Additionally, it is crucial to avoid dynamic changes in UE measurement behaviour, as it will require additional UE complexity.

For Option 2, it is unclear whether the network knows if this L3 measurement reporting is for FMW or not. And it seems to offer limited additional benefit compared to Option 1.

For Option 3, T2 cannot be determined until the NW re-indicating OD-SSB transmission, and the UE cannot determine its measurement behabior until the re-indicating OD-SSB transmission. The dynamic switch of UE measurement behabvior is less preferred.

For Option 4, it defined T2 based on OD-SSB deactivation, which is dynamic deactivated by MAC-CE, and the dynamic switch of UE measurement behabvior is less preferred.

Therefore, Option 1 appears to be the most straightforward. Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify

Proposal 2: The start point of T2 is based on Tend (Option 1), i.e. the time point of OD-SSB indication plus Tuncertain +Tidentify.

# OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Sub-issue 3-1: Lower bound for measurement in FMW

- Background - The agreement in last meeting is as follow. Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Wayforward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW.

where, measCycleSCell in 38.331 is shown below.

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

A lower bound that is too short or absent will introduce additional UE complexity and limit support for this feature. Therefore, it is crucial to carefully select a reasonable lower bound for measurement delay. In this section, the HST feature is referenced to determine the appropriate lower bound.

According to the legacy deactivated SCell measurement requirement (as shown in Appendix), it can be observed that, with the shortest measCycleSCell of 160 ms, CSSF =1, Kp =1, the minimum times (lower bounds) are given as follows:

- Time period for PSS/SSS detection in FR1 = 5 x 160 ms = 800 ms

- Time period for PSS/SSS detection in FR2 = 24 (PC3) x 160 ms = 3840 ms

- Time period for time index detection in FR1 = 3 x 160 ms = 480 ms

- Measurement period for intra-frequency measurements without gaps in FR1 = 5 x 160 ms = 800 ms

- Measurement period for intra-frequency measurements without gaps in FR2 = 24 (PC3) x 160 ms = 3840 ms

On the other hand, according to the high speed trains measurement requirement (as shown in Appendix), the lower bounds are as follows:

- Time period for PSS/SSS detection in FR1 = 600 ms

- Time period for PSS/SSS detection in FR2 = 600 ms

- Time period for time index detection in FR1 = 120 ms

- Measurement period for intra-frequency measurements without gaps in FR1 = 200 ms

- Measurement period for intra-frequency measurements without gaps in FR2 = 400 ms

Considering that the measurement and cell detection are limited to the serving SCell, without neighbor cell measurements, the lower bound can be defined based on an OD-SSB periodicity of 20 ms. This results in the following:

- Time period for PSS/SSS detection in FR1 = 100 ms

- Time period for PSS/SSS detection in FR2 = 480 ms

- Time period for time index detection in FR1 = 60 ms

- Measurement period for intra-frequency measurements without gaps in FR1 = 100 ms

- Measurement period for intra-frequency measurements without gaps in FR2 = 200 ms

Note this provides significant enhancement not only comparing with legacy SCell measurement but also comparing with cell measurement in HST, specifically,

- In FR1 PSS/SSS detection, 8 times faster than legacy deactivated SCell measurement and 6 times faster in HST

- In FR1 intra-frequency measurements, 8 times faster than legacy deactivated SCell measurement and 2 times faster in HST

Proposal 3: The lower bounds are defied as follows (based on OD-SSB periodicity of 20ms)

- Time period for PSS/SSS detection in FR1 = 100 ms

- Time period for PSS/SSS detection in FR2 = 480 ms

- Time period for time index detection in FR1 = 60 ms

- Measurement period for intra-frequency measurements without gaps in FR1 = 100 ms

- Measurement period for intra-frequency measurements without gaps in FR2 = 200 ms

# Conclusion

Proposal 1: When NW re-activates the OD-SSB within the T2, either Option 1 or Option 3 should be considered, if the OD-SSB transmission is available.

Proposal 2: The start point of T2 is based on Tend (Option 1), i.e. the time point of OD-SSB indication plus Tuncertain +Tidentify.

Proposal 3: The lower bounds are defied as follows (based on OD-SSB periodicity of 20ms)

- Time period for PSS/SSS detection in FR1 = 100 ms

- Time period for PSS/SSS detection in FR2 = 480 ms

- Time period for time index detection in FR1 = 60 ms

- Measurement period for intra-frequency measurements without gaps in FR1 = 100 ms

- Measurement period for intra-frequency measurements without gaps in FR2 = 200 ms

# References

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, Feb. 2025.

# Appendix - HST

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps x KFR x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.6.3-3: Measurement period When highSpeedMeasFlag-r16 is configured (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kgap ) x max(MGRP, SMTC period)) Note 1 x CSSFintra |
| DRX cycle≤ 160 ms | max(200 ms, ceil(M2Note 2 x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| 160 ms < DRX cycle≤ 320 ms | max(200 ms, ceil(M2Note 2 x 4 x Kgap) x max(MGRP, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(Y Note 3 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40 ms, Y=5 when SMTC > 40 ms NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 5: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 6: When highSpeedMeasCA-Scell-r17 is configured, the requirements also apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 9.2.6.3-4: Measurement period for intra-frequency measurements with gaps when highSpeedMeasFlagFR2-r17 is configured (FR2-1) when SMTC period≤40ms

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400 ms, ceil(M1Note 2 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 80 ms | max(400 ms, ceil(M1Note 2 x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | max(400 ms, ceil(Mmeas_period with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320 ms | Ceil( Mmeas_period with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

# Appendix - the legacy deactivated SCell measurement

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCel |  |

Table 9.2.5.2-4: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR2)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |