3GPP TSG-RAN WG4 Meeting #114-bis									    R4-2503131

Wuhan, CN 7th - 11th April 2025

Agenda Item:	7.25.2.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

Issue 2-1-1: RRM requirements impacts for OD-SIB1

Agreement:

RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

In [1], the RRM impact of OD-SIB1 will be discussed in this meeting.

Since the UL WUS from a UE to NES cell would be transmitted for OD-SIB1, the NES cell may transmit SIB1 in response to the UL WUS (RAN1#116 agreement). This may lead to impact on paging interruption requirement, which is discussed in this paper.

# 2 Potential impact on paging interruption due to UL-WUS for OD-SIB1

During cell re-selection in IDLE mode, UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. And the interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms, as specified in 4.2.2.6 in 38.133 (as attached below).

#### 4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

However, UL WUS is required for OD-SIB1 indication, as illustrated as below, and the UL WUS transmission would cause additional paging interruption, on top of the current requirement.

Figure 1: OD-SIB1 timeline

One possible approach is to extend the interruption time to include the UL WUS transmission.

Proposal 1: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission.

# Conclusion

Proposal 1: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission.

# References

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Feb. 2025.