3GPP TSG-RAN WG4 Meeting #114-bis				                      R4-2503132

Wuhan, CN 7th - 11th April 2025

Agenda Item:	7.30.3

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for IoT NTN phase 3

WID/SID:        IoT_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

This paper presents our view on the random access requirements for CB-Msg3 and the corresponding CR work split.

# 2  Requirement for CB-Msg3

The procedural requirements for CB-MSG3 transmission was discussed in WF [1].

Issue 1-1-1: Procedural requirements for CB-MSG3 transmission Agreement: - Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133. - Further discuss the content to include in the next meeting.

Specifically, it should include at least the following subclauses:

- Requirements for CB-Msg3 transmission

- Correct behavior when receiving CB-Msg4

- Correct behavior when not receiving CB-Msg4

Requirements for CB-Msg3 transmission

According to the R2#128 agreement, the CB-Msg3 PUSCH information is provided is system information (SIB) and the TA validation parameters is FFS. Thus, based on PUR requirement as in 4.6A.3, TS 36.133, the example TP for the requirements for CB-Msg3 transmission is provided in Appendix as below. Note that any need for dedicated signalling for the TA validation parameters is pending to further R2 progress.

Agreements (RAN2#128, Nov’24):

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

Correct behavior when receiving CB-Msg4 for CB-Msg3

The detailed behavior when receiving CB-Msg4 is pending further progress in R2, such as the contention resolution window/timer, sending ACK.

Correct behavior when not receiving CB-Msg4 for CB-Msg3

The detailed behavior when not receiving CB-Msg4 is pending further progress in R2 as well, such as TX power ramping, backoff timer, and fallback procedures.

Proposal 1: Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN, and the detailed behavior is pending further progress in R2.

•	Requirements for CB-Msg3 transmission

•	Correct behavior when receiving CB-Msg4

•	Correct behavior when not receiving CB-Msg4

# 3  CR work split for CB-Msg3

CR work split based on the following Table 1.

Table 1. CR work split for CB-Msg3

| Core requirement | Volunteer | Comment |
| --- | --- | --- |
| 6.6A Random Access for UE category NB-IoT for Satellite Access |  | New subclause for CB-Msg3 |
| 6.2.4A Random Access Requirements for Cat-M1 UEs with CB-Msg3 for Satellite Access |  | New subclause for CB-Msg3 It appears that a separate new subclause is necessary. |
| 7.20A UE transmit timing for NB-IoT for Satellite Access |  | Revised to apply to CB-Msg3 |
| 7.24A UE transmit timing for Category M1 for Satellite Access |  |  |

Proposal 2: CR work split based on Table 1.

# Summary

The observations and proposals are summarized as the following:

Proposal 1: Include the following subclauses in Random Access requirement for CB-Msg3 in IoT NTN, and the detailed behavior is pending further progress in R2.

•	Requirements for CB-Msg3 transmission

•	Correct behavior when receiving CB-Msg4

•	Correct behavior when not receiving CB-Msg4

Proposal 2: CR work split based on Table 1.

# References

- R4-2502606, WF on RRM requirements for IoT_NTN_Ph3, Feb. 2025.

# Appendix: TP for requirements for CB-Msg3

### 6.6A.4	Requirements for CB-Msg3

#### 6.6A.4.1	Introduction

The requirements in this clause are applicable when the UE is configured with CB-Msg3 as specified in [TS 36.331].

#### 6.6A.4.2	Requirements for CB-Msg3 transmission

The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage.

The UE is allowed to transmit using CB-Msg3 resources provided in [SIBxx] that the UE is synchronized towards the serving cell prior to transmission. The UE determines the CB-Msg3 transmission occasion according to the received CB-Msg3 configuration [TS 36.331].

The UE shall determine the uplink timing,  TTA, for transmitting on CB-Msg3 using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before CB-Msg3 transmission.

Editor’s note: any need for dedicated signalling for the TA validation parameters is pending to further R2 progress.

#### 6.6A.4.3 Correct behavior when receiving CB-Msg4

Editor’s note: the detailed behavior when receiving CB-Msg4 is pending further progress in R2.

#### 6.6A.4.4 Correct behavior when not receiving CB-Msg4

Editor’s note: the detailed behavior when receiving CB-Msg4 is pending further progress in R2.