3GPP TSG-RAN WG4 Meeting #114-bis				                      R4-2503134

Wuhan, CN 7th - 11th April 2025

Agenda item:			7.31.5

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for IoT-NTN TDD mode

WID/SID:        IoT_NTN_TDD-Core

Document for:	Discussion

# 1  Introduction

Issue 0: General aspects

Agreement (online):

For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings:

•	The target SNR

•	Applicable coverage (whether the EC needs to be considered)

•	Applicable Rmax

•	Applicable DRX cycle

o	Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

In RAN4#114, whether the EC (Enhanced Coverage) needs to be considered was discussed [1]. Our view is provided in this paper.

# 2  Enhanced coverage for NTN TDD

R4-2419795 Way Forward for [113][317] IoT_NTN_TDD, Nov. 2024. ### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements Agreement: - RAN4 to consider the following values for analyzing the impact of requirements, as necessary: - N = 9, D = {8, 20, 30}, U = {8, 20, 30} - N = 8, D = 20, U = 20 - Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis. Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision. Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

In RAN4#113, it was agreed the RAN4 requirement analysis focuses on normal coverage [2]. According to 36.133 the SNR (Es/IoT) side condition for normal coverage is  -6 dB and the it is ranged from -15 dB to -6dB for Enhanced coverage.

-  The requirements for normal coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: NSCH Ês/Iot  -6 dB and NRS Ês/Iot  -6 dB.

-	The requirements for enhanced coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: -15 dB  NSCH Ês/Iot < -6 dB and -15 dB  NRS Ês/Iot < -6 dB.

To consider enhanced coverage, it will result in more repetition and longer delay to support the low SNR and it seems not suitable for NTN TDD with NGSO, as the available RS is less in the TDD mode.

Observation 1: In WF R4-2419795, in terms of SNR, the requirement analysis focuses on the requirements for normal coverage.

Observation 2: Delay for EC will be longer than NC as the SNR condition is very low (-15 dB  NRS Ês/Iot < -6 dB).

### 1.1.1 RAN1#118b Agreement The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain). - For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain). #### 2.1.2 Agreements during RAN1#119 Agreements and observations stemming from evaluations of feasibility of DL sync and other aspects Observation RAN1 makes the following observations on downlink synchronization: Case 1: For N=9, D=8: - For NPSS: - For one-shot detection (90ms delay), 9 sources provided simulation results, with a median required SNR of 1.30dB. The link budget margin is (median margin): - 4.21 dB for LEO-1200 (0dBi antenna gain) - -1.89 dB for LEO-600 (-5.5dBi antenna gain). - For two-shot detection (180ms delay), 3 sources provided simulation results, with a median required SNR of -1.20dB. The link budget margin is (median margin): - 6.71dB for LEO 1200 (0dBi antenna gain) - 0.61dB for LEO-600 (-5.5dBi antenna gain). - For four-shot detection (360ms delay), 3 sources provided simulation results, with a median required SNR of -3.80dB. The link budget margin is (median margin): - 9.31dB for LEO-1200 (0dBi antenna gain) - 3.21dB for LEO-600 (-5.5dBi antenna gain). - NOTE: The assumptions for combining for 2-shot and 3-shot detection are different across companies. - For NSSS: - For one-shot detection (180ms delay), 5 sources provided simulation results, with a median required SNR of -2.50dB. The link budget margin is (median margin): - 8.01dB for LEO-1200 (0dBi antenna gain) - 1.91dB for LEO-600 (-5.5dBi antenna gain) - For NPBCH: - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin): - 2.03dB for LEO-1200 (0dBi antenna gain) - -4.07dB for LEO-600 (-5.5dBi antenna gain) - For 640ms combining, 6 sources provided results, with a median required SNR of -4.25dB. The link budget margin is (median margin): - 9.76dB for LEO-1200 (0dBi antenna gain) - 3.66dB for LEO-600 (-5.5dBi antenna gain)

Besides, it can be observed from the agreement in RAN1#118b, the target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz:

- For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain).

- For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain).

And all required SNR to support downlink synchronization for (N=9, D=8) fall within the SNR range of normal coverage.

Based on the above discussion, for TDD NB-IoT in NTN, we propose to define RRM requirement based on normal coverage, rather than enhanced coverage.

Observation 3: The operating DL CNR is 4.91dB/5.51dB (i.e. with 0dBi UE antenna gain) with the carrier frequency of 1.6GHz, for LEO-600/1200 respectively, and the required SNR for downlink synchronization physical channels with case (N=9, D=8) is > -6dB.

Proposal 1: For TDD NB-IoT in NTN, define RRM requirement based on normal coverage (Ês/Iot  -6 dB).

# References

- R4-2502607, WF on RRM requirements for IoT_NTN_TDD, Moderator (Qualcomm Incorporated), R4#114, Feb. 2025.

- R4-2419795, Way Forward for [113][317] IoT_NTN_TDD, Moderator (Qualcomm Incorporated), R4#113, Nov. 2024.