3GPP TSG- WG4 Meeting # 114bis	R4-2503145

# Wuhan, China, April 07 – 11, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on simultaneous Rx-Tx

Agenda Item:	6.12.2

Document for:	Approval

Introduction

In RAN4#114, the WF R4-2502840 was approved that

- To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD. - For FDD-FDD band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this due to UL CA_n5-n8 is TBD - For FDD-SDL band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this due to LB-LB RX switching including LB-LB configurations in RP-242532 are TBD.

This paper provides further analysis based on the WF about how to handle the exceptional cases for FDD-FDD and FDD-SDL.

Discussion

For TDD-FDD inter-band CA, RAN4 has agreed and endorsed to capture in TS 38.101-1 that

Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD CA combinations.

For TDD-SDL, it was agreed to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD. In this case, the UE capability defined in TS38.306 should be updated. And LS should be sent to RAN2.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD, TDD-SDL and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for TDD-FDD, TDD-SDL and certain TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD and TDD-FDD inter-band NR-CA or NR-DC combinations: - Intra-band NR-CA or NR-DC component - Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]). | BC | CY | N/A | N/A |

Proposal 1: Send LS to RAN2 to update UE capability simultaneousRxTxInterBandCA according to the conclusions in RAN4 for TDD-SDL and FDD-TDD band combination.

For TDD-TDD and TDD-FDD inter-band CA, taking CA_nA-nB for example, the capability simultaneousRxTx does not differentiate the two cases. Just both supported or neither.

- simultaneous (nAUL, nBDL)

- simultaneous (nBUL, nADL)

For FDD-FDD and FDD-SDL inter-band CA, it was concluded that simultaneous Rx-Tx is mandatory without signalling. The exceptions should be considered. The frequency range of UL in band n8 and DL in band n5 is overlapped, and it is impossible to implement simultaneous (n5DL, n8UL). Meanwhile, simultaneous (n5UL, n8DL) can be supported by default. In RAN4#113, RAN4 sent LS[3] to RAN2 to ask necessary modification in RAN2 spec and/or add UE capability to enable the functionality of different simultaneous Rx-Tx capability for (n5UL, n8DL) and (n8UL, n5DL) in CA_n5-n8 with two FDD bands frequency range partially overlap

Figure 1 Two possible operations illustration for CA_n5-n8

However, CA_n5-n8 would not be the only FDD-FDD BC that hardly supports simultaneous Rx-Tx. For band combination without sufficient separation between DL carrier in one band and UL carrier in the other band, simultaneous DL-UL operation is difficult. In [1], two operators in North America identify two sub-1GHz to be aggregated, CA_n26-n106 with 2MHz isolation between n26 DL and n106 UL. How to design the duplexer for the BC is questionable.

Figure 2 Frequency range illustration for CA_n26-n106

Observation 1: The FDD-FDD band combination CA_nA-nB with overlapping frequency or without sufficient frequency separation between band nA UL and band nB DL can hardly support simultaneous operation of (nA UL, nB DL).

A method should be investigated of how to notify the network about the scheduling restriction between nA UL and nB DL for such FDD-FDD combo. If one day, the duplexer or advanced UE RF front end architecture is implemented and enable the simultaneous (nA UL, nB DL). The forward-compatibility should be considered.

For Rel-19 low band carrier aggregation via switching [2], there is no simultaneous Rx-Tx between FDD UL and SDL. From the duplexer design perspective, the frequency separation between the bands does not pose a feasibility challenge in general. Thing are quite similar to that for FDD-FDD without sufficient isolation between nA UL and nB DL in CA_n12-n29.

Figure 3 Frequency range illustration for CA_n12-n29

Observation 2: FDD-SDL band combination in Rel-19 low band carrier aggregation via switching without sufficient frequency separation results in duplexer design difficulty, and makes it hard to implement simultaneous Rx-Tx for FDD UL and SDL.

In [1], CA_n28-n67 was also proposed for CA via switching, n67 DL (738-758MHz) overlaps with n28 UL (703-748MHz). The reason for non-simultaneous Rx-Tx is same as that for CA_n5-n8 (n5DL, n8UL).

Proposal 2: A method should be investigated of how to notify the network about the scheduling restriction between nA UL and nB DL for such FDD-FDD and FDD-SDL combos with overlapping frequency or without sufficient frequency separation between band nA UL and band nB DL, which is against with the mandatory capability of simultaneous Rx-Tx.

Proposal 3: If some day, the duplexer or advanced UE RF front end architecture is implemented to enable the simultaneous (nA UL, nB DL), the forward-compatibility should be considered.

Conclusion

In this contribution we further the rules for simultaneous RxTx for FR1, according to the analysis, we have the following observations and proposals:

Observation 1: The FDD-FDD band combination CA_nA-nB with overlapping frequency or without sufficient frequency separation between band nA UL and band nB DL can hardly support simultaneous operation of (nA UL, nB DL).

Observation 2: FDD-SDL band combination in Rel-19 low band carrier aggregation via switching without sufficient frequency separation results in duplexer design difficulty, and makes it hard to implement simultaneous Rx-Tx for FDD UL and SDL.

Proposal 1: Send LS to RAN2 to update UE capability simultaneousRxTxInterBandCA according to the conclusions in RAN4 for TDD-SDL and FDD-TDD band combination.

Proposal 2: A method should be investigated of how to notify the network about the scheduling restriction between nA UL and nB DL for such FDD-FDD and FDD-SDL combos with overlapping frequency or without sufficient frequency separation between band nA UL and band nB DL, which is against with the mandatory capability of simultaneous Rx-Tx.

Proposal 3: If some day, the duplexer or advanced UE RF front end architecture is implemented to enable the simultaneous (nA UL, nB DL), the forward-compatibility should be considered.

- References

[1] R4-2411176, Motivation for a new work item on LB+LB CA based on switching in Rel-19, Apple, , Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc, RAN4#112.

[2] RP-243317, New WID on low band carrier aggregation via switching, RAN#106

[3] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#113

RAN WG4 Meeting #114bis	R4-250xxxx

Wuhan, China, 07 – 11 April, 2025

Title:	Draft LS on capability of simultaneousRxTxInterBandCA

Response to:

Release:	Rel-19

Work Item:	TEI19

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	HU Dan

E-mail Address: hudan11@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Nil

1. Overall Description:

In Athens meeting, RAN4 had agreed that

- To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD.

And the clarification for FDD-TDD CA band combination has been captured in the general part of sub-clause 5.2A.2 Inter-band CA that

Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD CA combinations.

Meanwhile, the Note 1 attached to FDD-TDD band combinations in Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) is removed.

NOTE 1:	Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability.

To accommodate RAN4’s progress in Rel-19, the definition for UE capability simultaneousRxTxInterBandCA in TS38.306 can be considered to update.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully requests RAN2 to update the UE capability simultaneousRxTxInterBandCA based on RAN4 progress in Rel-19

.

3. Date of Next TSG WG RAN4 Meeting:

TSG-RAN4 Meeting#115  	 				19 – 23 May 2025		                   Malta, MT

TSG-RAN4 Meeting#116  	 				25 – 29 August 2025		                     India