3GPP TSG- WG4 Meeting # 114bis	R4-2503152

# Wuhan, China, April 07 – 11, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on LB-LB switching

Agenda Item:	7.8.2

Document for:	Approval

Introduction

In RAN#106, the WID[1] for Low band carrier aggregation via switching was approved

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In RAN4#114 meeting, the WF[2] was approved with the discussion outcome of switching period, location of switching period and the capability related issues.

Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded Issue 5: Switching period location - Discuss the switching period location based on the following options: - Option 1: Switching period location is entirely up to RAN1 design - Option 2: Switching period location is defined relative to the slot boundary - Other options are not precluded Issue 3: Switching period capability - Discuss whether the switching delay capability can be per band combination

This paper is to provide further thinking concerning the LB-LB switching in Rel-19.

UE requirements for LB-LB switching

In Rel-16~ Rel-18, up to 4 bands UL Tx switching for CA was specified. The standardization efforts in RAN4 includes:

- Length and location of switching period

- Time mask

- Applicability of DL interruption

- etc..

Considering Rel-19 low band CA via switching is also related to UE switching behaviour, the requirements listed above need to be specified, as well. Besides, some of the standardization method can be referred to.

Time mask

It is RAN1’s task to design the semi-static switching pattern based on RRC configuration for low band CA via switching. RAN4 is to discuss how to determine time mask for carrier switching.

There are at least two options:

- Up to RRC configuration

- On the Scell (SDL)

Regarding “up to RRC configuration”, in Tx switching, the switching period location is up to RRC configuration uplinkTxSwitchingPeriodLocation if the switching period isn’t fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. To simplify, the way to determine location of switching period can be inherited for low-band CA via switching.

Proposal 1: At least the location of the switching gap in time mask can be indicated via RRC configuration.

With respect to “on the SCell (SDL)”, in the Athens meeting, it was proposed to consider switching period at the tail of switch-from carrier to avoid the impact to PDCCH. PDCCH is an important physical layer control channel, but it is not the only important RAN1 channel/signal. Other reference signals, including DMRS, SRS, CSI-RS, and control information, such as, HARQ-ACK, SR and CSI carried on PUCCH, UCI multiplexed on PUSCH also have impact on UL/DL performance. If physical layer channel/signal need to be considered to determine the location of switching gap, it is up to RAN1’s evaluation on how to minimize the impact.

Observation 1: Important physical layer channels/signals include not only PDCCH, but also reference signals, including DMRS, SRS, CSI-RS, and control information, such as, HARQ-ACK, SR and CSI carried on PUCCH, UCI multiplexed on PUSCH.

Proposal 2: It is up to RAN1’s responsibility on if physical layer channel/signal need to be considered to determine the location of switching gap and how to minimize the impact if needed.

Switching gap

In UL Tx switching, the switching periods were discussed based on the referenced possible implementation and UE architectures. For Rel-19 low band CA via switching, the example band combination is FDD-SDL, and the switching between case 1(Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2) and case 2 (Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1) lies in Rx switching. For initial analysis, there are two types of possible Rx chain architectures for LB-LB Rx switching

- Option 1                    (b)  Option2

Figure 1 Reference UE architecture for low band & low band Rx switching

Table 1 The architecture analysis for the reference ones

| Shared btw SDL band and FDD band | Option 1 | Option 2 |
| --- | --- | --- |
| PLL | Not Shared | Shared |
| Antenna | Shared | Shared |
| Filter | Not Shared | Not Shared |
| LNA | Not Shared | Shared |
| Rx channel | Not Shared | Shared |

For Option 1, only the antenna is shared by the FDD band and the SDL band, and for the rest parts, including phase lock loop, local oscillator, low noise amplifier and filters are used respectively to the two bands. While the DL operation is ongoing on the FDD bands, before the Rx operation switched to the SDL band, the other Rx channel of the SDL band can be ready in advance for DL transmission.

For option 2, the LO and PLL is shared by the two bands. Rx switching requires LO retuning, and Rx channel parameter loading. 140us is sufficient for this possible implementation.

Conclusion

In this contribution we discussed on low band CA via switching, according to the analysis, we have the following observation and proposals:

Observation 1: Important physical layer channels/signals include not only PDCCH, but also reference signals, including DMRS, SRS, CSI-RS, and control information, such as, HARQ-ACK, SR and CSI carried on PUCCH, UCI multiplexed on PUSCH.

Proposal 1: At least the location of the switching gap in time mask can be indicated via RRC configuration.

Proposal 2: It is up to RAN1’s responsibility on if physical layer channel/signal need to be considered to determine the location of switching gap and how to minimize the impact if needed.

- References

[1] RP-243317, New WID on low band carrier aggregation via switching, Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE, RAN#106, December, 2024

[2] R4-2503016, WF on low-low band switching, Apple, RAN4#114, February, 2025.