3GPP TSG-RAN WG4 Meeting #114bis	R4-2503159

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on NTN NGSO Channel Model

Agenda item:	7.10.4.1

Document for:	Discussion

Introduction

The ongoing work item on NTN NGSO Testing is looking to introduce a channel model for the development of NGSO performance requirements and core requirements.

The WID [2] was revised during RAN #106, and targets a completion of the performance part by RAN#111.

In the current version of the WID the performance objectives are as follows

- Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

During RAN4#114-bis the agenda has two separate agenda items, however the discussion in this document focusses the discussion on what Nokia believes is the main aspect of the channel model itself, which Nokia believes should be sufficient to start to develop performance requirements with.

Discussion

Use of extant channel model for performance requirements

Nokia believes the main aspect to be discuss regarding the channel model at this stage of the work item is the impact on RAN4 demodulation requirements, as such this is captured in Issue 1-2-3 of the WF from RAN4#114

Issue 1-2-3 Impact on RAN4 demodulation requirements - Proposals - Option 1: - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 2: - Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin - Option 3 - Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

To this end, Nokia strongly supports that RAN4 moves to attempt to define parameters for simulation such that the channel model can be assessed to understand the margin that is produced from different implementations and whether this is applicable for requirements definition.

RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with.

Conclusion

The paper discusses the topic of NTN NGSO Channel model for discussion at RAN4#114-bis

The following Observations and Proposals were made:

Proposal 1: RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with.

- References

- R4-2502314, “Way Forward for [114][325] NTN_testing_NGSO_channel_model”, Samsung, RAN4#114, Athens, February 2025

- RP-242901,  “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Apple, Thales, Viasat, Globalstar, RAN#106, Madrid, December 2024