3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2503164

Wuhan, China, 7th – 11th April 2025

Agenda Item:	6.17.3

Source: 	Nokia

Title: 	TP to TR 38.749: BS RF requirements for Protection of EESS

Document for:	Approval

# 1.	Introduction

The revised WID on mmWave UE spurious emission was approved at TSG RAN#107 [1]. One of the objectives of this WI is:

- For BS/IAB/Repeater, reflect the latest regulatory requirements on protection of Earth Exploration Satellite Service requirements if any

This contribution provides a text proposal to update the BS RF requirements clause in TR 38.749 [2] with the RAN4 agreements in previous meetings.

# 2.	Discussion

The changes in the revised WID [1] are to clarify the additional UE spurious emission requirements are agreed to be specified from Rel-15 onwards:

- For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 from Rel-15 till Rel-19, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes (FR2 PC1, PC2, PC3, PC4, PC5, PC6, PC7) are considered in the above work - Note: Considering the transition date(s) for the applicability of 2 sets of unwanted emissions introduced by the ITU-R Radio Regulations, RAN4 has agreed to apply the new requirement from Rel-15.

The BS/IAB/Repeater requirements were discussed at TSG RAN4#112-bis and TSG RAN4#113, and corresponding Rel-19 draft CRs [2-7] were endorsed to reflect the latest regulatory requirements. On the other hand, formal CRs in earlier releases [8-13] were also agreed at TSG RAN4#113 and TSG RAN4#114 as specification maintenance because the related regulatory requirements came into force on 1 January 2024. The agreed changes in the earlier releases will be automatically propagated into the Rel-19 specifications when they are created, so there is no need for formal CRs in Rel-19.

Therefore, RAN4 has also agreed to apply the new BS/IAB/Repeater requirements in the earlier releases of the specifications from Rel-15 onwards. It is proposed to approve the text proposal below to record this agreement in TR 38.749. The WID should also be revised the clarify this RAN4 agreement for BS/IAB/Repeater.

# 3.	Text Proposal

<Start of change>

# 9	BS RF requirements

The Earth Exploration Satellite Service (EESS) protection requirements in the frequency range 23.6-24.0 GHz (as per WRC-19 Radio Regulations Resolution 750 [7] and EU Decision 2020/590 [8]) were specified via additional OTA OBUE requirements and additional OTA spurious emission requirements in the BS specifications.

The only modification in legacy specifications was that ITU-R applicability date (i.e. 1 September 2027) was considered in those specifications. Within this WI, the BS additional OTA OBUE limits and additional OTA transmitter spurious emissions requirements for protection of EESS were defined as:

- The emission level of -3dBm/200MHz applies to BS brought into use on, or before 1 September 2027 in countries not adopting EU Decision 2020/590 [8], while

- The emission level of -9dBm/200MHz applies to BS brought into use after 1 September 2027, or to BS in countries adopting EU Decision 2020/590 [8].

RAN4 has agreed to apply the new BS/IAB/Repeater requirements in the earlier releases of the specifications from Rel-15 onwards.

<End of change>

# References

[1]	RP-250818, “Revised WID: mmWave in NR: UE spurious emissions and EESS (Earth Exploration Satellite Service) protection”, China Unicom.

[2]	R4-2417085, “Draft CR to TS 38104 - EESS protection - BS RF aspects”, Ericsson, Nokia, Huawei, ZTE, CATT.

[3]	R4-2500145, “TR 38.749 v0.1.0 Protection of Earth Exploration Satellite Service (EESS)”, Huawei, HiSilicon.

[3]	R4-2417533, “Draft CR to TS 38.141-2 on EESS protection”, Nokia.

[4]	R4-2417674, “DraftCR on TS38.106 on mmWave EESS protection for NCR”, China Unicom.

[5]	R4-2417085, “Draft CR for TS 38.115-2, On EESS protection in mmWave”, CATT.

[6]	R4-2418708, “draft CR on TS38.174_IAB BS EESS protection”, ZTE Corporation, Sanechips.

[7]	R4-2419673, “Draft CR to TS 38.176-2: implementation of updated EESS protection”, Huawei, HiSilicon.

[8]	R4-2418922, “(NR_newRAT-Core) CR to TS 38104 - EESS protection CEPT”, Ericsson.

[9]	R4-2419010, “(NR_newRAT-Perf) CR to TS 38.141-2 on EESS protection”, Nokia.

[10]	R4-2419870, “(NR_repeaters-Core) CR on TS38.106 on mmWave EESS protection for NCR for R17”, China Unicom.

[11]	R4-2418701, “(NR_IAB-Core) CR on TS38.174_IAB BS EESS protection”, ZTE Corporation, Sanechips.

[12]	R4-2419842, “(NR_IAB-Perf, NR_mmWave_protect-Perf) Implementation of updated EESS protection requirement notes”, Huawei, HiSilicon.

[13]	R4-2502330, “(NR_repeaters-Perf)CR for TS38.115-2: On mmWave EESS protection”, CATT.