3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2503167

Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.13.5.3

Source: 	Nokia

Title: 	Simplification of BS TRP test methods

Document for:	Approval

# 1.	Introduction

The revised WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at TSG RAN#106 [1]. The objective of this WI on simplification of BS TRP test methods is revised as follow:

- TRP simplification: Simplify BS TRP estimation methods (e.g. Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods.

This topic was discussed at TSG RAN4#114 [2] and the WF was agreed [3]. This contribution provides proposals to progress this topic in RAN4.

# 2.	Discussion

It was agreed in the WF [3]:

- Agreement: - To keep the current TRP testing method, TRP sampling grid and testing chamber in the specification - Further discuss how to improve the test procedure and the applicability of testing method for different requirements in Annex N of TS 38.141-2.

To improve the test procedure, it was proposed [4,5] to define a sequence for the TRP measurement procedures to be used in the test. The TRP measurement procedures are described in the normative annex I of TS 38.141-2 [6], where it is stated in the General annex I.1:

The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values. The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex I.2 to annex I.9.

The TRP measurement procedures that can provide an accurate estimate of TRP values are described in annex I.2 to annex I.9. The TRP measurement procedures described in annex I.10 to annex I.12 use peak emission to estimate the TRP that can provide an over-estimate of TRP values. Note that it is stated that pre-scan is used for the spurious emission frequencies so it is not intended to be used for in-band frequencies.

The main issue for the two similar proposals is that there is no option for the test house to skip some intermediate TRP sampling grids in case it deems appropriate, e.g., results in pre-scan have shown that the BS under test would be unlikely or marginal to pass the test requirements with the intermediate TRP sampling grids, and thus it makes sense to measure with the dense grid directly instead of going through the intermediate grids first. Therefore, it is proposed to allow the test house to skip some of the intermediate TRP sampling grids in the proposed sequence for the TRP measurement procedures, as described in the following:

OTA base station output power, ACLR and operating band unwanted emissions: Step 1: Orthogonal cut grid (two-cuts and optional three cuts) as described in annex I.5 or Beam-based directions as described in annex I.10 may be performed if chosen for the test, go to step 2 if this step is not chosen or the BS under test fails the test in this step. Step 2: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

OTA spurious emissions: Step 1: Pre-scan as described in annex I.13. Step 2: Orthogonal cut grid (two-cuts and optional three cuts) as described in annex I.5 or Peak method as described in annex I.11 or Equal sector with peak average as described in annex I.12 may be performed if chosen for the test, go to step 3 if this step is not chosen or the BS under test fails the test in this step. Step 3: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

# 3.	Conclusion

This contribution has provided proposals to progress the objective of the WI for TRP simplification in RAN4, it is proposed to allow the test house to skip some of the intermediate TRP sampling grids in the proposed sequence for the TRP measurement procedures, as described in the following:

OTA base station output power, ACLR and operating band unwanted emissions: Step 1: Orthogonal cut grid (two-cuts and optional three cuts) as described in annex I.5 or Beam-based directions as described in annex I.10 may be performed if chosen for the test, go to step 2 if this step is not chosen or the BS under test fails the test in this step. Step 2: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

OTA spurious emissions: Step 1: Pre-scan as described in annex I.13. Step 2: Orthogonal cut grid (two-cuts and optional three cuts) as described in annex I.5 or Peak method as described in annex I.11 or Equal sector with peak average as described in annex I.12 may be performed if chosen for the test, go to step 3 if this step is not chosen or the BS under test fails the test in this step. Step 3: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC.

[2]	R4-2500565, “Topic summary for [114][304] NR_BS_RF_Part3_OTA_TRP”, ZTE.

[3]	R4-2502375, “Way Forward for [114][304] NR_BS_RF_Part3_OTA_TRP”, ZTE Corporation.

[4]	R4-2501647, “TS 38.141-2 Annex I: Discussion on improvement of TRP measurement methods”, Ericsson.

[5]	R4-2501879, “Discussion on the applicability improvement of TRP measurement”, ZTE Corporation, Sanechips.

[6]	3GPP TS 38.141-2 v19.0.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.