3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2503168

Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.26.3

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#105 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#114 [2] and the WF was agreed [3].

This contribution provides proposals to progress the issues according to the agreed WF.

# 2.	Discussion

2.1	Concept of LP-WUS power boosting and associated detail

It is stated in the agreed WF as follows:

- Adopt following content as the concept of LP-WUS power boosting. - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Clarify the declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate. - The minimum power boost level is included in the BS manufacturer declaration table, but not defined as a minimum requirement. - FFS on the detail of the minimum power boosting level, e.g. - Whether it can be different for different BS CBW

As the concept of LP-WUS power boosting is based on that of NB-IoT power boosting, the current BS manufacturer declarations for NB-IoT support (extracted below from TS 38.141-1 [4]) can be used as reference for LP-WUS.

| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x |  |
| --- | --- | --- | --- | --- |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). | x |  |

As an example, the BS manufacturer declarations for LP-WUS support can be specified as follows:

| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS for each supported band, frequency range and channel bandwidth. | x |  |
| --- | --- | --- | --- | --- |
| D.y | LP-WUS power dynamic range | If the BS supports LP-WUS operation, manufacturer shall declare the maximum power dynamic range it could support with a minimum of [+3dB] (Note z). | x |  |
| NOTE z: This manufacturer may declare a different value for each supported band, frequency range and channel bandwidth. |  |  |  |  |

Note that BS manufacturer may declare a single value for the supported LP-WUS power boost level, in which case it is likely to be limited by the narrowest bandwidth supported by the BS, or the BS manufacturer may declare a different value for each supported band, frequency range and channel bandwidth.

2.2	How to comprehend the de-boost on the NR transmission power

It is stated in the agreed WF at TSG RAN4#113 [5] as follows:

- Proposals - Proposal 1: LP-WUS power boosting does not change the total maximum transmit power of BS. (CATT) - There is no power boosting for the case where all RBs are used by LP-WUS and 0 RBs for NR, i.e., SCS 30kHz and CBW 5MHz. - For the cases where there would be RB(s) allocated to NR, x dB NR power degradation is allowed. - Proposal 2: The average power over all REs should be fixed and when the power boost of WUS is enabled with or w/o the NR signal at the same time: (Ericsson) - The WUS signal should be transmitted always with x dB above the average power. - There is no NR signal de-boosting in normal transmission. - The NR de-boosting transmission power is for the testing purpose only and whether to de-boost NR signal transmission power should be up to network implementation. - WF - Consider the de-boosting behaviour together with other relevant issues - Companies are encouraged to think about the draft spec language to reflect corresponding proposals

This topic was discussed at TSG RAN4#114 [2] but no conclusion was reached. On the other hand, it is stated in the agreed WF at TSG RAN4#110-bis [6] as follows:

- With following assumption to guide future discussion, RAN4 will revisit the above concept after BS core requirements are settled. - Transmission power is shared between NR OFDM signal and NR LP-WUS for the same carrier. - The rated carrier output power and rated total output power are not changed with LP-WUS power boosting.

As the rated carrier output power and rated total output power are not changed with LP-WUS power boosting, the BS must attain the required power for LP-WUS power boosting from by reducing the transmission power for NR signal, either by de-boosting the NR signal (i.e., PSD reduction) or reducing the transmission bandwidth of the NR signal (i.e., RB blanking). However, this should be treated as implementation issue of the BS scheduler and thus there is no need to reflect this in the specifications.

2.3	General consideration on other requirements

It is stated in the agreed WF as follows:

- For RAN4 simulation campaign and discussion in next meeting, the latest RAN1 agreements on waveform design are expected to be used. - FFS on the following options, others are not precluded. - Option 1: No need to specify transmit signal quality requirement for LP-WUS. - Option 2: Instead of defining transmit signal quality requirement for LP-WUS, just do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

On option 1, the simulation results on the LP-WUS BLER Vs SNR with OOK-based (ED) and OFDM-based (IQ) receivers and different EVM levels (EVM%) using the latest RAN1 agreements [7] are provided in figure 1.

(a) M=2										(b) M=4

Figure 1: LP-WUS BLER Vs SNR with different EVM levels

It can be observed that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirement do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. Therefore, there is no need to specify the EVM requirement for LP-WUS/LP-SS.

On option 2, the simulation results on the PAPR (at different percentiles) with different transmission schemes (LP-WUS with or without adjacent NR 64/256QAM) using the latest RAN1 agreements [7] are provided in figure 2 below. Note that the first groups of bars in each sub-figure (a-c) include the PAPR of different QAM signals for NR as references.

(a) 90%ile PAPR

(b) 99%ile PAPR

(c) 99.9%ile PAPR

Figure 2: PAPR with different transmission schemes

It can be observed that the impact on overall PAPR (at different percentiles) is minor (around 0.2 dB) when LP-WUS is transmitted together with the QAM signals for NR. Therefore, there is no need to do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

# 3.	Conclusion

This contribution has provided proposals to progress the issues according to the agreed WF in last RAN4 meeting.

Proposal 1: To specify the BS manufacturer declarations for LP-WUS support as follows:

| D.x | LP-WUS operation | Manufacturer shall declare the support of LP-WUS for each supported band, frequency range and channel bandwidth. | x |  |
| --- | --- | --- | --- | --- |
| D.y | LP-WUS power dynamic range | If the BS supports LP-WUS operation, manufacturer shall declare the maximum power dynamic range it could support with a minimum of [+3dB] (Note z). | x |  |
| NOTE z: This manufacturer may declare a different value for each supported band, frequency range and channel bandwidth. |  |  |  |  |

Proposal 2: How to attain the required power for LP-WUS power boosting should be treated as implementation issue of the BS scheduler and thus there is no need to reflect this in the specifications.

Proposal 3: There is no need to specify the EVM requirement for LP-WUS/LP-SS.

Proposal 4: There is no need to do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

# References

[1]	RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[2]	R4-2500569, “Topic summary for [114][308] NR_LPWUS”, Huawei.

[3]	R4-2502305, “WF for [114][308] NR_LPWUS”, Huawei.

[4]	3GPP TS 38.141-1 v19.0.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.

[5]	R4-2419802, “WF for [113][308] NR_LPWUS”, Huawei.

[6]	R4-2406140, “Way Forward for [110bis][314] NR_LPWUS”, Huawei.

[7]	RP-250194, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.