3GPP TSG-RAN WG4 Meeting #114bis	R4-2503189

Wuhan, China,  7th April – 11th April 2025

Agenda item:	7.1.1.2.1

Source:	Apple

Title:	On Rel-19 power domain enhancement for FR1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

##### 2.1 Converting outer RB allocations into inner RB allocations

#### Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible

RAN4 found that a viable scheme for the power domain enhancement is the conversion of ‘outer’ RB allocations into ‘inner’ RB allocations. This is achieved via channel extension. It is proposed that the total number of RBs (“original UE channel RBs” + “RB extension”) is an element of Table 5.3.2-1 from TS 38.101-1.

When extending the UE CBW, the combined bandwidth should be related to the set of specified channel sizes from TS 38.101-1 clause 5.3.2. Higher granularity would provide issues for correctly applying the RF spectrum emission requirements and increase implementation complexity. The RF spectrum emission requirements are defined with respect to the channel sizes of clause 5.3.2. Consequently, any extension which does not lead to a regular channel size would result into undefined emission requirements.

Proposal 1: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should be an element of Table 5.3.2-1 from TS 38.101-1. No intermediate extension shall be allowed as it introduces increased complexity for implementation and testing.

Further simplification could be achieved by keeping the extension equal on both sides. If deployment scenarios allow, then avoid defining extension where the lower side is extended by X while the upper side is extended by Y. This approach reduces two-dimensional complexity for implementation to a one-dimensional issue. The UE testing complexity and time for UE vendor and RAN5 is reduced as less variations are available.

Proposal 2: If deployment scenarios allow, then avoid defining extension where the lower side is extended by X while the upper side is extended by Y. Keep the extension equal on both sides. This proposal does not restrict one sided extension which would therefore still be allowed.

##### 2.2 On IBE

The discussion on IBE has not settled and the issue on how to properly implement this requirement is still open.

#### Issue 1-1-4: How to derive the extended IBE - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW

In general, concerns of a UE producing strong unwanted emissions inside the extended channel should not be large. Aside from the channel extension feature the UE is already tested for compliance to ACLR, SEM and Spur. The main difference is that the output power can be up to 1.5dB higher when the channel extension is applied.

- Consider the network configuring a channel being equal to original CBW plus the extension. If the network would only schedule the RB allocations which are part of inner RB allocations then it would have to conform to the IBE of that channel. Concerns that a UE would create significant emissions just because smaller channel is configured together with the channel extension feature do not seem to be justified.

- In case of channel extension feature being enabled the BB filter of the UE would be applied with respect to the original channel which adds additional confidence that emissions should behave gracefully.

The question would be whether RAN4 even needs to define dedicated emission requirements inside the extended channel. The same hardware is already tested for regular conformance. This is just a different context where the UE is tested again for a very similar emission performance. There is no need to apply unnecessary burden on a UE which has already been tested for conformance with regular channels and emission requirements (SEM, ACLR, Spur etc.).

Observation 1: Aside from the channel extension feature the UE is already tested for compliance to ACLR, SEM and Spur. The main difference is that the output power is up to 1.5dB higher when the channel extension is applied. Worries about new spurs or other issues popping up do not seem justified. There is no need to apply unnecessary burden on a UE which has already been tested for conformance with regular channels and emission requirements (SEM, ACLR, Spur etc.).

Since it might not be acceptable to define channel extension without requirements, at least adopt the most relaxed requirements. The extended IBE requirement shall align with the following rules:

- The updated requirements shall not introduce a requirement inside a frequency region (i.e. guard band of the original channel) where no emission requirement existed in the first place. A new emission requirement inside the guard band would have to be very mild to not require additional power back-off. But if it is mild then the question is why RAN4 should define such a requirement. Therefore, the Guard Band of the original UE channel shall be preserved. That means that the extended IBE requirement shall not define emission limit inside a frequency region which was formerly excluded from any emission requirement. No requirements shall apply to  extended RBs which are overlapping with the guard band of the original channel.

- The guard of the extended channel does not require exception. This frequency region has already been subject to emission requirement. Most likely subject to the General Spurious Emission requirements. Therefore, this frequency range could be subject to the extended IBE requirement under the side condition that the extended IBE represents the same or a relaxed limit.

- For partially filled channel the IBE inside the original UE channel shall be calculated as usual. No change in requirement is needed for this case. The IBE requirement outside the original UE channel can be extended from the outermost IBE limit. In case the allocation borders the lower or upper channel edge the IBE can be calculated by setting ΔRB to 0 and apply the requirement to the entire lower or upper extension (by preserving the guard band).

- The IBE for fully allocated channels be calculated as by setting the ΔRB to 1

- Using the proposed IBE from Pi/2 BPSK would avoid tightening and allow more widely adoption on UE side.

With the requirements stated above we find Option2 acceptable (from ‘Others’ in Issue 1-2-3 from WF [3]) if no requirements apply to the extended RBs which are overlapping with the guard band of the original channel (circled RBs).

Proposal 3: The RB extension IBE proposal Option2 from ‘Others’ in Issue 1-2-3 could be acceptable if no requirements apply to the extended RBs which are overlapping with the guard band of the original channel (circled RBs).

##### 2.3 On A-MPR

The WF [2] from RAN4#113 proposes further discussion on a method to enable power domain enhancement for network signalling:

#### Issue 1-1-3: A-MPR handling - WF - FFS, considering that additional emission in most cases are applied on one side of the UE CBW

When specifying A-MPR for a certain network signalling label various simulations are done to identify those RB allocations which are limited by the additional emission requirement. The RB allocations are grouped into regions allowing additional power back-off compared to MPR. It is important to understand that the typical procedure is to only define A-MPR for those RB allocations which are exceeding MPR. If the emission limit requires power back-off below or close to MPR then there is often no A-MPR assigned. This can be done due to the total allowed power back-off being determined through max(MPR, A-MPR).

| Example channel where A-MPR is only defined for those RB allocations with power back-off exceeding MPR: | MPR enhancement scheme where all outer allocations are converted into inner allocations: |
| --- | --- |

Figure 1: Unintended power back-off tightening if power domain enhancement scheme is applied to channels which are subject to A-MPR

When converting PC3 ‘outer’ allocations into ‘inner’ allocations in an A-MPR scenario the power back-off allowance could be tightened by up to 1dB for DFT-s-OFDM and up to 1.5dB for CP-OFDM. In case of PC2 the maximum tightening is even larger for ‘edge’ allocations as those feature increased MPR compared to ‘outer’ allocations. If the power domain enhancement scheme is directly applied to channels being subject to A-MPR then it might imply considerable tightening for RB allocations which are limited by the additional emission requirement but are not part of an A-MPR region. We have a strong preference to only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation.

Figure 2: Example for NS scenario with channel subject to MPR and A-MPR

The figure 2 provides an example for typical NS scenario where channels close to the emission requirement are subject to A-MPR while channels with larger guard gap do not have A-MPR specified. To avoid unintended power back-off tightening, the power domain enhancement scheme shall be only applied to channels C, D, E etc. as those are not subject to A-MPR. If the enhancement scheme is only applied to channels which are not subject to A-MPR then MPRenh can be directly applied on all RB allocations of this channel.

Observation 2: The typical procedure for defining A-MPR and regions is to only define A-MPR for those RB allocations which are exceeding MPR. If the emission limit requires power back-off below or close to MPR then there is often no A-MPR assigned. This can be done due to the total allowed power back-off being determined through max(MPR, A-MPR).

Observation 3: If the power domain enhancement scheme of converting ‘outer’ to ‘inner’ is directly applied to channels being subject to A-MPR then it might imply considerable tightening for the power back-off of up to 1.5dB for PC3 and up to 3.5dB for PC2.

Observation 4: Tightening the power back-off might require additional testing which adds time and cost to a UE supporting the new power domain enhancement scheme.

Proposal 4: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations are made:

Converting outer RB allocations into inner RB allocations:

Proposal 1: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should be an element of Table 5.3.2-1 from TS 38.101-1. No intermediate extension shall be allowed as it introduces increased complexity for implementation and testing.

Proposal 2: If deployment scenarios allow, then avoid defining extension where the lower side is extended by X while the upper side is extended by Y. Keep the extension equal on both sides. This proposal does not restrict one sided extension which would therefore still be allowed.

On IBE:

Observation 1: Aside from the channel extension feature the UE is already tested for compliance to ACLR, SEM and Spur. The main difference is that the output power is up to 1.5dB higher when the channel extension is applied. Worries about new spurs or other issues popping up do not seem justified. There is no need to apply unnecessary burden on a UE which has already been tested for conformance with regular channels and emission requirements (SEM, ACLR, Spur etc.).

Proposal 3: The RB extension IBE proposal Option2 from ‘Others’ in Issue 1-2-3 could be acceptable if no requirements apply to the extended RBs which are overlapping with the guard band of the original channel (circled RBs).

On A-MPR:

Observation 2: The typical procedure for defining A-MPR and regions is to only define A-MPR for those RB allocations which are exceeding MPR. If the emission limit requires power back-off below or close to MPR then there is often no A-MPR assigned. This can be done due to the total allowed power back-off being determined through max(MPR, A-MPR).

Observation 3: If the power domain enhancement scheme of converting ‘outer’ to ‘inner’ is directly applied to channels being subject to A-MPR then it might imply considerable tightening for the power back-off of up to 1.5dB for PC3 and up to 3.5dB for PC2.

Observation 4: Tightening the power back-off might require additional testing which adds time and cost to a UE supporting the new power domain enhancement scheme.

Proposal 4: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2503013, “WF on Power domain enhancement”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114

- R4-2418170, “Discussion on UE RF power enhancement for NR single carrier”, vivo, 3GPP TSG-RAN4 Meeting #113