3GPP TSG-RAN WG4 Meeting #114bis	R4-2503190

Wuhan, China,  7th April – 11th April 2025

Agenda item:	6.11.2

Source:	Apple

Title:	On simultaneous RxTx for CA_n40A-n41A

WI/SI:	LTE_NR_R19_Simult_RxTx-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

A new work item [1] on simultaneous Rx/Tx has been agreed in RAN#104. One of its objectives is to clarify and improve existing mandatory simultaneous Rx/Tx requirements. This discussion paper discusses three main topics. The first topic is about the Rel-17 agreement on FR1+FR1 FDD-TDD combinations and the missing requirement in TS 38.101-1/3. The second topic is about the correct handling and request of fallbacks. The third topic is about CA_n40A-n41A and the requirements when supporting simultaneous Rx/Tx. The l

# 2  Discussion

#### 2.1 On MIMO performance of CA_n40A-n41A

At RAN4#114 meeting it has been noted that RAN2 signaling could potentially grant the indication of the same band combination twice, once with one set of capabilities and once with another, by utilizing distinct feature-sets. The WF [2] captures the request for further confirmation:

Way forward on Simultaneous Rx/Tx for CA_n40A-n41A: It has been observed that RAN2 signalling might allow to indicate the same band combination twice with different capabilities by utilizing separate feature-sets. Companies are encouraged to check if current RAN2 framework allows to indicate different number of MIMO layers with and without simultaenous Rx/Tx.

To further confirm the feature-set solution we conducted an in-depth analysis on the RAN2 signalling framework. The challenge with IE simultaneousRxTxInterBandCA and IE maxNumberMIMO-LayersPDSCH is that the first one is part of IE CA-ParametersNR while the latter is part of IE FeatureSets without obvious connection to each other.

RAN2 specifies a band combination list in which the band combination information elements are contained. Each entry from IE BandCombinationList features a bandList which contains the bands from the DL combo. A UE can indicate support for additional capabilities like simultaneousRxTxInterBandCA and simultaneousRxTxInterBandCAPerBandPair through the optional information element CA-ParametersNR. The band combination element refers to a specific feature-set combination via a FeatureSetCombinationId. It can be viewed as a pointer to a specific entry from IE FeatureSetCombination. This information element bundles the individual uplink and downlink feature-sets for each band of the band combination. This is achieved by referencing to the actual feature-set via the FeatureSetDownlinkId and the FeatureSetUplinkId for each band individually. The IE FeatureSets consists of a sequence of information elements which itself contains the parameters for uplink and downlink. The maximum DL-MIMO layer indication is provided by maxNumberMIMO-LayersPDSCH and part of IE FeatureSetDownlinkPerCC.

Figure 1 depicts the signaling framework with focus on the relation of simultaneous Rx/Tx and the maximum supported DL-MIMO layers. It visualises the case of a UE indicating the band combination CA_n40A-n41A twice to support different capabilities, once with support of simultaneous Rx/Tx and once without its support. In this example the individual band combination entries refer to FeatureSetCombinationId 0 and 1 from the FeatureSetCombination list. The elements from this list contain references to different feature-sets which either specify fourLayer or twoLayer DL-MIMO support.

Figure 1: Simplified scheme on RAN2 signalling framework depicting the relation of simultaneous Rx/Tx and max supported DL-MIMO layers

Observation 1: RAN2 signalling framework allows a UE to indicate the support of a specific band combination multiple times with different capabilities. In case of CA_n40A-n41A it creates the option to indicate the band combination twice with different capabilities for simultaneous Rx/Tx and number of supported DL-MIMO layers.

The signalling framework enables a UE to indicate reduced DL MIMO rank to the network which has the potential to improve throughput at low to medium SNR ranges. The benefit of reduce DL MIMO rank has been analysed for self-interference limited handhelds in [3]. Low to medium SNR roughly translates to cell edge and mid cell locations which represents the largest extent of the cell. In summary self-interference limited handhelds can support simultaneous Rx/Tx by indicating two-layer DL MIMO. There is one remaining caveat. A handheld which only supports two-layer DL MIMO on band n41 for simultaneous Rx/Tx operation needs to pass the reference sensitivity exception test for cross-band isolation with 4Rx REFSENS requirement. This is due to RAN4 specification referring to the number of supported/equipped antennas for a specific band. When the Feature-Set solution was first discussed last RAN4 meeting it was assumed that it would also allow the UE to indicate 2Rx support/equipment for the specific band combination. However, RAN2 framework does not support clear signalling with respect to the supported/equipped number of antenna ports. It is not possible to indicate support/equipment of 2Rx which forces a self-interference limited handheld to conform to 4Rx REFSENS. This requirement poses an unnecessary constraint and should not be applicable in this scenario but instead be replaced by 2Rx REFSENS. With the fundamental issue for self-interference limited handhelds laid out above, it is proposed to apply the 2Rx REFSENS if two-layer DL MIMO is indicated with simultaneous Rx/Tx for a band in a band combination. To keep modification at a minimum it shall be implemented case-by-case.

Observation 2: RAN2 framework does not support clear signalling with respect to the supported/equipped number of antenna ports. It is not possible to indicate support/equipment of 2Rx for a specific band combination.

Observation 3: A UE which supports simultaneous Rx/Tx with four-layer DL-MIMO on band n41 needs to pass the MSD test requirements with 4Rx REFSENS. A UE which only supports two-layer DL-MIMO still needs to pass the MSD test with 4Rx REFSENS even though only 2Rx are used in the field for this specific configuration.

Proposal 1: It is proposed to apply 2Rx REFSENS if two-layer MIMO is indicated with simultaneous Rx/Tx for a band in a band combination. For minimum modification it is also proposed to implement on case-by-case basis.

Proposal 2: Consider implementation of 2Rx REFSENS for CA_n40A-n41A as proposed below:

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. …. n40; 15; 5, 10, 15, 20, 25, 30, 40, 50; -100 + 10log10(NRB/25); TDD. 30; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.5 + 10log10(NRB/11). n411,11, n901,11; 15; 5, 10, 15, 20, 25, 30, 35, 40, 45, 50; -94.8 + 10log10(NRB/52); TDD. 30; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.5 + 10log10(NRB/11). …. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. Note 11: If carrier aggregation with band n40, simultaneous Rx-Tx operation and two-layer DL MIMO is configured then 2Rx REFSENS applies. This is not applicable to FWA form factor devices and UEs indicating IE [TBD] for 6Rx capability. [Alternative Note 11:] If the UE indicates maxNumberMIMOLayers-PDSCH = 2 for band n41 within a CA band combination, then the 2Rx REFSENS requirement applies.

In a typical deployment scenario simultaneous Rx/Tx does not occur for every slot. If the network configures the same TDD format for both bands but utilises flexible slots divergently, simultaneous Rx/Tx is typically infrequent compared to the total number of symbols. In case the network configures different TDD formats, the chance of simultaneous receive and transmit is highly dependent on the difference between the two formats. It can be infrequent or common. In any way, from throughput perspective it would be more beneficial for a self-interference limited handheld to flexibly switch between two-layer and four-layer DL MIMO. Two-layer DL MIMO would be scheduled for slots where Tx on one band and Rx on the other band is occurring. Four-layer DL MIMO would be scheduled for slots where simultaneous transmit on both bands is occurring and for slots where simultaneous receive on both bands is occurring. This flexible approach would minimise self-interference and maximise throughput. However, this is currently not possible with today’s signalling.

Observation 4: Flexible switching between two-layer and four-layer DL MIMO, based on TDD format, would minimize self-interference and allow to maximize throughput. However, current signaling limitations prevent this approach.

# 4	Conclusions

This contribution provides considerations on concurrent MIMO and simultaneous Rx/Tx operation. The following observations and proposals are made:

Observation 1: RAN2 signalling framework allows a UE to indicate the support of a specific band combination multiple times with different capabilities. In case of CA_n40A-n41A it creates the option to indicate the band combination twice with different capabilities for simultaneous Rx/Tx and number of supported DL-MIMO layers.

Observation 2: RAN2 framework does not support clear signalling with respect to the supported/equipped number of antenna ports. It is not possible to indicate support/equipment of 2Rx for a specific band combination.

Observation 3: A UE which supports simultaneous Rx/Tx with four-layer DL-MIMO on band n41 needs to pass the MSD test requirements with 4Rx REFSENS. A UE which only supports two-layer DL-MIMO still needs to pass the MSD test with 4Rx REFSENS even though only 2Rx are used in the field for this specific configuration.

Proposal 1: It is proposed to apply 2Rx REFSENS if two-layer MIMO is indicated with simultaneous Rx/Tx for a band in a band combination. For minimum modification it is also proposed to implement on case-by-case basis.

Proposal 2: Consider implementation of 2Rx REFSENS for CA_n40A-n41A as proposed below:

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. …. n40; 15; 5, 10, 15, 20, 25, 30, 40, 50; -100 + 10log10(NRB/25); TDD. 30; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.5 + 10log10(NRB/11). n411,11, n901,11; 15; 5, 10, 15, 20, 25, 30, 35, 40, 45, 50; -94.8 + 10log10(NRB/52); TDD. 30; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.5 + 10log10(NRB/11). …. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. Note 11: If carrier aggregation with band n40, simultaneous Rx-Tx operation and two-layer DL MIMO is configured then 2Rx REFSENS applies. This is not applicable to FWA form factor devices and UEs indicating IE [TBD] for 6Rx capability. [Alternative Note 11:] If the UE indicates maxNumberMIMOLayers-PDSCH = 2 for band n41 within a CA band combination, then the 2Rx REFSENS requirement applies.

Observation 4: Flexible switching between two-layer and four-layer DL MIMO, based on TDD format, would minimize self-interference and allow to maximize throughput. However, current signaling limitations prevent this approach.

# 4	References

- RP-242088, “Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19”, Huawei, 3GPP TSG RAN Meeting #105

- R4-2502840, “WF on simultaneous Rx-Tx for CA_n40A-n41A”, Huawei, 3GPP TSG-WG4 Meeting #114

- R4-2500305, “On simultaneous RxTx for CA_n40A-n41A”, Apple, 3GPP RAN WG4 Meeting #114

- R4-2503014, “WF on 6Rx requirements”, AT&T, 3GPP TSG-RAN WG4 Meeting #114