3GPP RAN WG4 Meeting #114bis																R4-2503191

Wuhan, China, 7th – 11th April 2025

Agenda item:	7.26.2.2

Source:	Apple

Title:	On LP-WUR Rx requirements

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. During RAN4#114 a fundamental agreement was made with respect to different sets of requirements. It was decided to consider two sets which can feature different performance in noise figure, interference and differ in modulation aspects. Only if those sets turn out to be similar it shall be considered to merge those.

Issue 1-1-1: How to define two LP-WUR types Agreement: - Define two set of RF requirements as the baseline, considering the following aspects including - Noise figures - Interference performance - Modulation - Consider to merge two sets of requirements into one if the requirements are close to each other

# 2  Discussion

##### 2.2 ACS and blocking aspects

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB Agreements - Same interference signal level as MR for LP-WUR ACS requirements - FFS at least for one set of Rx requirements. - The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point - FFS whether two sets of LP-WUS wanted signal level can be defined. - Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation - FFS number of guard RBs. Issue 3-1-1: IBB, OBB requirements Way forward: - Same interference level as MR - FFS at least for one set of Rx requirements. - FFS on wanted signal in blocking test.

Last meeting a fundamental agreement was reached regarding various sets of requirements. Two sets were chosen that may exhibit different noise figures, interference characteristics, and modulation aspects. Only if these sets are found to be similar will they be considered for merging. A second set of requirements allows to define relaxed requirements. Relaxation for ACS, IBB and OBB is highly important for devices which implement a dedicated LP focusing on low power and low cost. It has been discussed (e.g. in [3]) that it is imperative to consider relaxed requirements for at least ACS, IBB and OBB. Relaxations allow to reduce power consumption which is one of the main goals when considering low power receiver design. However, LP might be mostly used close the BS where interference might be higher if another UE were located at cell edge and configured at neighboring channels. RAN4 should further discuss whether ACS, IBB and OBB relaxation for the second set can be useful in the field.

Observation: Relaxation for applications in the areas of ACS, IBB and OBB is of importance for devices that are designed with a dedicated LP focus on minimizing power consumption and cost. These relaxations enable a reduction in power consumption, which is a primary objective when designing low-power receivers. However, it’s worth noting that LP may be primarily utilized close to the BS, where interference levels tend to be higher if another UE were located at the cell edge and configured on neighboring channels.

Proposal: RAN4 should further discuss whether ACS, IBB and OBB relaxation for the second set can be useful in the field.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

IBB and OBB relaxation for the second set can be useful in the field.

Observation: Relaxation for applications in the areas of ACS, IBB and OBB is of importance for devices that are designed with a dedicated LP focus on minimizing power consumption and cost. These relaxations enable a reduction in power consumption, which is a primary objective when designing low-power receivers. However, it’s worth noting that LP may be primarily utilized close to the BS, where interference levels tend to be higher if another UE were located at the cell edge and configured on neighboring channels.

Proposal: RAN4 should further discuss whether ACS, IBB and OBB relaxation for the second set can be useful in the field.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2503028, “WF on UE RF requirements for LP-WUS”, vivo, 3GPP TSG-RAN WG4 Meeting #114

- R4-2500159, “Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS”, Huawei, HiSilicon, TSG-RAN WG4 Meeting #114

- R4-2503003, “LS on LP-WUS UE RF”, vivo, RAN4, 3GPP TSG-RAN WG4 Meeting #114