3GPP TSG-RAN WG4 Meeting #114bis	R4-2503193

Wuhan, China,  7th April – 11th April 2025

Agenda item:	6.12.2

Source:	Apple

Title:	n48: Adding channels of 50MHz and beyond

WI/SI:	NR_bands_CA_ENDC_R19_BWs-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The scope of the Rel-19 WI [1] is the introduction of new channel bandwidth sizes to certain bands. In case of band n48 it is considered to introduce several channel sizes ranging from 50MHz to 100MHz. Due to nearby emission requirements captured by NS_27 there is a need for A-MPR analysis. Due to the large amount of newly added channels and their size in terms of N_RB extensive simulation campaigns are required. The WF [2] proposes a workflow which breaks the extensive work into smaller tasks. While previous contributions have focused on providing simulation results our paper from last meeting [3] had specific proposals for RB regions and A-MPR. This led to the WF [4] capturing the proposed RB regions for further evaluation.

# 2  Discussion

In previous contributions we provided our simulations results and proposed RB regions for channels of 50MHz and beyond. For convenience the table 1 from [3] is provided below as table 1. To our understanding the A-MPR table from NS_27 can be re-used as is. It does not seem necessary to add additional A-MPR columns as A1 to A8 can cover the A-MPR for the newly defined RB regions.

Table 1: RB regions for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz |  | Region A |  |  |  |  |  | Region B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS |  | A-MPR |  | LCRB*12*SCS | A-MPR |
| 50 MHz | 3575 ≤ FC < 3605 |  | <12.6 MHz |  |  |  | A7 |  |  |  |
|  |  |  | ≥12.6 MHz ≤31.0 MHz |  | ≥22.32 MHz |  | A2 |  |  |  |
|  |  |  |  |  | <22.32 MHz |  | A1 |  |  |  |
|  |  |  | ≥38.52 MHz |  | ≤3.6 MHz |  | A7 |  |  |  |
|  | 3645 < FC ≤ 3675 |  |  | >36.0 MHz |  |  |  |  |  |  |
|  |  |  |  | ≤36.0 MHz |  | ≥22.32 MHz |  |  |  |  |
|  |  |  |  | ≥17.6 MHz |  | <22.32 MHz |  |  |  |  |
|  |  |  |  | ≤10.08 MHz |  | ≤3.6 MHz |  |  |  |  |
|  | 3605 ≤ FC ≤ 3645 |  | ≤6.12 MHz |  | ≤3.6 MHz |  | A8 |  |  |  |
|  |  |  |  | ≥42.48 MHz | ≤3.6 MHz |  | A8 |  |  |  |
| 60 MHz | 3580 ≤ FC < 3620 |  | <15.84 MHz |  |  |  | A7 |  |  |  |
|  |  | ≥15.84 MHz ≤31.0 MHz |  |  |  | ≥27.36 MHz | A2 |  |  |  |
|  |  |  |  |  |  | <27.36 MHz | A1 |  |  |  |
|  |  | ≥44.64 MHz |  |  |  | ≤3.6 MHz | A7 |  |  |  |
|  | 3630 < FC ≤ 3670 |  |  | >42.48MHz |  |  |  | A7 |  |  |
|  |  |  |  | ≤42.48 MHz |  | ≥27.36 MHz |  | A2 |  |  |
|  |  |  |  | ≥27.32 MHz |  | <27.36 MHz |  | A1 |  |  |
|  |  |  |  | ≤13.68 MHz |  | ≤3.6 MHz |  | A7 |  |  |
|  | 3620 ≤ FC ≤ 3630 | ≤6.12 MHz |  |  |  | ≤3.6 MHz |  | A8 |  |  |
|  |  |  |  | ≥52.2 MHz |  | ≤3.6 MHz |  | A8 |  |  |
| 70 MHz | 3585 ≤ FC ≤ 3665 | ≤22.32 MHz |  |  |  | <25.56 MHz |  | A7 |  |  |
|  |  |  |  | ≥55.8 MHz |  | <25.56 MHz |  | A7 |  |  |
|  |  |  |  |  |  | ≥25.56 MHz |  | A7 |  |  |
| 80 MHz | 3590 ≤ FC ≤ 3660 | ≤22.32 MHz |  |  |  | <25.56 MHz |  | A7 |  |  |
|  |  |  |  | ≥55.8 MHz |  | <25.56 MHz |  | A7 |  |  |
|  |  |  |  |  |  | ≥25.56 MHz |  | A7 |  |  |
| 90 MHz | 3595 ≤ FC ≤ 3655 | ≤25.92 MHz |  |  |  | <28.8 MHz |  | A7 |  |  |
|  |  |  |  | ≥62.28 MHz |  | <28.8 MHz |  | A7 |  |  |
|  |  |  |  |  |  | ≥28.8 MHz |  | A7 |  |  |
| 100 MHz | 3600 ≤ FC ≤ 3650 | ≤29.16 MHz |  |  |  | <32.4 MHz |  | A7 |  |  |
|  |  |  |  | ≥69.12 MHz |  | <32.4 MHz |  | A7 |  |  |
|  |  |  |  |  |  | ≥32.4 MHz |  | A7 |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Table 2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | 6 |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | 7 |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

Proposal: It is proposed to specify the RB region from Table 1. The A-MPR from NS_27 table can be re-used.

# 4	Conclusions

This contribution provides a first batch of simulation results. RB regions are proposed for channel of 50MHz and beyond. The following proposal is made:

Proposal: It is proposed to specify the RB region from Table 1. The A-MPR from NS_27 table can be re-used.

# 4	References

- RP-241684, “Basket WID on adding channel bandwidth support to existing NR bands”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #104

- R4-2414369, “WF on NS_27 A-MPR for larger than 40MHz UL CBW for n48 “, CableLabs, 3GPP TSG-RAN WG4 Meeting #112

- R4-2500306, “n48: Adding channels of 50MHz and beyond”, Apple, 3GPP TSG-RAN WG4 Meeting #114

- R4-2502841, “WF on channel bandwidth for n48”, Apple, CableLabs, 3GPP TSG-RAN WG4 Meeting #114