3GPP TSG-RAN WG4 Meeting # 114Bis		R4-2503194

Wuhan, Hubei, China, April 07 – 11, 2025

Agenda item:	7.7.2

Source:	Apple

Title:	UE RF requirements for LB CA via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #114, Athens, Greece, February 17 –21, 2025), initial discussions were held on several aspects of the NR_LBCA_Sw work item. As captured in the topic WF [1], no agreements were reached on the following UE RF requirements:

- WID scope clarification

- Reference RF architecture(s)

- Switching time value(s), location, and capability

- Time mask for switching

In this discussion paper, we will share our views on the UE RF requirements listed above.

# Discussion

  - Reference architectures

The UE RF requirements for switching between the FDD band and the SDL band will be greatly influenced by the choice of UE RF architecture. As was highlighted in our previous contribution [2], during the band switching process, several events have to occur within the UE baseband, transceiver, and RF front end. Depending on the UE implementation, the main events that could potentially contribute to the switching time are the following:

- Antenna tuning

- PLL settling and relocking (or retuning), depending on the PLL architecture

- RFFE parameters loading, including ET parameters

- RFIC programming and calibration,  I am safe

To perform the switching, the events above can be performed sequentially or concurrently, depend on the UE implementation and capability. The switching time will be optimized if some of the events listed above can be performed concurrently (or in parallel).

- Antenna tuning

As described in [3], a conceptual RF front-end diagram representing the two switching states for the band combination CA_n5A-n29A is shown in Figure 1 below:

Figure 2.1-1:	Simplified model for realizing CA aggregation between n5 and n29 through switching [2]

Based on the diagram above,  we will assume that the two bands involved in the switching share the same antenna. This means that, as captured in Figure 2.1-2 below, the antenna will cover the frequency range 717MHz-894MHz, which represents a total range of 177MHz and a fractional bandwidth of about 22%. Achieving good LB antenna radiation efficiency for such a high fractional bandwidth is very challenging. Therefore, for optimal antenna radiation efficiency, antenna tuning will be needed for LB-LB switching for CA_n5A-n29A combination.

Observation #1: For optimal antenna radiation efficiency, antenna tuning is needed to implement LB-LB via switching for the CA_n5A-n29A combination.

Figure 2.1-2:	Frequency range for the CA_n5A-n29A combination

- PLL retuning:

One of the main contributors of the switching time is the PLL settling time when the LO is changed. Depending on the receiver architecture, there are two scenarios, depending on whether the PLL retuning is needed or not:

- If the local oscillators (LOs) are generated from the same PLL, LO retuning will be necessary when switching between the two bands.

- However, if the two LOs are generated from two different PLLs (PLL1 and PLL2), PLL retuning may not be necessary, depending on the UE implementation.

Based on the aspects the highlighted above, we can propose two reference architecture options as shown in Figure 2.1-3 to implement CA_n5A-n29A via switching:

- Option 1 architecture: shared antenna with shared LO

- Option 2 architecture: shared antenna with separate LOs

Figure 2.1-3:	Reference architectures for the CA_n5A-n29A combination

Proposal #1: To accommodate different UE implementations, we propose two reference architecture options for the CA_n5A-n29A combination:

- Option 1 architecture: shared antenna and shared LO

- Option 2 architecture: shared antenna with separate LOs

  - Switching period values and capability

From the previous meeting way forward (Meeting #114, Athens, Greece, February 17 –21, 2025), the decision on the switching time is captured below:

As can be seen from the WF on the switching time for switching between an FDD and an  SDL band, the agreement from the previous meeting was to choose the set of two values  35S and 140S. Most of the RF front-ends are controlled through the RFFE MIPI interface with several front-end devices sharing the same control bus. To control these devices, a longer lead time might be required. To accommodate this longer lead time, we believe a switching period of 100S could be also an option. Therefore, we propose adding 100S as an additional value for the switching period.

Proposal #2: To accommodate different UE implementations, we propose a set of three values, 35S, 100S, and 140S,  for the switching period when switching between the FDD band and the SDL band.

As explained in the previous sections, the switching period is affected by several UE events that occur during the switching process. Some of these events could be band combination dependent. For example, antenna tuning may not be required for some band combinations if the fractional bandwidth is small enough so that optimal antenna radiation efficiency can be achieved without tuning. CA_n12A-n29A is such a band combination with only about 6.5% fractional bandwidth. Without antenna tuning, the switching time could be lower for these band combos when compared to the combos that require antenna tuning. For these reasons, the switching period will be band combination dependent.

Proposal #3: The switching delay capability can be per band combination.

  - Time mask and switching period location

From the topic WF, we can highlight the decision made in the last meeting on the time mask as shown below.

For LB-LB CA via switching, we could use, as a baseline, the same concepts developed for the UL Tx switching feature to define the time masks. As such, if we assume that the network always allocates enough scheduling gaps to ensure that there is no victim band (meaning no symbol blanking to accommodate the switching period) because of the semi-static nature of the LB-LB switching, therefore we can define the switching period locations as shown in Figures 2.3-1 and 2.3-2 below. In these diagrams, the switching periods can be located either in the “switch from” band or in the “switch to” band, and either at the beginning or the end of the slots. The exact locations of the periods can be finalized after RAN1 defines the switching patterns and indicates whether or not any additional considerations on prioritizing/protecting certain transmissions/receptions are needed for a victim band.

Figure 2.3-1:	Time mask with the switching period located on the FDD carrier

Figure 2.3-2:	Time mask with the switching period located on the SDL carrier

Proposal #4: As a baseline, we can assume that the network always allocates enough scheduling gaps to accommodate the switching time without blanking additional symbols. In this case, the switching periods can be located either in the “switch from” band or in the “switch to” band, and either at the beginning or the end of the slots. The exact locations of the periods can be finalized after RAN1 defines the switching patterns.

  - Scope clarification

As highlighted in its description [4], the main objective of the WID is to specify the UE requirements when switching from a LB FDD (f<1GHz) band to a LB SDL band to implement LB-LB CA combinations. Given the limited amount of time left to complete this work item, RAN4 should focus on specifying the IE requirements only for 2-band CA combos with one FDD band and one SDL band. However, in the future, the following additional UE aspects could be considered:

- Higher-order band combination:

A LB-LB CA combination that needs to be specified could also be a subset of a higher-order band combinations. For example, CA_n5A-n29A is a subset of CA_n5A-n25A-n29A (3-band combo) and CA_n5A-n25A-n29A-n66A (4-band combo). The effects of switching between n5 and n29 on the other bands may not be negligible should be further considered.

- Higher number of Rx chains:

Likewise, the FDD band involved in the switching could be a 4Rx-capable band. The current 3GPP TS38.101-1 specifications list n5, n8, m13, n26, n28, n71, n85, and n105v as 4Rx bands. Potential LB-LB switching aspects for CA combinations involving those bands might could be considered in the future

Proposal #5: The objective of the Rel-19 WI is to specify UE requirements for switching between one LB FDD band and one LB SDL band. However, additional UE aspect involving higher-order band combinations and higher number of Rx chains should not be precluded and could be considered in the future.

# 3	Conclusions

Observation #1: For optimal antenna radiation efficiency, antenna tuning is needed to implement LB-LB via switching for the CA_n5A-n29A combination.

Proposal #1: To accommodate different UE implementations, we propose two reference architecture options for the CA_n5A-n29A combination:

- Option 1 architecture: shared antenna and shared LO

- Option 2 architecture: shared antenna with separate LOs

Proposal #2: To accommodate different UE implementations, we propose a set of three values, 35S, 100S, and 140S,  for the switching period when switching between the FDD band and the SDL band.

Proposal #3: The switching delay capability can be per band combination.

Proposal #4: As a baseline, we can assume that the network always allocates enough scheduling gaps to accommodate the switching time without blanking additional symbols. In this case, the switching periods can be located either in the “switch from” band or in the “switch to” band, and either at the beginning or the end of the slots. The exact locations of the periods can be finalized after RAN1 defines the switching patterns.

Proposal #5: The objective of the Rel-19 WI is to specify UE requirements for switching between one LB FDD band and one LB SDL band. However, additional UE aspect involving higher-order band combinations and higher number of Rx chains should not be precluded and could be considered in the future.

# 4	References

R4-2503016, “WF on low-low band switching”, by Apple,  3GPP TSG RAN Meeting #114, Athens,  Greece, Athens, Greece, February 17-21, 2025

R4-2500313, “Initial Views on UE RF requirements for LB CA via switching”, by Apple,  3GPP TSG RAN Meeting #114, Athens,  Greece, Athens, Greece, February 17-21, 2025

RP-241537, “Rx switched nX-n29 low-low carrier aggregation”, by TELUS, Bell Mobility, 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20st June 2024

RP-243317, “New WID on low band carrier aggregation via switching”, by Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024