3GPP RAN WG4 Meeting #114bis		R4-2503196

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.1.1.1.1

Source:	Apple

Title:	On Rel-19 HPUE CA DC MSD consideration

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

For HPUE inter-band UL CA/EN-DC, RAN4 has agreed to work on the feasible unified equations, LUT, or simplified guidelines for MSD for HPUE band combinations as normative requirements to replace the existing method using HPUE MSD tables, as captured in the approved “WF on MSD rule” in RAN4 #113 meeting [2]. In last RAN4 meeting, though substantial efforts had been spent by a few companies to generate a set of preliminary LUTs for HPUE MSD as captured in the “WF on MSD rules” (R4-2502864) which however was not agreed and the WF was withdrawn due to the concern that a unified LUT may not be applicable to all MSD mechanisms for single UL transmission where further clarifications on the increase of interference power level and the impact to main and diversity Rx paths are deemed necessary. In addition, how LUT(s) would be generated and whether they are necessary to align with the existing HPUE MSD requirements within certain tolerance may need to be further discussed.

In this contribution, we share our views on the underlining assumptions to enable a unified HPUE MSD PCx for different single UL MSD mechanisms. For 2UL IMD MSD, we also provide our views on whether IMD order is sufficient to design LUT/set of equations or both IMD order and the IMD index should be accounted for. In addition, we propose RAN4 to only specify 2UL IMD MSD with equal UL power to simplify the HPUE MSD requirements with LUT(s).

# Discussion

## 2.1	Single UL MSD

Single UL MSD can be due to UL harmonic, Rx harmonic mixing, or cross-band interference. For UL harmonic and cross-band interference, the mechanisms for MSD aggravation from PC3 to PC2 and PC1.5 in general are quite similar as they both are primarily caused by the inter-modulation products of the aggressor UL due to power amplifier (PA) nonlinearity. On the other hand, for Rx harmonic mixing, the MSD aggravation is mainly due to the aggressor UL power increase, i.e., 3dB and 6dB for PC2 and PC1.5, respectively.

Observation 1: For UL harmonic and cross-band interference, the mechanisms for MSD aggravation from PC3 to PC2 and PC1.5 in general are quite similar as they both are primarily caused by the inter-modulation products of the aggressor UL due to PA nonlinearity.

Observation 2: For Rx harmonic mixing, the MSD aggravation is mainly due to the aggressor UL power increase, i.e., 3dB and 6dB for PC2 and PC1.5, respectively.

In an ideal situation for 1Tx PC2 HPUE, the PA non-linearity (OIPx – Output x order Intercept Point) is expected to be improved by 3.5dB as compared to P3 PA as PC2 ACLR is 1dB tighter than PC3 ACLR, assuming ACLR is dominated by IM3. However, considering the non-ideality in practical PA design, it may be admissible to assume PC2 PA non-linearity is effectively improved by 3dB as compared to PC3 PA. With that assumption, it can be expected that all PC2 IMD products (including harmonics) power would increase by 3dB as compared to PC3, and that would essentially align the interference power increase among all MSD mechanisms.

Observation 3: In an ideal situation for 1Tx PC2 HPUE, the PA non-linearity is expected to be improved by 3.5dB as compared to PC3 PA as PC2 ACLR is 1dB tighter than PC3 ACLR, assuming ACLR is dominated by IM3.

Observation 4: Considering the non-ideality in practical PA design, it may be admissible to assume PC2 PA non-linearity is effectively improved by 3dB as compared to PC3 PA.

Observation 5: With the assumption that PC2 PA non-linearity is effectively improved by 3dB as compared to PC3 PA, it can be expected that the interference power for all MSD mechanisms would increase by 3dB and 6dB for PC2 and PC1.5 respectively.

Despite the interference power increase can be assumed the same for all single UL MSD mechanisms based on the above justification, for UL harmonic and cross-band interference, the MSD aggravation (PCx) could be different between 1Tx and 2Tx configurations due to the interference power imbalance between the main and diversity Rx paths as has been analyzed in [3]. For 2Tx configuration, the PCx also depends on the interference power imbalance amount. On the other hand, for Rx harmonic mixing, the MSD contribution would be dominated by PCB isolation according to the analysis in [4], as a result, the PCx is expected to be the same between 1Tx and 2Tx configurations.

Figure 2.1-1, Figure 2.1-2, and Figure 2.1-3 show the PCx dependency over PC3 MSD with 1Tx configuration, 2Tx configuration with 6dB interference power imbalance, and 2Tx with 9dB interference power respectively based on our analysis by only taking into account interference from PA but not from other RF components.

Figure 2.1-1 PCx dependency over PC3 MSD with 1Tx configuration

Figure 2.1-2 PCx dependency over PC3 MSD with 2Tx configuration and 6dB interference imbalance

Figure 2.1-3 PCx dependency over PC3 MSD with 2Tx configuration and 9dB interference imbalance

Based on the above assessment, for single UL HPUE MSD, 2 LUTs as categorized below can be considered:

- LUT1: 1Tx for UL harmonic and cross-band interference and 1Tx/2Tx for Rx harmonic mixing

- LUT2: 2Tx for UL harmonic and cross-band interference

Another option for further simplification is to have only one LUT for both 1Tx and 2Tx configurations where the PCx values shall accommodate both configurations with potential compromising between 1Tx and 2Tx.

Proposal 1: RAN4 to consider the following two options for single UL HPUE MSD LUT definition:

- Option 1: Define 2 LUTs as categorized below:

  - LUT1: 1Tx for UL harmonic and cross-band interference and 1Tx/2Tx for Rx harmonic mixing

  - LUT2: 2Tx for UL harmonic and cross-band interference

- Option 2: Define only one LUT for both 1Tx and 2Tx configurations where the PCx values shall accommodate both configurations with potential compromising between 1Tx and 2Tx.

## 2.2	2UL IMD MSD

For 2UL IMD MSD, prior to Rel-19, there has been 3 UL power configurations specified as summarized in Table 2.2-1.

| PC | Aggressor UL Power (dBm) | Release |
| --- | --- | --- |
| PC3 | 20 + 20 | Rel-15 |
| PC2 (2Tx or 3Tx) | 23 + 23 | Rel-18 or earlier |
| PC1.5 (3Tx) | 23 + 27.8 | Rel-18 |
| PC1.5 (2Tx or 3Tx) | 26 + 26 | Rel-19 |

Table 2.2-1 2UL IMD MSD UL power configurations

In Rel-19, we anticipate new PC1.5 configurations with (FDD + TDD) as (PC2 + PC2) and (PC2 + PC1.5) to be introduced with 2Tx or 3Tx. For PC2 FDD band pairing with PC2 TDD band, the FDD band UL may be implemented with 1Tx or 2Tx which could potentially result in different 2UL IMD MSD impact, similar to the UL harmonic MSD with 1Tx or 2Tx. Therefore, it is proposed RAN4 to select an example band combination for 2UL IMD MSD analysis with 1Tx and 2Tx in PC2 FDD band to observe the MSD difference to cover all possible UL configurations before the 2UL IMD MSD simplification studies.

Proposal 2: RAN4 to select an example band combination for 2UL IMD MSD analysis with 1Tx and 2Tx in PC2 FDD band to observe the MSD difference.

On the other hand, in RAN4 2UL IMD MSD for HPUE has been evaluated without taking into account PC2 PA linearity improvement over PC3 PA, meaning that the IMD power was simply scaled up with UL power and IMD order and index as shown in Table 2.2-2.

| PC | UL Power (dBm) | IMD Order | IMD Index | Δ IMD Power (dB) |
| --- | --- | --- | --- | --- |
| PC3 | 20 + 20 | IMD2/3/4/5 | Any | 0 (baseline) |
| PC2 (2Tx or 3Tx) | 23 + 23 | IMD2 | (1,1) | 6 |
|  |  | IMD3 | (2,1), (1,2) | 9 |
|  |  | IMD4 | (3,1), (2,2), (1,3) | 12 |
|  |  | IMD5 | (4,1), (3,2), (2,3), (1,4) | 15 |
| PC1.5 (3Tx) | 23 + 27.8 | IMD2 | (1,1) | 10.8 |
|  |  | IMD3 | (2,1) | 13.8 |
|  |  | IMD3 | (1,2) | 18.6 |
|  |  | IMD4 | (3,1) | 16.8 |
|  |  | IMD4 | (2,2) | 21.6 |
|  |  | IMD4 | (1,3) | 26.4 |
|  |  | IMD5 | (4,1) | 19.8 |
|  |  | IMD5 | (3,2) | 24.6 |
|  |  | IMD5 | (2,3) | 29.4 |
|  |  | IMD5 | (1,4) | 34.2 |
| PC1.5 (2Tx) | 26 + 26 | IMD2 | (1,1) | 12 |
|  |  | IMD3 | (2,1), (1,2) | 18 |
|  |  | IMD4 | (3,1), (2,2), (1,3) | 24 |
|  |  | IMD5 | (4,1), (3,2), (2,3), (1,4) | 30 |
| PC1.5 (3Tx) (2Tx in FDD) | 26 + 26 | IMD2 | (1,1) | TBC |
|  |  | IMD3 | (2,1), (1,2) | TBC |
|  |  | IMD4 | (3,1), (2,2), (1,3) | TBC |
|  |  | IMD5 | (4,1), (3,2), (2,3), (1,4) | TBC |

Table 2.2-2 2UL Δ IMD power relative to PC3 under different IMD order and index

It can be seen that for 2UL with equal power configuration, the IMD power increase only depends on the IMD order but not IMD index. However, for 2UL with unequal power configuration, the IMD power increase would depend on both IMD order and IMD index.

Observation 6: For 2UL with equal power configuration, the IMD power increase only depends on the IMD order but not IMD index. For 2UL with unequal power configuration, the IMD power increase would depend on both IMD order and IMD index.

Considering that the 2UL IMD MSD is rather complicated with unequal power configuration, in the spirit of HPUE MSD simplification, it is proposed RAN4 to only specify 2UL IMD MSD with equal UL power, that is, (20+20) for PC3, (23+23) for PC2, and (26+26) for PC1.5. In the case of (FDD + TDD) with (PC3 + PC1.5) as specified Rel-18, the MSD would be specified with (26+26) dBm while in the conformance test UE is allowed to only reach up to 23 dBm due to the PA limitation.

Proposal 2: RAN4 to only specify 2UL IMD MSD with equal UL power.

# 3	Conclusion

In this contribution, we share our views on the underlining assumptions to enable a unified HPUE MSD PCx for different single UL MSD mechanisms. For 2UL IMD MSD, we also provide our views on whether IMD order is sufficient to design LUT/set of equations or both IMD order and the IMD index should be accounted for. In addition, we propose RAN4 to only specify 2UL IMD MSD with equal UL power to simplify the HPUE MSD requirements with LUT(s).

Observation 1: For UL harmonic and cross-band interference, the mechanisms for MSD aggravation from PC3 to PC2 and PC1.5 in general are quite similar as they both are primarily caused by the inter-modulation products of the aggressor UL due to PA nonlinearity.

Observation 2: For Rx harmonic mixing, the MSD aggravation is mainly due to the aggressor UL power increase, i.e., 3dB and 6dB for PC2 and PC1.5, respectively.

Observation 3: In an ideal situation for 1Tx PC2 HPUE, the PA non-linearity is expected to be improved by 3.5dB as compared to PC3 PA as PC2 ACLR is 1dB tighter than PC3 ACLR, assuming ACLR is dominated by IM3.

Observation 4: Considering the non-ideality in practical PA design, it may be admissible to assume PC2 PA non-linearity is effectively improved by 3dB as compared to PC3 PA.

Observation 5: With the assumption that PC2 PA non-linearity is effectively improved by 3dB as compared to PC3 PA, it can be expected that the interference power for all MSD mechanisms would increase by 3dB and 6dB for PC2 and PC1.5 respectively.

Proposal 1: RAN4 to consider the following two options for single UL HPUE MSD LUT definition:

- Option 1: Define 2 LUTs as categorized below:

  - LUT1: 1Tx for UL harmonic and cross-band interference and 1Tx/2Tx for Rx harmonic mixing

  - LUT2: 2Tx for UL harmonic and cross-band interference

- Option 2: Define only one LUT for both 1Tx and 2Tx configurations where the PCx values shall accommodate both configurations with potential compromising between 1Tx and 2Tx.

Observation 6: For 2UL with equal power configuration, the IMD power increase only depends on the IMD order but not IMD index. For 2UL with unequal power configuration, the IMD power increase would depend on both IMD order and IMD index.

Proposal 2: RAN4 to only specify 2UL IMD MSD with equal UL power.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2420380 “WF on MSD rule”, Skyworks et al, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

R4-2215660 “MSD analysis for CA_n3-n78 with PC2 n3 UL”, Apple, 3GPP RAN WG4 Meeting #104bis-e, online, October 10th – 19th, 2022

R4-152188 “MSD analysis for inter-band CA with lower band Rx harmonic mixing problem”, MediaTek Inc., 3GPP TSG RAN WG4 Meeting #74bis, Rio de Janeiro, Brazil, April 20th – 24th, 2015