3GPP RAN WG4 Meeting #114bis		R4-2503197

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.1.1.1.2

Source:	Apple

Title:	On PC1.5 intra-band UL CA MPR

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the discussions were concluded with an approved “WF on TN HPUE” [2]. For PC1.5 intra-band UL CA, the preliminary MPR values in square brackets for PC1.5 intra-band contiguous UL CA with contiguous allocations were captured in the WF which will be further confirmed in this meeting. On the other hand, the MPR values for PC1.5 intra-band contiguous UL CA with non-contiguous allocations and PC1.5 intra-band non-contiguous UL CA have not yet been formerly discussed. In this contribution, we provide our analysis and proposed MPR values for PC1.5 intra-band contiguous UL CA with non-contiguous allocations. We also propose that MPR for PC1.5 intra-band NC UL CA is 3dB higher than that of PC2 intra-band NC UL CA based on our analysis on RAN4 PA configuration assumptions for MPR evaluations for PC3, PC2 and PC1.5 intra-band NC UL as presented in RAN4 #113 meeting [3].

# Discussion

## 2.1	MPR for PC1.5 intra-band contiguous UL CA with non-contiguous allocations

MPR/A-MPR evaluations usually involve a rather time-consuming simulation and/or measurement campaign. For dual Tx architecture, the evaluation process is of particular challenging as the Reverse Inter-Modulation Distortion (RIMD) between two power amplifiers (PAs) via finite antenna isolation is not straightforward to model nor is easy to emulate on measurement bench. Therefore, the MPR inaccuracy is expected to be higher than for single Tx architecture.

For PC1.5 intra-band contiguous UL CA, RAN4 has agreed to prioritize the dual Tx (26dBm+26dBm) single LO architecture for MPR/A-MPR evaluations. In principle, we can go through another laborious round of simulation and/or measurement champaign as was carried out for PC2 MPR/A-MPR with dual Tx. However, considering the aforementioned inaccuracy over this evaluation process, it may not be worth the effort to repeat the similar process as has been done for PC2 MPR evaluation with dual Tx. Instead, we could leverage the existing PC2 MPR requirements with dual Tx between single carrier and intra-band contiguous UL CA to derive the PC1.5 MPR requirements as in our view it is reasonable to assume the PC2 MPR difference between single carrier and intra-band contiguous UL CA would be similar to that of PC1.5 based on the commonality of Tx RF architecture between them.

Figure 2.1-1 illustrates the concept on how we derived the MPR values for PC1.5 intra-band contiguous UL CA with contiguous allocations [4].

Figure 2.1-1 Derivation of PC1.5 contiguous UL CA MPR

Table 2.1-1 and Table 2.1-2 summarize the derived PC1.5 MPR values for intra-band contiguous UL CA with contiguous allocations for 10dB and 20dB antenna isolation respectively.

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.5 | 6.0 | 4.0 | 9.0 |
|  | QPSK | 3.0 | 5.5 | 3.5 | 8.5 |
|  | 16QAM | 3.5 | 6.0 | 3.5 | 9.0 |
|  | 64QAM | 4.5 | 6.5 | 6.5 | 9.0 |
|  | 256QAM | 7.5 | 8.0 | 9.0 | 9.5 |
|  | QPSK | 3.0 | 6.5 | 4.0 | 9.5 |
| CP-OFDM | 16QAM | 3.5 | 6.5 | 4.0 | 9.5 |
|  | 64QAM | 4.0 | 6.0 | 5.5 | 9.0 |
|  | 256QAM | 7.5 | 7.5 | 8.0 | 9.0 |

Table 2.1-1 Derived PC1.5 MPR values for intra-band contiguous UL CA with 10dB antenna isolation

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.5 | 3.5 | 8.5 |
|  | QPSK | 2.5 | 5.0 | 3.0 | 8.0 |
|  | 16QAM | 3.0 | 5.5 | 3.0 | 8.5 |
|  | 64QAM | 4.0 | 6.0 | 6.0 | 8.5 |
|  | 256QAM | 6.5 | 7.0 | 8.0 | 8.5 |
|  | QPSK | 2.5 | 6.0 | 3.5 | 9.0 |
| CP-OFDM | 16QAM | 3.0 | 6.0 | 3.5 | 9.0 |
|  | 64QAM | 3.5 | 5.5 | 5.0 | 8.5 |
|  | 256QAM | 6.5 | 6.5 | 7.0 | 8.0 |

Table 2.1-2 Derived PC1.5 MPR values for intra-band contiguous UL CA with 20dB antenna isolation

For PC1.5 intra-band contiguous UL CA with non-contiguous allocations, the same methodology can also be applied to derive the MPR values, as conceptually illustrated in Figure 2.1-2.

Figure 2.1-2 Derivation of PC1.5 contiguous UL CA with non-contiguous allocations MPR

Table 2.1-3 summarizes the dual-Tx PC2 MPR difference between contiguous and non-contiguous allocations for intra-band contiguous UL CA (non-contiguous allocations MPR – contiguous allocations MPR) which are calculated based on the specified dual-Tx PC2 MPR values for intra-band contiguous UL CA with contiguous allocations in Table 2.1-4 and non-contiguous allocations in Table 2.1-5 respectively as recaptured from the latest UE RF specifications [5]. Notice that for bandwidth class C 256QAM inner allocations as highlighted in yellow in Table 2.1-3 below, we do not expect MPR for non-contiguous allocations to be better than contiguous allocations. Therefore, we propose the MPR difference is set as 0dB instead of -0.5dB as calculated.

| Modulation |  | MPR for bandwidth class B (dB) |  |  | MPR for bandwidth class C (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer/outer1 | outer2 | inner | outer/outer1 | outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 2.5 | N/A | 0.5 | 0.5 | N/A |
|  | QPSK | 1.0 | 2.5 |  | 0.5 | 0.5 |  |
|  | 16QAM | 0.5 | 2.5 |  | 0.5 | 0.5 |  |
|  | 64QAM | 2.0 | 2.0 |  | 0 | 0.5 |  |
|  | 256QAM | 1.0 | 1.0 |  | -0.5 -> 0 | 0 |  |
| CP-OFDM | QPSK | 1.5 | 2.5 | N/A | 0.5 | 0.5 | N/A |
|  | 16QAM | 1.0 | 2.5 |  | 0.5 | 0.5 |  |
|  | 64QAM | 2.0 | 2.5 |  | 0.5 | 0.5 |  |
|  | 256QAM | 1.5 | 1.5 |  | 1.0 | 0.5 |  |

Table 2.1-3 Dual-Tx PC2 MPR difference between contiguous and non-contiguous allocations

Table 2.1-4 Dual-Tx PC2 MPR for intra-band contiguous UL CA with contiguous allocations

Table 2.1-5 Dual-Tx PC2 MPR for intra-band contiguous UL CA with non-contiguous allocations

Further notice that the outer2 MPR requirements were only specified for non-contiguous allocations to account for IM5 falling into the general spurious emission region where the -30dBm/MHz requirement needs to be fulfilled. The PC1.5 outer2 MPR requirements can be derived from PC2 outer2 MPR requirements by assuming that the PC1.5 IM5 power level is 3dB higher than that of PC2. Therefore, it requires additional (3/5 = 0.6) dB back-off as compared to PC2 which can be rounded to 0.5dB considering that MPR has been specified with 0.5dB granularity.

Observation 1: PC1.5 outer2 MPR requirements can be derived from PC2 outer2 MPR requirements by assuming that the PC1.5 IM5 power level is 3dB higher than that of PC2.

Observation 2: PC1.5 outer2 MPR requires additional (3/5 = 0.6) dB back-off as compared to PC2 which can be rounded to 0.5dB considering that MPR has been specified with 0.5dB granularity.

Table 2.1-6 and Table 2.1-7 summarize the derived PC1.5 intra-band contiguous UL CA MPR values for non-contiguous allocations with 10dB and 20dB antenna isolation respectively. For outer2, the MPR values for 20dB antenna isolation are tightened by 0.5dB from that of 10dB antenna isolation to account for less reverse IMD impact.

| Modulation |  | MPR for bandwidth class B (dB) |  |  | MPR for bandwidth class C (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer/outer1 | outer2 | inner | outer/outer1 | outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 4.5 | 8.5 | 14.5 | 4.5 | 9.5 | 15.0 |
|  | QPSK | 4.0 | 8.0 |  | 4.0 | 9.0 |  |
|  | 16QAM | 4.0 | 8.5 |  | 4.0 | 9.5 |  |
|  | 64QAM | 6.5 | 8.5 |  | 6.5 | 9.5 |  |
|  | 256QAM | 8.5 | 9.0 |  | 9.0 | 9.5 |  |
| CP-OFDM | QPSK | 4.5 | 9.0 | 15.5 | 4.5 | 10.0 | 16.0 |
|  | 16QAM | 4.5 | 9.0 |  | 4.5 | 10.0 |  |
|  | 64QAM | 6.0 | 8.5 |  | 6.0 | 9.5 |  |
|  | 256QAM | 9.0 | 9.0 |  | 9.0 | 9.5 |  |

Table 2.1-6 Derived PC1.5 MPR values for non-contiguous allocations with 10dB antenna isolation

| Modulation |  | MPR for bandwidth class B (dB) |  |  | MPR for bandwidth class C (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer/outer1 | outer2 | inner | outer/outer1 | outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.0 | 14.0 | 4.0 | 9.0 | 14.5 |
|  | QPSK | 3.5 | 7.5 |  | 3.5 | 8.5 |  |
|  | 16QAM | 3.5 | 8.0 |  | 3.5 | 9.0 |  |
|  | 64QAM | 6.0 | 8.0 |  | 6.0 | 9.0 |  |
|  | 256QAM | 7.5 | 8.0 |  | 8.0 | 8.5 |  |
| CP-OFDM | QPSK | 4.0 | 8.5 | 15.0 | 3.5 | 9.5 | 15.5 |
|  | 16QAM | 4.0 | 8.5 |  | 4.0 | 9.5 |  |
|  | 64QAM | 5.5 | 8.0 |  | 5.5 | 9.0 |  |
|  | 256QAM | 8.0 | 8.0 |  | 8.0 | 8.5 |  |

Table 2.1-7 Derived PC1.5 MPR values for non-contiguous allocations with 20dB antenna isolation

Proposal 1: RAN4 to take the MPR values in Table 2.1-6 and Table 2.1-7 for PC1.5 for intra-band contiguous UL CA with non-contiguous allocations into consideration.

## 2.2	MPR for PC1.5 intra-band NC UL CA

For PC3 intra-band NC UL CA with dualPA-Architecture, it can be certain that the MPR requirements had been evaluated based on (PC3 + PC3) PA configuration where each PA would back off by 3 dB under PC3 maximum output power. On the other hand, for PC2 intra-band NC UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2). Since (PC3 + PC3) is the PA configuration for PC3 MPR requirements, if the same PA configuration is applied for PC2, we would expect 3dB higher MPR than PC3. However, this is not the case as observed in Table 2.2-1 where the specified MPR values are quite comparable between PC2 and PC3 [5].

| PC3 |  | PC2 |  |
| --- | --- | --- | --- |
| B (MHz) | MPR (dB) | B (MHz) | MPR (dB) |
| 0 ≤ B < 1.08 | 15.0 | 0 ≤ B < 1.44 | 15.5 |
| 1.08 ≤ B < 2.16 | 14.5 | 1.44 ≤ B < 2.88 | 15.0 |
| 2.16 ≤ B < 3.24 | 13.5 | 2.88 ≤ B < 5.76 | 14.0 |
| 3.24 ≤ B < 5.04 | 12.5 | 5.76 ≤ B < 10.8 | 12.0 |
| 5.04≤ B < 10.08 | 11.5 | 10.8 ≤ B < 23.04 | 10.5 |
| 10.08 ≤ B < 16.38 | 10.5 | 23.04 ≤ B | 9.0 |
| 16.38 ≤ B < 21.78 | 10.0 |  |  |
| 21.78 ≤ B | 9.0 |  |  |
| NOTE: B is the effective aggregated bandwidth in MHz |  |  |  |

Table 2.2-1 MPR requirements for intra-band NC UL CA with dualPA for -30dBm/MHz emission

Observation 3: For PC3 intra-band NC UL CA with dualPA-Architecture, MPR has been specified based on (PC3 + PC3) PA configuration.

Observation 4: For PC2 intra-band NC UL CA with dualPA-Architecture, the PA configuration could potentially be (PC3 + PC3), (PC3 + PC2), or (PC2 + PC2).

For (PC3 + PC2) PA configuration, though the PC2 PA is more linear, the MPR would be dominated by the emission from the PC3 PA. Using a conceptual 2-tone IM3 analysis as illustrated in Figure 2.2-1, it can be seen that the IM3 power level at (2fCC1 – fCC2) for (PC2 + PC3) PA configuration would be lower than that of (PC3 + PC3) PA configuration under the same total output power, owing to the better linearity of PC2 PA. However, the IM3 power at (2fCC2 – fCC1) would be the same between (PC2 + PC3) and (PC3 + PC3) PA configurations under the same total output power.

Figure 2.2-1 A conceptual 2-tone IM3 analysis between (PC3+PC3) and (PC2+PC3) PA configurations

Since both sides of the IM3 need to fulfill the same emission requirement, the MPR is also expected to be the same between (PC3 + PC3) and (PC2 + PC3) PA configurations when both are operating at PC3. However, if (PC2 + PC3) PA configuration is operating at PC2, the MPR would be expected to be 3dB higher than that of PC3. Therefore, to have comparable MPR between PC3 and PC2 as observed in Table 2.2-1, the only possible PA configuration for PC2 would be (PC2 + PC2) [6]. And that implies MPR for PC1.5 with the same (PC2 + PC2) PA configuration would be 3dB higher than PC2 which also means there is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Observation 3: For PC2 intra-band NC UL CA with dualPA-Architecture, MPR based on both (PC3 + PC3) and (PC2 + PC3) PA configurations would be 3dB higher than that of PC3.

Observation 4: The comparable MPR values between PC3 and PC2 as defined in current RAN4 specifications implies that the only possible PA configuration for PC2 would be (PC2 + PC2).

Observation 5: As PC1.5 intra-band NC UL CA with dualPA-Architecture would be based on the same (PC2 + PC2) PA configuration for PC2, the PC1.5 MPR is expected to be 3dB higher than that of PC2.

Observation 6: There is virtually no performance gain for PC1.5 intra-band NC UL CA as compared to its PC2 counterpart.

Proposal 2: PC1.5 intra-band NC UL CA MPR is 3dB higher than that of its PC2 counterpart.

Considering that supporting PC1.5 intra-band contiguous UL CA without UL MIMO could be a diminishing return, we propose to introduce UL Tx switching with UL MIMO or Tx diversity as an enhancement feature for intra-band NC UL CA in Rel-20 [7] to mitigate the coverage loss issue due to the considerable MPR under simultaneous non-contiguous carriers transmission.

Proposal 3: Introduce UL Tx switching for intra-band NC UL CA with UL MIMO or Tx diversity as an enhancement feature to intra-band NC UL CA without UL MIMO in Rel-20.

# 3	Conclusion

In this contribution, we provide our analysis and proposed MPR values for PC1.5 intra-band contiguous UL CA with non-contiguous allocations. We also propose that MPR for PC1.5 intra-band NC UL CA is 3dB higher than that of PC2 intra-band NC UL CA based on our analysis on RAN4 PA configuration assumptions for MPR evaluations for PC3, PC2 and PC1.5 intra-band NC UL as presented in RAN4 #113 meeting.

Proposal 1: RAN4 to take the following MPR values for PC1.5 intra-band contiguous UL CA with non-contiguous allocations into consideration.

| Modulation |  | MPR for bandwidth class B (dB) |  |  | MPR for bandwidth class C (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer/outer1 | outer2 | inner | outer/outer1 | outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 4.5 | 8.5 | 14.5 | 4.5 | 9.5 | 15.0 |
|  | QPSK | 4.0 | 8.0 |  | 4.0 | 9.0 |  |
|  | 16QAM | 4.0 | 8.5 |  | 4.0 | 9.5 |  |
|  | 64QAM | 6.5 | 8.5 |  | 6.5 | 9.5 |  |
|  | 256QAM | 8.5 | 9.0 |  | 9.0 | 9.5 |  |
| CP-OFDM | QPSK | 4.5 | 9.0 | 15.5 | 4.5 | 10.0 | 16.0 |
|  | 16QAM | 4.5 | 9.0 |  | 4.5 | 10.0 |  |
|  | 64QAM | 6.0 | 8.5 |  | 6.0 | 9.5 |  |
|  | 256QAM | 9.0 | 9.0 |  | 9.0 | 9.5 |  |

Derived PC1.5 MPR values for non-contiguous allocations with 10dB antenna isolation

| Modulation |  | MPR for bandwidth class B (dB) |  |  | MPR for bandwidth class C (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer/outer1 | outer2 | inner | outer/outer1 | outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.0 | 14.0 | 4.0 | 9.0 | 14.5 |
|  | QPSK | 3.5 | 7.5 |  | 3.5 | 8.5 |  |
|  | 16QAM | 3.5 | 8.0 |  | 3.5 | 9.0 |  |
|  | 64QAM | 6.0 | 8.0 |  | 6.0 | 9.0 |  |
|  | 256QAM | 7.5 | 8.0 |  | 8.0 | 8.5 |  |
| CP-OFDM | QPSK | 4.0 | 8.5 | 15.0 | 3.5 | 9.5 | 15.5 |
|  | 16QAM | 4.0 | 8.5 |  | 4.0 | 9.5 |  |
|  | 64QAM | 5.5 | 8.0 |  | 5.5 | 9.0 |  |
|  | 256QAM | 8.0 | 8.0 |  | 8.0 | 8.5 |  |

Derived PC1.5 MPR values for non-contiguous allocations with 20dB antenna isolation

Proposal 2: PC1.5 intra-band NC UL CA MPR is 3dB higher than that of its PC2 counterpart.

Proposal 3: Introduce UL Tx switching for intra-band NC UL CA with UL MIMO or Tx diversity as an enhancement feature to intra-band NC UL CA without UL MIMO in Rel-20.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2502863 “WF on TN HPUE”, Samsung, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, February 17th – 21st, 2025

R4-2417763 “On PC1.5 intra-band non-contiguous UL CA MPR”, Apple, 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, November 18th – 22nd, 2024

R4-2411168 “MPR for PC1.5 intra-band contiguous UL CA”, Apple, TSG RAN WG4 Meeting #112, Maastricht, Netherlands, August 19th – 23rd, 2024

3GPP TS 38.101-1 V19.0.0 (2024-12)

R4-2114948 “WF on PC2 intra-band UL NC CA and contiguous CA with 2Tx architecture”, Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #100e, online, August 16th – 27th, 2021

R4-2500324 “Motivation for UL Tx switching for intra-band NC UL CA”, Apple et al, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, February 17th – 21st, 2025