3GPP RAN WG4 Meeting #114bis		R4-2503198

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.6.1

Source:	Apple

Title:	On PC2 RedCap n1 and n3 1Rx REFSENS

WI/SI:	NR_PC2_RedCap_UE-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

A new work item on NR power class 2 (PC2) RedCap UE was approved in RAN #104 meeting [1]. The main objective of the core part is to apply the PC2 RF requirements for non-RedCap UE to RedCap UE based on 1Tx architecture with 26 dBm PA and the following additional clarifications:

- The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

- For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

- Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

In RAN #106 meeting, it was approved to specify RedCap PC2 RF requirements for FDD bands with n1 and n3 as two example bands where the revised objective was captured in the “Revised WID on NR power class 2 RedCap UE” [2]. In last RAN4 meeting, it was agreed that the existing HD-FDD REFSENS for RedCap PC3 can be applicable for RedCap PC2 HD-FDD, and the 2Rx RSD for 1Tx PC2 specified for non-Redcap UE can be applied to PC2 RedCap UE, as captured in the approved “WF on NR PC2 RedCap UE in FDD bands” [3]. The REFSENS for PC2 1Rx FD-FDD, on the other hand, need to be separately defined as they have never been specified for non-RedCap UE. Furthermore, how to present the REFSENS requirements for PC2 1Rx FD-FDD also needs to be discussed.

In this contribution, we share our PC2 1Rx REFSENS analysis for n1 and n3 and propose the corresponding requirements based on the REFSENS (1Tx (23dBm) /1Rx) + RSD representation which is the Option 1 in the approved WF.

# Discussion

The PC2 RSD requirements have already been specified for several FDD bands for non-RedCap UE, including n1 and n3 [4]. The requirements can be directly reused for RedCap UE supporting 2Rx on the same bands which was already agreed in last RAN4 meeting [3]. However, since the 1Rx FD-FDD requirement has not been specified for non-RedCap UE, the PC2 1Rx REFSENS requirements would have to be developed and exclusively specified for RedCap UE FDD bands supporting 1Rx.

Observation 1: The PC2 1Rx REFSENS requirements have to be developed and exclusively specified for RedCap UE FDD bands supporting 1Rx as they have not been specified for non-RedCap UE.

In this contribution, we share our PC2 1Rx REFSENS analysis for n1 and n3 with 1Tx and propose the corresponding requirements based on the REFSENS (1Tx (23dBm) /1Rx) + RSD representation. Our analysis was carried out based on the existing PC2 2Rx RSD requirements with 1Tx via reverse engineering, with the assumption that the Tx noise in primary Rx (P-Rx) path is effectively 6dB higher than the diversity Rx (D-Rx) path, where the Tx to Rx band isolation is assumed to be 50dB at Rx port and 46dB at the antenna port, and the main and diversity antenna isolation is assumed to be 10 dB, as conceptually illustrated in Figure 2-1.

Figure 2-1 Tx band to Rx band isolation assumption for FDD bands

For n1, since the duplex spacing is relatively wide at 190 MHz, it is expected that the PC2 Tx noise into the Rx band after the duplexer isolation is negligible for both primary Rx and diversity Rx paths as compared to Rx intrinsic noise floor. As a result, the PC2 RSD for both 2Rx and 1Rx can be reasonably assumed to be 0 dB for all channel bandwidths up to 20 MHz.

Observation 2: For n1, it is expected that the PC2 Tx noise into the Rx band after the duplexer isolation is negligible for both primary Rx and diversity Rx paths as compared to Rx intrinsic noise floor.

Observation 3: The n1 PC2 RSD for both 2Rx and 1Rx can be reasonably assumed to be 0 dB for all channel bandwidths up to 20 MHz.

For n3, the PC2 2Rx RSD with 1Tx has been specified as 0.5 dB for all channel bandwidths up to 20 MHz. Based on the above Tx band to Rx band isolation assumption, we can reverse-engineer the PC2 Tx noise into both primary Rx path and diversity Rx path and then derive the 1Rx RSD from the primary Rx path sensitivity degradation, as shown in Table 2-1.

|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| PC3 2Rx REFSENS (dBm) | -97.0 | -93.8 | -92.0 | -90.8 |
| PC2 1Tx 2Rx RSD (dB) | 0.5 | 0.5 | 0.5 | 0.5 |
| PC2 1Tx 2Rx REFSENS (dBm) | -96.5 | -93.3 | -91.5 | -90.3 |
| PC3 MRC noise (dBm) | -96.0 | -92.8 | -91.0 | -89.8 |
| PC3 primary Rx noise (dBm) | -93.5 | -89.8 | -88.0 | -86.8 |
| PC3 diversity Rx noise (dBm) | -92.4 | -89.8 | -88.0 | -86.8 |
| PC2 MRC noise (dBm) (REFSENS) | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 primary Tx noise (dBm) | -101.0 | -97.0 | -95.0 | -94.0 |
| PC2 diversity Tx noise (dBm) | -107.0 | -103.0 | -101.0 | -100.0 |
| PC2 primary Rx + Tx noise (dBm) | -92.8 | -89.0 | -87.2 | -86.0 |
| PC2 diversity Rx + Tx noise (dBm) | -92.3 | -89.6 | -87.8 | -86.6 |
| PC2 MRC noise (dBm) (derived) | -95.5 | -92.3 | -90.5 | -89.3 |
| PC2 1Rx RSD (dB) | 0.7 | 0.8 | 0.8 | 0.8 |

Table 2-1 n3 PC2 1Rx RSD analysis based on PC2 2Rx RSD reverse engineering

Based on the above observations and analysis, we propose the PC2 1Rx RSD for n1 and n3 as in Table 2-2.

| PC2 1Rx RSD (dB) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 |
| n3 | 0.7 | 0.8 | 0.8 | 0.8 |

Table 2-2 Proposed PC2 1Rx RSD for n1 and n3

Proposal: The PC2 1Rx REFSENS requirements for n1 and n3 are defined as REFSENS (1Tx (23dBm) /1Rx) + RSD with RSD as proposed in Table 2-2.

# 3	Conclusion

In this contribution, we share our PC2 1Rx REFSENS analysis for n1 and n3 and propose the corresponding requirements based on the REFSENS (1Tx (23dBm) /1Rx) + RSD representation.

Observation 1: The PC2 1Rx REFSENS requirements have to be developed and exclusively specified for RedCap UE FDD bands supporting 1Rx as they have not been specified for non-RedCap UE.

Observation 2: For n1, it is expected that the PC2 Tx noise into the Rx band after the duplexer isolation is negligible for both primary Rx and diversity Rx paths as compared to Rx intrinsic noise floor.

Observation 3: The n1 PC2 RSD for both 2Rx and 1Rx can be reasonably assumed to be 0 dB for all channel bandwidths up to 20 MHz.

Proposal: The PC2 1Rx REFSENS requirements for n1 and n3 are defined as REFSENS (1Tx (23dBm) /1Rx) + RSD with RSD as proposed in the table below.

| PC2 1Rx RSD (dB) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 |
| n3 | 0.7 | 0.8 | 0.8 | 0.8 |

# 4	References

RP-241665 “New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom, MediaTek, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17th – 20th, 2024

RP-243267 “Revised WID on NR power class 2 RedCap UE”, China Telecom, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th – 12th, 2024

R4-2502874 “WF on NR PC2 RedCap UE in FDD bands”, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, February 17th – 21st, 2025

3GPP TS 38.101-1 V19.0.0 (2024-12)