3GPP TSG-RAN WG4 Meeting #114bis                      R4-2503205

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	Discussion on BS demodulation performance requirements for MMSE-IRC

Agenda Item:	7.18.3

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for BS requirements:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

Number of interferers for 2Rx

Status in the WF [1]: - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx. - For the number of interferers for 2Rx under HetNet, further decide between the following: - Option 1: 2 interferers - Option 2: 1 interferer - For HomNet scenario, further decide among the following: - Option 1: 2 interferers - Option 2: 1 interferer - Option 3: Not to consider 2Rx requirements for HomNet scenario. - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

Based on our simulation results provided in [2], the lowest performance gain of MMSE-IRC over MMSE for 1T2R is 2.50dB with 15kHz SCS 5MHz and 2.04dB with 30kHz SCS 10MHz, which is still large enough to verify the correct implementation of MMSE-IRC.

Observation 1: The lowest performance gain of MMSE-IRC over MMSE for 1T2R is 2.50dB with 15kHz SCS 5MHz and 2.04dB with 30kHz SCS 10MHz.

Regarding the number of interferers for 2Rx, we do not observe large performance difference when using 1 or 2 interferers. Therefore, we prefer to use same test configuration for 2,4 and 8Rx requirements.

Proposal 1: Define requirements for 2Rx MMSE-IRC receiver with 2 interferers for both HomNet and HetNet.

Propagation condition

Status in the WF [1]: Select between the following options: - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet - Option 2: TDLC300-100 for all cases - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2) Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

First of all, we do not think it is practical to use the same propagation condition for different test scenarios. Therefore, we prefer only consider option 1 or option 3.

Based on our simulation results, we do not observe large performance difference when using TDLA30-10, TDLB100-400 or TDLC300-100. If companies have concern on the high Doppler for TDLB100-400, we are fine to use option 3 as a compromise.

Proposal 2: Different propagation condition should be defined for different test scenarios. Support ‘TDLA30-10 for HetNet and TDLB100-400 for HomNet’. If companies have concern on the high Doppler for TDLB100-400, fine to use ‘TDLA30-10 for HetNet and TDLC300-100 for HomNet’ as a compromise.

Manufacturer declaration on MMSE-IRC receiver

Status in the WF [1]: - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.xxx; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x. - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC

During the RAN4 Rel-19 package discussion phase, we heard from infra vendors that MMSE-IRC has already been implemented in their existing products. Therefore, we think it can be reasonable not to introduce new manufactory declaration for BS MMSE-IRC receiver, otherwise, nearly all BS need to declare such feature in the test.

At the same time, it is not our purpose to limit BS implementation. If infra vendors have strong view to introduce such manufacturer declaration, we are also fine with that.

Proposal 3: There is no need to introduce new manufactory declaration for BS MMSE-IRC receiver.

Applicability

Status in the WF [1]: - Candidate options on test requirement applicability for different BS classes and test scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS - Other options are not precluded. - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed - Proposal 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

Regarding the test requirement applicability for different BS classes and test scenarios, we propose to follow the LTE approach and other than that, we do not see the reason to introduce additional applicabilities.

Observation 2: The LTE MMSE-IRC requirements has specified that ‘HomNet requirements are not applicable for Local Area BS and Home BS’. Besides, no additional applicability is introduced.

Given that NR spec has only specified Wide Area BS, Medium Range BS, or Local Area BS, i.e., no Home BS, the following applicability is proposed:

- HomNet requirements are not applicable for Local Area BS

- No additional applicability for different BS classes and test scenarios is proposed.

- Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Proposal 4: HomNet requirements are not applicable for Local Area BS. No additional applicability for different BS classes and test scenarios is proposed. Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Connection diagram

In LTE, new connection diagrams with UL ICI are specified for 1&2 interferers for 2Rx, 2 interferers for 4 Rx and 2 interferers and 8 Rx, i.e., Figure I.3-7a to Figure I.3-7d in TS36.141.

Observation 3: New connection diagrams with UL ICI are specified in LTE for 1&2 interferers for 2Rx, 2 interferers for 4 Rx and 2 interferers and 8 Rx, i.e., Figure I.3-7a to Figure I.3-7d in TS36.141.

The new MMSE-IRC requirements in Rel-19 are the first requirements introducing inter-cell interference at the BS side. To align with LTE, we believe new connection diagrams are also needed for the new MMSE-IRC requirements for NR and the following should be covered:

- BS type 1-C performance requirements in D.5 (TS38.141-1)

- BS type 1-H performance requirements in D.6 (TS38.141-1)

- Set up for OTA testing in E.3 (TS38.141-2)

Proposal 5: New connection diagrams are needed for the new MMSE-IRC requirements for NR and the following should be covered:

- BS type 1-C performance requirements in D.5 (TS38.141-1)

- BS type 1-H performance requirements in D.6 (TS38.141-1)

- Set up for OTA testing in E.3 (TS38.141-2)

Conclusion

Observation 1: The lowest performance gain of MMSE-IRC over MMSE for 1T2R is 2.50dB with 15kHz SCS 5MHz and 2.04dB with 30kHz SCS 10MHz.

Proposal 1: Define requirements for 2Rx MMSE-IRC receiver with 2 interferers for both HomNet and HetNet.

Proposal 2: Different propagation condition should be defined for different test scenarios. Support ‘TDLA30-10 for HetNet and TDLB100-400 for HomNet’. If companies have concern on the high Doppler for TDLB100-400, fine to use ‘TDLA30-10 for HetNet and TDLC300-100 for HomNet’ as a compromise.

Proposal 3: There is no need to introduce new manufactory declaration for BS MMSE-IRC receiver.

Observation 2: The LTE MMSE-IRC requirements has specified that ‘HomNet requirements are not applicable for Local Area BS and Home BS’. Besides, no additional applicability is introduced.

Proposal 4: HomNet requirements are not applicable for Local Area BS. No additional applicability for different BS classes and test scenarios is proposed. Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Observation 3: New connection diagrams with UL ICI are specified for 1&2 interferers for 2Rx, 2 interferers for 4 Rx and 2 interferers and 8 Rx, i.e., Figure I.3-7a to Figure I.3-7d in TS36.141.

Proposal 5: New connection diagrams are needed for the new MMSE-IRC requirements for NR and the following should be covered:

- BS type 1-C performance requirements in D.5 (TS38.141-1)

- BS type 1-H performance requirements in D.6 (TS38.141-1)

- Set up for OTA testing in E.3 (TS38.141-2)

Reference

R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom. RAN4#114, February, 2025

R4-2503204, On BS demodulation performance requirements for MMSE-IRC: Simulation Results, China Telecom, RAN4#114bis, Apr 2025