3GPP TSG-RAN WG4 Meeting #114bis                      R4-2503209

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for NR channel bandwidth less than 5MHz

Agenda Item:	7.3.4

Document for:	discussion

Introduction

RAN4#114bis is the first meeting for the UE demodulation performance part for WI: NR channel BW less than 5MHz for FR1 Phase 2. The Core part has been completed in RAN#107, which has specified RF requirements for CA/DC with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band [1]:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz or 5MHz in band n100 and 5MHz or 10MHz in band n101 NOTE: Consideration of 5MHz channel bandwidth in the above “3MHz or 5MHz” is specific to band n100 and does not apply to other bands. - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In this contribution, our views on the UE demodulation performance part are given.

Proposed work plan

According to the approved WID, the Perf part of this WI targets June 2025, which means RAN4 Demod part only has 2 meetings. Considering the above, the following work plan is proposed:

- RAN4#114bis:

  - Agree on the test scope, test parameters and test applicability.

- RAN4#115:

  - Collection of simulation results.

  - Draft CR endorsed, and big CR agreed.

Proposal 1: As the WI Perf. Part targets June 2025, the following work plan is proposed:

- RAN4#114bis:

  - Agree on the test scope, test parameters and test applicability.

- RAN4#115:

  - Collection of simulation results.

  - Draft CR endorsed, and big CR agreed.

Discussion

Test coverage

In Rel-18, RAN4 has specified requirements for 3MHz CHBW which limited to single carrier scenario. Thus, in its performance part, corresponding PDCCH demodulation requirements for 3MHz channel bandwidth is introduced for both 2Rx and 4Rx FDD 15kHz SCS.

Observation 1: In Rel-18, RAN4 has defined PDCCH demodulation requirements for 3MHz channel bandwidth for both 2Rx and 4Rx FDD 15kHz SCS.

In Rel-19, the Core part of this WI has introduced RF requirements for FDD + FDD CA with 3MHz for one of the CC, which will impact PDSCH demodulation requirements for CA, and the test scope should be aligned with R18, i.e., cover both 2Rx and 4Rx with FDD 15kHz SCS.

In addition, the SDR requirements may also be impacted as 3MHz CC may be additionally selected for highest data rate.

Proposal 2: Define 3MHz channel bandwidth single carrier requirements in PDSCH CA demodulation requirements for both 2Rx and 4Rx with FDD 15kHz SCS, i.e., add new rows for 3MHz in Table 5.2A.2.1-1 and Table 5.2A.3.1-1 in TS38.101-4.

Proposal 3: RAN4 to discuss whether there are SDR requirement impact for UE supporting 3MHz channel bandwidth in CA.

Test parameters

We think the test configurations (simulation assumptions) for new 3MHz requirements for CA should be aligned with the existing parameters for other channel bandwidths. Thus, we propose the following:

- Rank: 2

- Modulation format and code rate: 16QAM, 0.48 (MCS 13)

- Propagation condition: TDLA30-10

- Correlation matrix and antenna configuration: 2x2 and 2x4, ULA Low

- For the other parameters: Follow the Common test parameters for CA in Table 5.2A-1 in TS38.101-4

- Test metric: SNR (dB) at 70% max throughput

For the number of PRBs for DL transmission, it is proposed to align with the configuration in Table 5.3.2-1 of TS38.101-1, i.e., 15PRBs for 15kHz SCS:

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 3 MHz; …. NRB; …. 15; 15; …. 30; N/A; …. 60; N/A; ….

Proposal 4: The test configurations for new 3MHz requirements for CA should be aligned with the existing parameters for other channel bandwidths:

- Rank: 2

- Modulation format and code rate: 16QAM, 0.48 (MCS 13)

- Propagation condition: TDLA30-10

- Correlation matrix and antenna configuration: 2x2 and 2x4, ULA Low

- For the other parameters: Follow the Common test parameters for CA in Table 5.2A-1 in TS38.101-4

- Test metric: SNR (dB) at 70% max throughput

Proposal 5: 15 PRBs for DL transmission should be used for 3MHz requirements for FDD 15kHz SCS.

Receiver assumption

MMSE-IRC is mandatory for 2Rx and 4Rx UEs, so we propose to use MMSE-IRC as the receiver assumption.

Proposal 6: Use MMSE-IRC as the receiver assumption for 3MHz requirements.

Test applicability

In this Perf part, a new CC with 3MHz channel bandwidth is introduced. We think the existing test applicability rule in 5.1.1.7 in TS38.101-4 can be reused.

Proposal 7: The existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused.

Conclusion

Proposal 1: As the WI Perf. Part targets June 2025, the following work plan is proposed:

- RAN4#114bis:

  - Agree on the test scope, test parameters and test applicability.

- RAN4#115:

  - Collection of simulation results.

  - Draft CR endorsed, and big CR agreed.

Observation 1: In Rel-18, RAN4 has defined PDCCH demodulation requirements for 3MHz channel bandwidth for both 2Rx and 4Rx FDD 15kHz SCS.

Proposal 2: Define 3MHz channel bandwidth single carrier requirements in PDSCH CA demodulation requirements for both 2Rx and 4Rx with FDD 15kHz SCS, i.e., add new rows for 3MHz in Table 5.2A.2.1-1 and Table 5.2A.3.1-1 in TS38.101-4.

Proposal 3: RAN4 to discuss whether there are SDR requirement impact for UE supporting 3MHz channel bandwidth in CA.

Proposal 4: The test configurations for new 3MHz requirements for CA should be aligned with the existing parameters for other channel bandwidths:

- Rank: 2

- Modulation format and code rate: 16QAM, 0.48 (MCS 13)

- Propagation condition: TDLA30-10

- Correlation matrix and antenna configuration: 2x2 and 2x4, ULA Low

- For the other parameters: Follow the Common test parameters for CA in Table 5.2A-1 in TS38.101-4

- Test metric: SNR (dB) at 70% max throughput

Proposal 5: 15 PRBs for DL transmission should be used for 3MHz requirements for FDD 15kHz SCS.

Proposal 6: Use MMSE-IRC as the receiver assumption for 3MHz requirements.

Proposal 7: The existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused.

Reference

RP-250781, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation RAN#107, March 2025