3GPP TSG-RAN4 Meeting #114bis 	R4-2503216

# Wuhan, China, 7th -11th April, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT general aspects

Agenda Item:	7.24.2.1

Document for:	Approval

- Introduction

The WI on solutions for Ambient IoT in NR was approved at RAN#106 [1] and revised at RAN#107[2]. Last meeting, a WF was agreed [3]. In this contribution, we further discuss the open issues and give our proposals on system parameters and other general aspects of A-IoT.

- Discussion

  - R2D Channel bandwidth

Last meeting, it was agreed to define R2D channel bandwidth as the frequency resources used for transmitting R2D, and guard band. Values of R2D bandwidth is FFS.

Copied from [3]

Issue 2-1-3: R2D channel bandwidth definition

Agreement:

- Define R2D channel bandwidth as:

  - Channel bandwidth, BCH,R2D from a reader perspective: The frequency resources used for transmitting R2D, and guard band.

Issue 2-1-4: Values of R2D bandwidth

Agreement:

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

The following agreements were reached during RAN1#120:

Copied from R1-2501547 [4]

Agreement

For R2D transmission, only subcarrier spacing of 15 kHz is supported (as per TR 38.769), and no other SCS are further considered.

Agreement

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Agreement

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

Agreement

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform generation of OOK-4 modulation:

- Certain specification of DFT-s-OFDM waveform generation for OOK-4 modulation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

- This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

For reference,  the M values and the associated minimum Btx,R2D value are copied below:

Copied from TR38.769[5]

Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value.

Note:	Depending on further study, the maximum value of M may be less than 32.

Note:	The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint.

Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

RAN1 specifies minimum Btx ,R2D # of PRBs to guarantee a minimum performance. However, it does not define exact upper limits for Btx,R2D. RAN1 agreement is that whether to specify the maximum Btx,R2D # of PRBs is up to RAN4.

From a device perspective, RF-ED receiver can receive all signals falling into its receiver bandwidth. Wider R2D transmission bandwidths offer benefits including higher inventory rates and improved time-domain signal quality. Therefore, Btx,R2D upper limits could be implementation-specific to let operators balance performance against spectrum availability.

However, RAN4 considerations necessitate maximum bandwidth definitions because:

1) A-IoT BS RF requirements (ACLR, OBUE, etc.) scale with bandwidth

2) Device design - particularly interference rejection - requires known channel bandwidth limits

3)A-IoT device strict power consumption constraints

Given these considerations, defining maximum Btx,R2D is necessary..

Proposal 1：RAN4 specify maximum Btx,R2D # of PRBs.

The maximum R2D channel bandwidth need to balance performance and efficiency. Given the valuable spectrum resources and strict power consumption constraints for A-IoT devices, it is recommend to set the maximum Btx,R2D to 4 PRB. The key reasons include:

1) Practically, the time-domain signal quality for 4 PRB transmission bandwidth is good enough.

2) A wider Btx,R2D requires a wider device LPF bandwidth, leading to poor anti-interference performance.

3) A wider Btx,R2D demands a higher clock rate, leading to increased chip power consumption.

Proposal 2：Set the Maximum Btx,R2D as 4 PRBs.

Last meeting, it was agreed to further discuss the R2D channel bandwidth considering additional guard band.

On the guard band value, the spectrum of baseband R2D signals before and after FIR filters are simulated, and results are shown in the Figures 1, comparing the unfiltered (blue) and filtered (orange).

(a)                                   (b)

Figure 1: PSD of A-IoT R2D signals before and after filter:(a) 1 PRB transmission BW;(b) 4PRB transmission BW

Figure 1 shows two R2D channel bandwidth configurations:
(a) 200 kHz channel BW with 1 PRB transmission BW
(b) 800 kHz channel BW with 4 PRB transmission BW

These results don't account for RF impairments like PA degradation. Simulation results are presented in the Table 1:

Table 1 ACLR simulation result for 1PRB and 4PRB transmission BW

| Transmission BW(kHz) |  | 180kHz | 720kHz |
| --- | --- | --- | --- |
| CBW(kHz) |  | 200kHz | 800kHz |
| ACLR1 （without frequency offset） | Before filter | 22dB | 28dB |
|  | After filter | 31dB | 50dB |

Proposal 3：With 90% spectrum utilization, define R2D CBW as 200kHz，400kHz，600kHz and 800kHz；

For 200kHz R2D CBW, it includes 180kHz transmission BW and 10kHz guard band on each side.

For 400kHz R2D CBW, it includes 360kHz transmission BW and 20kHz guard band on each side.

For 600kHz R2D CBW, it includes 540kHz transmission BW and 30kHz guard band on each side.

For 800kHz R2D CBW, it includes 720kHz transmission BW and 40kHz guard band on each side.

  - D2R Channel bandwidth

Copied from [2]

Issue 2-1-6: D2R bandwidth

Agreement:

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

RAN1 progress in RAN1#120 are as follows：

Copied from R1-2501547 [4]

Agreement

For D2R, only 2SB (Double sideband) modulation is supported.

Agreement

For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length):

- the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation

- the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

- Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

RAN4 has identified that [1SB or] 2SB, SFO, FDMA and filter performance may have an impact on D2R channel BW.

The D2R transmission bandwidth is 1/ (R*chip length). R*chip length is still under discussion in RAN1 this meeting.  Based on RAN1’s agreement, only 2SB modulation is supported for D2R.

We use one of proposed options of D2R chip length values to analyze the impact of D2R chip length on channel bandwidth.

Table 2: Example D2R chip length for each (R, Btx,D2R)

|  | 15 kHz | 30 kHz | 60 kHz | 180 kHz | 360 kHz | 720 kHz | 1440 kHz | 2880 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R = 1 | 133.33 μs | 66.67 μs | 33.33 μs | 11.11 μs | 5.56 μs | 2.78 μs | 1.39 μs | 0.69 μs |
| R = 4 | 33.33 μs | 16.67 μs | 8.33 μs | 2.78 μs | 1.39 μs | 0.69 μs | \ | \ |
| R = 8 | 16.67 μs | 8.33 μs | 4.17 μs | 1.39 μs | 0.69 μs | \ | \ | \ |
| R = 16 | 8.33 μs | 4.17 μs | 2.08 μs | 0.69 μs | \ | \ | \ | \ |
| R = 32 | 4.17 μs | 2.08 μs | 1.04 μs | \ | \ | \ | \ | \ |
| R = 64 | 2.08 μs | 1.04 μs | \ | \ | \ | \ | \ | \ |
| R = 96 | 1.39 μs | 0.69 μs | \ | \ | \ | \ | \ | \ |
| R = 128 | 1.04 μs | \ | \ | \ | \ | \ | \ | \ |

Device support FMDA frequency shift as Figure 2.

The frequency shift value is defined as the offset between the 2SB carrier's center frequency and the shifted single-side carrier's center frequency. For R=1 (where no shift occurs), this represents a zero-shift special case.

For R ≥ 1: Each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of frequency shift in Hz is R/Tb = 1/(2 × chip length).

The impact of SFO (sampling frequency offset) can be assumed as the extension or the shortening of the chip length. The SFO of 105 PPM is assumed for Device 1.

Without SFO effect, the frequency shift is Transmission BW*R/4. If 105 PPM SFO accounts, the frequency shift changes to be Transmission BW*R*1.1/4. The 1.1 factor accounts for 10% SFO effect.

SFO additionally scales the frequency shift value by a factor of 1.1.

Figure 2: Diagram of frequency shift

The corresponding FDMA baseline frequency shift without SFO is calculated as the follows:

Table 3. FDMA baseline frequency shift without SFO for each (R, Btx,D2R)

|  | 15 kHz | 30 kHz | 60 kHz | 180 kHz | 360 kHz | 720 kHz | 1440 kHz | 2880 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R = 1 | 3.75kHz | 7.5kHz | 15kHz | 45kHz | 90kHz | 180kHz | 360kHz | 720kHz |
| R = 4 | 15kHz | 30kHz | 60kHz | 180kHz | 360kHz | 720kHz | \ | \ |
| R = 8 | 30kHz | 60kHz | 120kHz | 360kHz | 720kHz | \ | \ | \ |
| R = 16 | 60kHz | 120kHz | 240kHz | 720kHz | \ | \ | \ | \ |
| R = 32 | 120kHz | 240kHz | 480kHz | \ | \ | \ | \ | \ |
| R = 64 | 240kHz | 480kHz | \ | \ | \ | \ | \ | \ |
| R = 96 | 360kHz | 720kHz | \ | \ | \ | \ | \ | \ |
| R = 128 | 480kHz | \ | \ | \ | \ | \ | \ | \ |

If the conventional channel bandwidth concept is used, CBW can be a summary of frequency shift*2, 10% SFO effect and 90% filter spectrum utility (10% guardband).

CBW calculations:

D2R Channel BW = (Frequency shift * 2*1.1 + (2SB transmission bandwidth *1.1)/2) /0.9

The 1.1 factor accounts for 10% SFO effect.

The 0.9 divisor presents the 90% filter spectrum utility (10% guard).

Observation 1: Consider FMDA and SFO (assumed as 10^5ppm) effect, D2R CBW can be calculated as follows:

D2R Channel BW = (Frequency shift * 2*1.1 + (2SB transmission bandwidth *1.1)/2) /0.9

The 1.1 factor accounts for 10% SFO effect.

The 0.9 divisor presents the 90% filter spectrum utility (10% guardband).

The possible CBW values are calculated in Table 4.

Table 4 D2R CBW Calculation

| R Value | Transmission BW（kHz） | Freq shift (kHz) | Calculated CBW (kHz) |
| --- | --- | --- | --- |
| 1 | 15 | 3.75 | 18 |
| 4 | 15 | 15 | 46 |
| 8 | 15 | 30 | 83 |
| 16 | 15 | 60 | 156 |
| 32 | 15 | 120 | 303 |
| 64 | 15 | 240 | 596 |
| 96 | 15 | 360 | 889 |
| 128 | 15 | 480 | 1183 |
| 1 | 30 | 7.5 | 37 |
| 4 | 30 | 30 | 92 |
| 8 | 30 | 60 | 165 |
| 16 | 30 | 120 | 312 |
| 32 | 30 | 240 | 605 |
| 64 | 30 | 480 | 1192 |
| 96 | 30 | 720 | 1778 |
| 1 | 60 | 15 | 73 |
| 4 | 60 | 60 | 183 |
| 8 | 60 | 120 | 330 |
| 16 | 60 | 240 | 623 |
| 32 | 60 | 480 | 1210 |
| 1 | 180 | 45 | 220 |
| 4 | 180 | 180 | 550 |
| 8 | 180 | 360 | 990 |
| 16 | 180 | 720 | 1870 |
| 1 | 360 | 90 | 440 |
| 4 | 360 | 360 | 1100 |
| 8 | 360 | 720 | 1980 |
| 1 | 720 | 180 | 880 |
| 4 | 720 | 720 | 2200 |
| 1 | 1440 | 360 | 1760 |
| 1 | 2880 | 720 | 3520 |

Observation 2: The potential channel BW become too many when accounting for combinations of transmission bandwidth, chip length, and FDMA frequency shift.

With so many potential CBWs, network configuration will be complicated, and designing separate filters for each CBW would be inefficient. To streamline network configuration and BS filter design, it may need to limit the kinds of D2R channel BW.

Proposal 4: Minimize the kinds of D2R channel bandwidth.

  - R2D Channel raster

In order to configure a narrower UE channel BW inside a wider gNB channel BW, 10 kHz Channel raster enhancement is introduced in R18. For R2D channel raster, 10 kHz can be reused.

Filters typically are symmetric designs, as asymmetry degrades performance. The carrier center frequency need to align with the transmission bandwidth's center. This can be achieved by applying a raster offset of either -7.5 kHz or +2.5 kHz.

Figure 3 Diagram of channel raster offset

Proposal 5: R2D Channel raster from A-IoT BS perspective is 10 kHz.

Proposal 6: R2D channel raster offset can be -7.5kHz or +2.5kHz.

  - Foffset

As described in TS36.104, for NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset. A-IoT with one PRB transmission bandwidth is with similar narrow bandwidth, it can be applied to A-IoT.

In section 2.1, the ACLR for OOK-4 OFDM signal (both pre- and post-filter, excluding PA degradation) is significantly worse than NB-IoT. This necessitates frequency offset for A-IoT operation.

Figure 4 Diagram of A-IoT ACLR Foffset

Proposal 7: Apply the Foffset for Ambient IoT as Table 6:

Table 6: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for R2D CBW 200/400/600/800kHz | 100 kHz |
| Note：Foffset is start from A-IoT Channel Bandwidth edge |  |

- Summary

In this contribution, we discuss key open issues regarding A-IoT general aspects and present the following observations and proposals:

Proposal 1：RAN4 specify maximum Btx,R2D # of PRBs.

Proposal 2：Set the Maximum Btx,R2D as 4 PRBs.

Proposal 3：With 90% spectrum utilization, define R2D CBW as 200kHz，400kHz，600kHz and 800kHz；

For 200kHz R2D CBW, it includes 180kHz transmission BW and 10kHz guard band on each side.

For 400kHz R2D CBW, it includes 360kHz transmission BW and 20kHz guard band on each side.

For 600kHz R2D CBW, it includes 540kHz transmission BW and 30kHz guard band on each side.

For 800kHz R2D CBW, it includes 720kHz transmission BW and 40kHz guard band on each side.

Observation 1: Consider FMDA and SFO (assumed as 10^5ppm) effect, D2R CBW can be calculated as follows:

D2R Channel BW = (Frequency shift * 2*1.1 + (2SB transmission bandwidth *1.1)/2) /0.9

The 1.1 factor accounts for 10% SFO effect.

The 0.9 divisor presents the 90% filter spectrum utility (10% guardband).

Observation 2: The potential channel BW become too many when accounting for combinations of transmission bandwidth, chip length, and FDMA frequency shift.

Proposal 4: Minimize the kinds of D2R channel bandwidth.

Proposal 5: R2D Channel raster from A-IoT BS perspective is 10 kHz.

Proposal 6: R2D channel raster offset can be -7.5kHz or +2.5kHz.

Proposal 7: Apply the Foffset for Ambient IoT as Table 6:

Table 6: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for R2D CBW 200/400/600/800kHz | 100 kHz |
| Note：Foffset is start from A-IoT Channel Bandwidth edge |  |

- References

[1]RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.，RAN#107 Mar.,2025

[3] R4-2502858, WF on requirements for system parameters and A-IOT device, CMCC, RAN4#114, Feb. 2025

[4] R1-2501547, Session notes for 9.4 Solutions for Ambient IoT in NR, Ad-Hoc Chair (Huawei), RAN1 #120, Feb. 2025

[5] TR38.769, Study on solutions for ambient IoT (Internet of Things)

[6]R1-2407669, On general aspects of physical layer design for Ambient IoT, Huawei, HiSilicon, RAN1#118bis, Oct.,2024