3GPP TSG-RAN4 Meeting #114bis 	R4-2503217

# Wuhan, China, 7th -11th April, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT BS RF requirements

Agenda Item:	7.24.2.1

Document for:	Approval

- Introduction

The WI on solutions for Ambient IoT in NR was approved at RAN#106 [1] and revised at RAN#107[2]. Last meeting, a WF was agreed [3]. In this contribution, we further discuss the remaining RF requirements for Ambient-IoT BS.

- Ambient IoT BS TX requirements

  - Base station output power

The indoor Ambient IoT BS is type 1-C Micro NR BS characterised by requirements derived from Micro Cell scenarios with a BS to device minimum coupling loss equals to 53 dB.

During the last meeting, it was agreed that A-IoT BS output power, Prated,c,AC≤ [38]dBm.

Some companies raised concerns about potential coexistence issues, as 38dBm is higher than the 33dBm power assumption used in SI phase. Simulations typically don’t require the use of maximum output power, and the 33 dBm assumption from the SI phase remains valid.

38 dBm A-IoT BS output power with ACLR 20/30/40/50dB for Scenario 2-1c are simulated and results are showed in Table 1.

Figure 1 Diagram for Scenario 2-1 (D1T1-B- NR UE only outdoor, R2D: DL, CW2D/ D2R: UL)

Table 1 case c for scenario 2-1 simulation results for A-IoT BS Pout 38dBm

| A-IoT BS ACLR（dB） | 20 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- |
| (Baseline throughput- throughput with interference)/ Baseline throughput @5% CDF | 1.4% | Negligible degradation |  |  |
| (Baseline throughput- throughput with interference)/ Baseline throughput @50% CDF | 1.0% |  |  |  |

It can be seen that with A-IoT BS Pout 38dBm, negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed.

Observation 1：With A-IoT BS Pout 38dBm and ACLR 20/30/40/50dB, negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed.

Therefore, Output power up to 38dBm presents no coexistence issues, offering deployment flexibility to adjust power levels according to scenario requirements.

Proposal 1: Reuse the output power of Medium Range BS for indoor A-IoT BS, Prated,c,AC≤ 38dBm

  - Output power dynamic

Output power dynamic requirement usually includes RE power control dynamic range requirement and total power dynamic range requirement.

Copied from R4-2502859[3]

Issue 1-2-1: RE power control dynamic range

- Agreement:

  - Check whether RAN1 supports RE granularity operation for R2D transmission or not.

Issue 1-2-2: Total power dynamic range

- Agreement:

  - Check whether RAN1 supports PRB granularity operation for R2D transmission or not

For NR, the RE power control dynamic range is defined as the difference between the power of an RE and the average RE power for a BS at maximum output power for a specified reference condition.

For A-IoT, RE power control dynamic range is not needed because no RE level operation.

Proposal 2: RE power control dynamic range is not needed for A-IoT BS since there is no RE level operations in an A-IoT R2D carrier.

For NR, total power dynamic range is defined as the ratio of the maximum transmit power (when the number of RBs is full) to the minimum transmit power (when the number of RBs is one).

For A-IoT, based on SI conclusion, Minimum Btx,R2D # of PRBs may be range from 1 to 4.

Unlike NR, in A-IoT, even if the number of PRBs exceeds 1, the A-IoT R2D carrier treats two or more PRBs as a single unit without performing operations at the PRB level. As a result, there's no need to define a total power dynamic range for A-IoT base stations.

Proposal 3: Total power dynamic range is not necessary for A-IoT BS since there are no PRB level operations in an A-IoT R2D carrier.

  - Transmit ON/OFF power

2.3.1 Transmitter OFF power

It was agreed that define Transmitter OFF power for A-IoT BS and values FFS last meeting. [3]

The NR BS type 1-C NR requirements for transmitter OFF power can be reused.

Proposal 4: For A-IoT BS, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

2.3.2 Transmitter transient period

The transmitter transient period is the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. As described in 2.3.1, A-IoT BS employs a TDD similar on-and-off switching mechanism. The NR BS type 1-C NR requirements for Transmitter transient period can be reused.

Proposal 5: A-IoT BS reuse the NR BS type 1-C NR requirements for Transmitter transient period, as shown in Table 3.

Table 2: Minimum requirement for the transmitter transient period for A-IoT BS

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

  - Modulation quality

Copied from R4-2502859[3]

### Issue 1-4-2: Modulation quality

- Agreement:

  - FFS whether to define requirements such as modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

Consider effect to device modulation performance, it is need to define modulation depth, RF envelope ripple, RF envelop rise/fall time and RF Pulse width as in the standard of EPC RFID [5].

Copied from RFID Standard [5]:

Proposal 6: Define RF envelop requirements, including modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

However, A-IoT is with some differences with RFID in modulation and encoding. Modulation for A-IoT is OOK-4, for RFID is ASK. For encoding, According to WID [1], A-IoT R2D transmission uses only the Manchester line code as specified in TR 38.769. the encoding method will affect the RF pulse width.

Table 3 R2D difference between A-IoT and RFID

| R2D | AIoT | RFID |
| --- | --- | --- |
| Waveform | OFDM-based | Non-OFDM |
| modulation | OOK-4 | ASK |
| encoding | Manchester | PIE |

A larger R2D transmission BW brings better time-domain signal quality.When define modulation quality, the R2D transmission BW will be the worst case.

Figure 3 shows a simulation baseband waveform of R2D.The red is CP and the blue is R2D data.

Figure 2 Simulation time domain R2D waveform

Based on our baseband signal simulation, it can be calculated the following values in Table 4.

Table 4: Simulation results of A-IoT BS RF envelope parameters

| Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
| RF Envelope Ripple | Mh = Ml | 0 |  | 0.10(A–B) | V/m orA/m |
| RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
| RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
| RF Pulsewidth | PW |  | 0.5Tari |  | µs |

Additional considerations such as CP effects, varying chip lengths, digital filtering, and RF impairments may need to be included.

For CP effects, the following agreements were reached during RAN1#120:

Copied from R1-2501547 [4]

Agreement

For CP handling which retains subcarrier orthogonality, the candidate methods are clarified as follows:

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Detail design of Alt M2-1-1 or Alt M2-1-2 from the followings:

    - Alt M2-1-1

      - Candidate 1:

        - For M less than 6: Use Alt M1-1

        - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-2

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M +/- 1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.

  - FFS: applicable M values with or without combination of Method Type 1

For CP handling which does not retain subcarrier orthogonality, the candidate methods are clarified as follows:

- For Method Type 2 Alt M2-2 (as per TR 38.769)

  - FFS detail design of Alt M2-2 from the following:

    - Candidate 1:

      - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot.

For further down-selection among candidate methods which retains subcarrier orthogonality, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis:

- Method Type 1

- Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

- Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

- Note: The evaluation for CP handling at least includes a M value from {2,4,6,8}, a M value from {12, 16}, and a M value from {24, 32}

- Note: In this evaluation, the transmit power of the ON chips remains constant

RAN1 is currently addressing CP handling methods, and it’s anticipated that the discontinuities caused by CP can be largely disregarded.

Simulations show that the time-domain waveform remains nearly unchanged before and after filter, suggesting that the filter’s effect is minimal.

Observation 2：RAN1 is currently addressing CP handling methods, and it’s anticipated that the discontinuities caused by CP can be largely disregarded.

Observation 3：The time-domain waveform remains largely unchanged before and after filtering, indicating that the filter’s impact is negligible.

Proposal 7:  Consider RF impairments, the RF envelope requirements can be defined as Table 5.

Table 5: A-IoT BS RF envelope parameters

| Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
| RF Envelope Ripple | Mh = Ml | 0 |  | 0.15(A–B) | V/m orA/m |
| RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
| RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
| RF Pulsewidth | PW |  | 0.5Tari | 0.65Tari | µs |

  - Unwanted emissions

In SI phase, it is suggested that unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed.

    - Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also Recommendation ITU-R SM.328 [3].

The value of /2 shall be taken as 0.5%.

The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

Proposal 8:  The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

2.5.2	Adjacent Channel Leakage Power Ratio

Copied from R4-2502859

### Issue 1-5-2: ACLR

- Agreement:

  - Further discuss ACLR requirements considering following aspects:

    - No legacy NR UE indoor assumption

    - Whether 100KHz offset is needed or not

In SI it is assumed that the ACLR of A-IoT BS reuses the same values as legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB).

For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, Table 1 simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

Observation 4: For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

The spectrum of baseband R2D signals before and after FIR filters are simulated, and results are shown in the Figures 1, comparing the unfiltered (blue) and filtered (orange).

(a)                                    (b)

Figure 3: PSD of A-IoT R2D before and after filter,(a) 1 PRB transmission BW;(b) 4PRB transmission BW

Figure 3 shows two R2D channel bandwidth configurations:
(a) 200 kHz channel BW with 1 PRB transmission BW
(b) 800 kHz channel BW with 4 PRB transmission BW

These simulations exclude PA impairments. Simulation results are shown in the Table 1:

Table 6 ACLR simulation result for 1PRB and 4PRB transmission BW

| Transmission BW(kHz) |  | 180kHz | 720kHz |
| --- | --- | --- | --- |
| CBW(kHz) |  | 200kHz | 800kHz |
| ACLR1 （without frequency offset） | Before filter | 22dB | 28dB |
|  | After filter | 31dB | 50dB |

Considering that there is no indoor NR UE, ACLR1/2 can be relaxed to 30/40dB separately.

Proposal 9: Specify 1 to 4 PRB transmission BW A-IoT carrier ACLR 1=30dB，and ACLR2=40dB for A-IoT BS with Foffset=100kHz.

Table 7: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200/400/600/800kHz | BWChannel | A-IoT of same BW | Square (BWConfig) | 30 dB |
|  | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |

Figure 4 Diagram of A-IoT ACLR with 100kHz Foffset

2.5.4	Transmitter spurious emissions

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 6.6.1-1. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329.

For A-IoT BS, the transmitter spurious emissions should meet the NR BS requirements specified in Table 6.6.5.2.1-1 and Table 6.6.5.2.1-2 of TS 38.104[6].

Copied from R4-2502859[4]

### Issue 1-5-4: Transmitter spurious emissions

- Agreement:

  - Use the FR1 NR BS transmitter spurious emission requirement for A-IoT BS as starting point.

  - FFS on the co-location requirements if the A-IOT BS is co-located with BS operating in other bands.

Proposal 10: The transmitter spurious emissions for A-IoT BS should meet the NR BS requirements specified in Table 6.6.5.2.1-1 and Table 6.6.5.2.1-2 of TS 38.104.

Proposal 11: Reuse the co-existence and co-location requirements of NR Micro BS.

2.5.4	Transmitter intermodulation

Copied from R4-2502859[3]

### Issue 1-6: Transmitter intermodulation

- Agreement:

  - FFS.

Since A-IoT BS and NR BS operating in the same band are not co-located based on R19 A-IoT WI scope, there’s no need to define transmitter intermodulation requirements.

Proposal 12: There is no need to define transmitter intermodulation requirement

- Ambient IoT BS Rx requirements

  - Reference sensitivity level

The conclusion of reference sensitivity level for A-IoT BS in SI is as follows: [2]

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

Copied from R4-2502859[4]

Issue 2-1-1: Reference sensitivity level

- Agreement:

  - Define the reference sensitivity level specifically for 15 kHz D2R channel BW at least

  - Consider the follow aspects

    - Noise figure=10dB

    - IM=2dB

    - SNR

    - Coverage

  - FFS on the definition of channel bandwidth depending on RAN1 conclusions.

To determine the D2R modulation SNR, LLS is performed under the assumption in the following table.

Table 8: D2R LLS simulation parameters Applied

| Parameter | Value |
| --- | --- |
| Channel Model | AWGN |
| SFO | 105 ppm |
| Transmission bandwidth (double sideband) | 15kHz |
| TBS | 96 bits |
| FEC | 1/3 Convolutional Code |
| Line code | Manchester |
| Repetition | None |
| Carrier-wave waveform | Unmodulated single tone |

D2R demodulation SNR simulation result is as follows Figure 5:

Figure 5: D2R demodulation SNR simulation results

Proposal 13: The AWGN D2R modulation SNR is -6dB @ 10% BLER.

The impact of CW interference leakage on BS reference sensitivity depends on both the leakage level and the BS's cancellation capability. Given the flexible nature of CW deployment, it is recommend to apply 6 dB sensitivity degradation limit.

Proposal 14: Use 6dB as the desensitivity target for reference sensitivity.

  - In-band selectivity and blocking

    - Adjacent Channel Selectivity

For ACS interference aggressor, the potential aggressor is outdoor NR UE.

Copied from R4-2502859[4]

### Issue 2-3: ACS

- Agreement:

  - Evaluate ACS requirements considering no legacy NR UE indoor assumption

Figure 6 CDF of interference level from outdoor NR UE

Based on simulation, the interference level from outdoor NR UE is -84dBm for case 2-1f, which is much lower than the -52dBm ACS interference level due to penetration loss.There is no need to define ACS requirement.

Proposal 15: No ACS requirement for A-IoT BS.

    - In-band blocking

RAN4 reached the following consensus in SI: In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

Copied from R4-2502859

### Issue 2-4-1: general blocking

- Agreement:

  - Postpone the blocking requirements discussion after RAN4 make conclusion on ACS requirements.

Issue 2-4-2: narrowband blocking

- Agreement:

  - Further discuss whether Narrowband blocking requirements is needed or not.

Similar with ACS situation, the interference level from outdoor NR UE is quite low for case 2-1f. To ensure the A-IoT BS performance, the in-band blocking requirement from NB-IoT standalone Medium Range BS can be reused for A-IoT BS

Proposal 16: Reuse the in-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Copied from TS36.104

Table 7.6.1.1-3c: Blocking performance requirement for Medium Range BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 24, 42, 43, 48, 54, 65, 66, 70, 103, 106 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |

…

Table 7.6.1.1-4: Interfering signals for blocking performance requirement for NB-IoT standalone operation

| NB-IoT channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 0.2 | ±7.5 | 5MHz E-UTRA signal |

  - Receiver intermodulation

    - general intermodulation

For general intermodulation, it is assumed that there is no NR interference indoors, so there's no need to define it.

Proposal 17: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

    - narrowband intermodulation

Copied from R4-2502859[4]

### Issue 2-7-2: Narrowband intermodulation

- Agreement:

  - Further discuss whether narrowband intermodulation requirement is needed or not.

Regarding the narrowband intermodulation requirement, it can reuse the requirement of NB Micro BS.

Copied from TS36.104

Table 7.8.1-6c: Narrowband intermodulation performance requirement for Medium Range BS for NB-IoT standalone

| Channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 0.2 | PREFSENS + 6dB* | -47 | ±340 | CW |
|  |  | -47 | ±880 | 5MHz E-UTRA signal, 1 RB** |
| Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.1-5c. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. Note***: If a BS RF receiver fails the test of the requirement, the test shall be performed with the CW interfering signal frequency shifted away from the wanted signal by 180 kHz and the E-UTRA interfering signal frequency shifted away from the wanted signal by 360 kHz. If the BS RF receiver still fails the test after the frequency shift, then the BS RF receiver shall be deemed to fail the requirement. |  |  |  |  |

Proposal 18: No need to define Narrowband intermodulation performance requirement for A-IoT BS.

  - In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density.

Copied from R4-2502859[4]

### Issue 2-8: ICS

- Agreement:

  - FFS

NB-IoT only defines ICS for in-band operation and does not define it for standalone operation.

No need to define ICS for A-IoT BS.

Proposal 19: There's no need to define ICS for A-IoT BS.

- Summary

In this contribution, we give our opinions of A-IoT BS requirements, and has the following observations and proposals:

For TX requirements:

Observation 1：With A-IoT BS Pout 38dBm and ACLR 20/30/40/50dB, negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed.

Proposal 1: Reuse the output power of Medium Range BS for indoor A-IoT BS, Prated,c,AC≤ 38dBm

Proposal 2: RE power control dynamic range is not needed for A-IoT BS since there is no RE level operations in an A-IoT R2D carrier.

Proposal 3: Total power dynamic range is not necessary for A-IoT BS since there are no PRB level operations in an A-IoT R2D carrier.

Proposal 4: For A-IoT BS, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

Proposal 5: A-IoT BS reuse the NR BS type 1-C NR requirements for Transmitter transient period, as shown in Table 3.

Table 2: Minimum requirement for the transmitter transient period for A-IoT BS

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

Proposal 6: Define RF envelop requirements, including modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

Observation 2：RAN1 is currently addressing CP handling methods, and it’s anticipated that the discontinuities caused by CP can be largely disregarded.

Observation 3：The time-domain waveform remains largely unchanged before and after filtering, indicating that the filter’s impact is negligible.

Proposal 7:  Consider RF impairments, the RF envelope requirements can be defined as Table 5.

Table 5: A-IoT BS RF envelope parameters

| Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- |
| Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
| RF Envelope Ripple | Mh = Ml | 0 |  | 0.15(A–B) | V/m orA/m |
| RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
| RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
| RF Pulsewidth | PW |  | 0.5Tari | 0.65Tari | µs |

Proposal 8:  The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

Observation 4: For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

Proposal 9: Specify 1 to 4 PRB transmission BW A-IoT carrier ACLR 1=30dB，and ACLR2=40dB for A-IoT BS with Foffset=100kHz.

Table 7: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200/400/600/800kHz | BWChannel | A-IoT of same BW | Square (BWConfig) | 30 dB |
|  | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |

Figure 4 Diagram of A-IoT ACLR with 100kHz Foffset

Proposal 10: The transmitter spurious emissions for A-IoT BS should meet the NR BS requirements specified in Table 6.6.5.2.1-1 and Table 6.6.5.2.1-2 of TS 38.104.

Proposal 11: Reuse the co-existence and co-location requirements of NR Micro BS.

Proposal 12: There is no need to define transmitter intermodulation requirement.

For RX requirements:

Proposal 13: The AWGN D2R modulation SNR is -6dB @ 10% BLER.

Proposal 14: Use 6dB as the desensitivity target for reference sensitivity.

Proposal 15: No ACS requirement for A-IoT BS.

Proposal 16: Reuse the in-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Proposal 17: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

Proposal 18: No need to define Narrowband intermodulation performance requirement for A-IoT BS.

Proposal 19: There's no need to define ICS for A-IoT BS.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.，RAN#107 Mar.,2025

[3]R4-2502859 WF on requirements for A-IoT BS and CW，Huawei ,RAN4#114, Feb. 2025

[4] R1-2501547, Session notes for 9.4 Solutions for Ambient IoT in NR, Ad-Hoc Chair (Huawei), RAN1 #120, Feb. 2025

[5] EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard - Specification for RFID Air Interface Protocol for Communications at 860 MHz - 930 MHz, Release 3.0, Jan 2024