3GPP TSG-RAN WG4 Meeting #114bis							         		R4-2503237

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Flexible RB extension for improved MPR when NW BW is larger than UE CBW

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.2.1 Power domain enhancements for single carrier [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In R4#114, we provided further details on our proposal to signal extended emission BW for UEs based on RB extension [1]. Additionally, it was agreed that the OOBE regions is relative to the UE CBW as captured in way forward [2]. In this contribution, we further explain the concept of a flexible extended bandwidth based on RB extension in fractions of the UE NRB and define the associated guard bands and extended bandwidth in relation to the permitted emission.

# Discussion

## 2.1 Way forward input

Several agreements related to the extended UE bandwidth were captured in way forwards [2]. They are summarized here in italics.

#### Issue 1-1-1: Boundary to apply SE: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW. This choice was made to meet the 270% CBW ITU recommendation.

Issue 1-1-2: Integral region basis for OOBE: The OOBE requirements are based on UE CBW

In our view, these Spurious Emission (SE) and OOBE requirements are only an indirect way to meet the 270% CBW ITU recommendation since this implies -30dBm/MHz is met at 200% CBW+5MHz. We agree to verify that all outer RBs can be turned into inner RBs when extended CBW/NRB is 2xUECBW/2xUENRB.

Issue 1-3-2: UE capability: MPR reduction with extended CBW is an optional feature with UE capability

Other topics are still open as captured in [2]

- A-MPR handling

- How to derive the extended IBE

- Width/RB numbers of extended CBW at each side of UE CBW

- How to get the width/RB numbers of extended CBW at each side of UE CBW

- Guard band for extended CBW

- Approaches of deriving the new inner RB region with RBs in extended CBW

- NW signalling.

In this contribution, we provide our proposals on a flexible RB extension approach and associated guard bands together with our views on the open topics.

## 2.2 Using CBW or NRB Extension with OOBE Region Based on UE CBW

In the RAN4#114 meeting it was agreed to base the OOBE range of the extended BW on the initial UE CBW such that the -30dBm/MHz SE range would be at 200% CBW + 5MHz, thus de facto covering the ITU recommendation of -30dBm/MHz at 270% transmission BW.

That being agreed, we need to verify that SEM and ACLR can be met with two options in terms of maximum BW extension that can turn any outer into inner and reduce their MPR:

- Maximum extension based on UE CBW/2.

- Maximum extension based on UE NRB/2.

To visualize the different cases and their potential MPR impact, several cases are illustrated (not to scale) in Figure 1:

- On the top two rows, the initial UE CBW situation is illustrated:

  - ACLR range (dark blue) with the 270% CBW limit first,

  - SEM and SE ranges in dBm/MHz next (for legibility, the first MHz part of the SEM is not shown).

- The next two rows show the same for the case where the UE CBW is extended by CBW/2 and ACLR region is defined based on UE CBW.

- The next two rows show the same for the case where the UE NBR is extended by NRB/2 and ACLR region is defined based on UE CBW.

- The next two rows show the same for the case where the UE NBR is extended by NRB/2 and ACLR region is defined based on the extended BW.

- Finally, the last two rows illustrate the IMD3 and IMD5 range for the inner and outer allocations respectively (IMD of the allocated RB range and its image).

Figure 1: UE inner and outer IMD3 and IMD5 ranges versus different BW extensions and OOBE ranges.

Observations:

- The emission bandwidth of a UE with an extended bandwidth is lower than the equivalent UE using the entire extended bandwidth. This provides additional margin on emissions and enables turning some/all outer allocations into inner allocations.

- Only the UE NRB/2 extended NRB (or CBW/2 extended CBW) using the extended BW OOBE range does not cover the 270% CBW by overlapping the -30dBm/MHz SE region.

- Outer IMD5 stay confined within the 270% CBW range.

- For the initial UE CBW situation:

  - Inner IMD3 range overlaps with ACLR1 and -10dBm/MHz SEM range

  - Inner IMD5 range overlaps with ACLR1 and -13dBm/MHz SEM range

  - Outer IMD3 range overlaps with ACLR1 and -13dBm/MHz SEM range

  - Outer IMD5 range overlaps with ACLR1 and -30dBm/MHz SEM range

- For the UE CBW/2 or NRB/2 extended case with UE CBW OOBE:

  - Inner IMD3 range overlaps with extended region and may be subject to IBE

  - Inner IMD5 range overlaps with ACLR1 and -10dBm/MHz SEM range

  - Outer IMD3 range overlaps with ACLR1 and -13dBm/MHz SEM range

  - Outer IMD5 range overlaps with ACLR1 and -25dBm/MHz SEM range

- For the UE NRB/2 extended case with extended BW OOBE:

  - Inner IMD3 range overlaps with extended region and may be subject to IBE

  - Inner IMD5 range overlaps with ACLR1 and -10dBm/MHz SEM range

  - Outer IMD3 range overlaps with ACLR1 and -13dBm/MHz SEM range

  - Outer IMD5 range overlaps with ACLR1 and -13dBm/MHz SEM range.

Based on the above observations, the NBR or CBW extension approach have the same behaviour:

- The -30dBm/MHz SE region happens before 270% CBW is reached.

- The outer IMD3 has a 3dB more stringent SEM requirement than the initial inner IMD3.

- The outer IMD5 has a 12dB more stringent SEM requirement than the initial inner IMD5. Note that with the OOBE region based on extended BW, the requirement would stay the same.

- The outer IMD3 have similar overlap with extended case ACLR than the inner IMD3/5 of the initial case.

With the above analysis, meeting ACLR for the extended case should not be an issue as the majority of the outer IMD3 fall within the extended region similar to the majority of the inner IMD3 falling in band. However, the 1RB0 IMD5 case is significantly more stringent than the initial case and does not benefit from the lower PSD of larger bandwidth allocations.

It should be noted that for PC2 and PC1.5, the SEM and SE requirement becomes 3dB and 6dB tighter relative to PCmax, thus the PC2 and PC1.5 cases require further verification given that the delta MPR is the same between inner and outer for QPSK for PC3, PC2 and PC1.5. Furthermore, edge allocations should also be handled but this is fairly simple since a simple RB extension of 2RB for PC2 and 4RB for PC1.5 would turn edge allocations into outer.

Proposal 1:

- To verify that all outer RBs can be turned into inner, the critical case to verify is the 15kHz SCS 1RB0 case with IMD3 < -13dBm/MHz and IMD5 <-25dBm/MHz.

- The fully allocated case may not be the worst case and, at cell edge, where MPR reduction is beneficial, full allocation is unlikely.

- PC2 and PC1.5 have more stringent SEM requirements (3dB and 6dB respectively) thus 1RB0 cases should be further verified for HPUE.

Given that the extension based on CBW/2 or NRB/2 is similar, and the ceil(NRB/2) extension on both sides allows to turning any outer allocation into inner, we suggest the BW extension is based on NRB. Thus, the main parameter to define the OOBE region based on NRB is which guard band should apply.

In Table 1, we present the analysis of all the FR1 channel bandwidths for 15kHz and 30kHz SCS.

- Under the SCS and CBW headers:

- The first two columns list all the channel BW for 15kHz and 30kHz SCS

- The next two columns are 2x and 1.5x the channel BW which correspond to the symmetrical and asymmetrical CBW/2 extension cases.

- The colour coding maps the extended BW to the closest higher existing CBW. Above 50MHz, the 15kHz SCS extended BW are mapped to 30kHz CBW, while above 100MHz, the 30kHz SCS extended BW cannot be further mapped but could be mapped to a contiguous CA BW using the nominal channel spacing.

- Under the GB header:

- The first column lists all the channel minimum guard bands for 15kHz and 30kHz SCS

- The second column lists the maximum of two consecutive CBW minimum guard-bands. In general, the minimum guard band increases with increasing CBW, but there are exceptions (e.g., 35MHz and 40MHz have lower guard bands than 30MHz)

- Under the NRB header:

- The first column lists the NRB per CBW and SCS

- The second column is ceil(NRB/2)

- The third column corresponds to a symmetrical NRB/2 extension = NRB+2*ceil(NRB/2)

- The fourth column corresponds to a symmetrical NRB/2 extension = NRB+2*ceil(NRB/2)

- The colour coding compares the resulting number of RBs to the NRB of the closest higher existing CBW.

  - Light green colour indicates that the extended number of RBs is 1RB less than the NRB of the closest higher existing CBW.

  - Dark green colour indicates that the extended number of RBs is >1RB less than the NRB of the closest higher existing CBW.

- Under the GBext header: the guard band associated with a symmetrical NRB/2 extension (2x) and asymmetrical NRB/2 extension (1.5x) using the approach explained later in this chapter.

- Under the BWext header: the extended BW associated with a symmetrical NRB/2 extension (2x) and asymmetrical NRB/2 extension (1.5x) using the NRB BW and its associated extended guard band.

Table 1: CBW and NRB based bandwidth extension for all channel bandwidths.

| SCS [kHz] | CBW [MHz] |  |  | GB [kHz] |  | NRB |  |  |  | GBext [kHz] |  | BWext [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1x | 2x | 1.5x | GB | max | NRB | NRB/2 | 2x | 1.5x | 2x | 1.5x | 2x | 1.5x |
| 15 | 3 | 6 | 4.5 | 142.5 | 242.5 | 15 | 8 | 31 | 23 | 342.5 | 242.5 | 6.28 | 4.64 |
|  | 5 | 10 | 7.5 | 242.5 | 342.5 | 25 | 13 | 51 | 38 | 312.5 | 312.5 | 9.82 | 7.48 |
|  | 7 | 14 | 10.5 | 342.5 | 342.5 | 35 | 18 | 71 | 53 | 382.5 | 382.5 | 13.56 | 10.32 |
|  | 10 | 20 | 15 | 312.5 | 382.5 | 52 | 26 | 104 | 78 | 452.5 | 382.5 | 19.64 | 14.82 |
|  | 15 | 30 | 22.5 | 382.5 | 452.5 | 79 | 40 | 159 | 119 | 592.5 | 522.5 | 29.82 | 22.48 |
|  | 20 | 40 | 30 | 452.5 | 522.5 | 106 | 53 | 212 | 159 | 552.5 | 592.5 | 39.28 | 29.82 |
|  | 25 | 50 | 37.5 | 522.5 | 592.5 | 133 | 67 | 267 | 200 | 692.5 | 552.5 | 49.46 | 37.12 |
|  | 30 | 60 | 45 | 592.5 | 592.5 | 160 | 80 | 320 | 240 | 825.0 | 712.5 | 59.265 | 44.64 |
|  | 35 | 70 | 52.5 | 572.5 | 572.5 | 188 | 94 | 376 | 282 | 965.0 | 825.0 | 69.625 | 52.425 |
|  | 40 | 80 | 60 | 552.5 | 712.5 | 216 | 108 | 432 | 324 | 925.0 | 825.0 | 79.625 | 59.985 |
|  | 45 | 90 | 67.5 | 712.5 | 712.5 | 242 | 121 | 484 | 363 | 885.0 | 965.0 | 88.905 | 67.285 |
|  | 50 | 100 | 75 | 692.5 | 692.5 | 270 | 135 | 540 | 405 | 845.0 | 925.0 | 98.905 | 74.765 |
| 30 | 5 | 10 | 7.5 | 505 | 665 | 11 | 6 | 23 | 17 | 665.0 | 665.0 | 9.64 | 7.48 |
|  | 10 | 20 | 15 | 665 | 665 | 24 | 12 | 48 | 36 | 805.0 | 645.0 | 18.92 | 14.28 |
|  | 15 | 30 | 22.5 | 645 | 805 | 38 | 19 | 76 | 57 | 945.0 | 785.0 | 29.28 | 22.12 |
|  | 20 | 40 | 30 | 805 | 805 | 51 | 26 | 103 | 77 | 905.0 | 945.0 | 38.92 | 29.64 |
|  | 25 | 50 | 37.5 | 785 | 945 | 65 | 33 | 131 | 98 | 1045.0 | 905.0 | 49.28 | 37.12 |
|  | 30 | 60 | 45 | 945 | 945 | 78 | 39 | 156 | 117 | 825.0 | 1065.0 | 57.84 | 44.28 |
|  | 35 | 70 | 52.5 | 925 | 925 | 92 | 46 | 184 | 138 | 965.0 | 825.0 | 68.2 | 51.36 |
|  | 40 | 80 | 60 | 905 | 1065 | 106 | 53 | 212 | 159 | 925.0 | 825.0 | 78.2 | 58.92 |
|  | 45 | 90 | 67.5 | 1065 | 1065 | 119 | 60 | 239 | 179 | 885.0 | 965.0 | 87.84 | 66.4 |
|  | 50 | 100 | 75 | 1045 | 1045 | 133 | 67 | 267 | 200 | 845.0 | 925.0 | 97.84 | 73.88 |
|  | 60 | 120 | 90 | 825 | 965 | 162 | 81 | 324 | 243 | 845.0 | 885.0 | 118.36 | 89.28 |
|  | 70 | 140 | 105 | 965 | 965 | 189 | 95 | 379 | 284 | 845.0 | 845.0 | 138.16 | 103.96 |
|  | 80 | 160 | 120 | 925 | 925 | 217 | 109 | 435 | 326 | 845.0 | 845.0 | 158.32 | 119.08 |
|  | 90 | 180 | 135 | 885 | 885 | 245 | 123 | 491 | 368 | 845.0 | 845.0 | 178.48 | 134.2 |
|  | 100 | 200 | 150 | 845 | 845 | 273 | 137 | 547 | 410 | 845.0 | 845.0 | 198.64 | 149.32 |

Observations:

- Except for new 3MHz and 7MHz 2xCBW (symmetrical extension) up to 100MHz exactly maps onto an existing CBW.

- Most of the 1.5xCBW (asymmetrical extension) do not map exactly to an existing CBW (only 10 out of 23 do).

- NRB extension always results in at least one RB less the than the NRB of the closest higher existing CBW.

- Except for the 3MHz CBW, the extended BW based on RB extension and its associated extended guard band is smaller than the extended BW based on CBW. This means that the chosen guard band is already safe but if the extended channel edge was based on UE_CBW the margin would be even larger except for the 3MHz case.

Given the above, there is no real incentive to extend based on CBW/2 and extension based on NRB providing additional margin. That being said, it is essential that the UE and the BS have the same understanding about the OOBE regions. Thus, with NRB extension, it is essential to agree which guard band applies.

To make sure that the guard band is sufficient and thus guarantee that the emissions of any extended BW can be met by construction, it is important that it relates to the existing CBW that are measured in RAN5 and in certification, and note that the initial CBW is tested regardless, and no extra RB is transmitted even when the BW is extended. With proper construction of the guard band it should be feasible to guarantee any RB extension by measuring only the specified CBW as long as the extended BW is between two measured CBW. Then, the only additional case would be for the largest supported CBW.

Our proposal is that the extended RB BW inherit the guard-bands from the next corresponding existing higher CBW (CBW that uses the next higher NRB versus initial CBW NRB+NRBext).

- For 15kHz SCS extended NRB>270, the guard-band uses the corresponding 30KHz CBW.

- For 30kHz SCS extended NRB>273, the 100MHz guard band is used since this is what is applied in CA.

- The GBext and Bwext values in Table 1 are calculated based on this approach.

This approach is illustrated in Figure 2, for symmetrical and asymmetrical ceil(NRB/2) extensions of 5MHz and 10MHz channels.

Figure 2: guard-band mapping for 5MHz and 10MHz CBW symmetrical and asymmetrical ceil(NRB/2) extensions.

Explanation:

- The initial 5MHz channel NRB=25, a minimum guard band (GB)=242.5kHz and a ceil(NRB/2)=13

  - When extended symmetrically, NRB=51 which corresponds to a closest higher CBW=10MHz with NRB=52 and GB=312.5kHz.

    - When this guard band is applied to the 51 RBs, the extended bandwidth (Bwext)=9.82MHz and can be used for start of the OOBE regions based on the initial 5MHz CBW.

  - When extended asymmetrically, NRB=38 which also corresponds to a closest higher CBW=10MHz with NRB=52 and GB=312.5kHz as the 7MHz NRB=35 does not work.

    - When this guard band is applied to the 38 RBs, Bwext=7.48MHz and can be used for start of the OOBE regions based on the initial 5MHz CBW.

- The initial 10MHz channel NRB=52 with GB= 312.5kHz and a ceil(NRB/2)=26

  - When extended symmetrically, NRB=104 which corresponds to a closest higher CBW=20MHz with NRB=106 and GB=452.5kHz.

    - When this guard band is applied to the 104 RBs, Bwext=19.64MHz and can be used for start of the OOBE regions based on the initial 10MHz CBW.

  - When extended symmetrically, NRB=78 which corresponds to a closest higher CBW=15MHz with NRB=79 and GB=382.5kHz.

    - When this guard band is applied to the 78 RBs, Bwext=14.82MHz and can be used for start of the OOBE regions based on the initial 10MHz CBW.

Based on this approach, any number of RB extensions can be associated with a guard band that is already verified with the existing channel BW applicable in the band of interest.

Proposal on NRB based extension and associated guard band:

- The maximum bandwidth extension is based on adding ceil(NRB/2) symmetrically or asymmetrically to the initial NRB

  - A symmetrically extended BW extended NRB (NRBext) = NRB+2*ceil(NRB/2)

  - An asymmetrically extended BW extended NRB (NRBext) = NRB+2*ceil(NRB/2)

- The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext.

  - Thus, the extended BW BWext=NRBext*12*SCS+2*GBext+1*SCS

- For 15kHz SCS extended NRB>270, the guard-band uses the corresponding 30KHz CBW>50MHz.

- For 30kHz SCS extended NRB>273, the 100MHz guard band is used since this is what is applied in CA.

  - It should be noted that the minimum guard-band reduces from 70MHz to 100MHz, thus the 100MHz guard band is safe for BWext>100MHz and is the guard band associated with contiguous CA using 100MHz CBW and nominal channel spacing.

  - This enables extended bandwidths for channels that are part of contiguous channels in CA or collocated channels that are SCS aligned.

- Based on this worst-case guard-band approach we propose that RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band.

- In all cases, it is the BS responsibility to ensure that the RB extension enables the use of the extended guard band to define the out of band emissions based on BWext and initial CBW based OOBE regions.

  - In cases where a nearby band needs to be protected by the UE, the BS ensures that the RB extension and corresponding guard band is set in accordance with the protected band region by making sure that the protected band is within the spurious emission range (-30dBm/MHz) of the extended BW OOBE region.

Based on this guaranteed guard band approach, the UE that has been tested for emissions, up to a maximum CBW can meet the emission requirements for any extended bandwidth between two supported CBWs when redefining its outer/edge allocations to inner allocations.

Verification of the concept with measurement of a minimum set of cases.

Although we could not perform a complete set of measurement, we had the time to verify a few QPSK allocations, a 20MHz channel bandwidth for PC2 as we observed HPUE is a more critical case due to a more stringent SEM requirement, and the measurement is based on a pass/fail of ACLR, SEM and SE at the allowed PC2 . The results can be found in the table below. The table can be decoded as follows:

- In the first three columns provide the allocation name with LCRB and RBstart, OFDM type, and Edge/Inner/Outer type:

  - The allocations in green are inner allocations.

  - The allocations in yellow can use inner MPR if the UE CBW is extended by ceil(NRB/2) on the lower side.

  - The allocations in orange can use inner MPR if the UE CBW is extended by ceil(NRB/2) on both sides.

- Next column is the measured output power which accounts for the PC2 inner MPR

- The next four columns under ACLR margin provides:

  - The measured ACLR margin vs 31dB based on the initial UE CBW case for the lower and higher adjacent channel.

  - The measured ACLR margin vs 31dB based on ceil(NRB/2) extension with UE CBW MBW case for the lower and higher adjacent channel.

  - A negative value is a fail

- The next four columns under ACLR margin provides:

  - The measured SEM and SE worst case margin for the initial UE CBW case for the lower and higher adjacent channel.

  - The measured SEM and SE worst case margin for ceil(NRB/2) extension with UE CBW MBW case for the lower and higher adjacent channel.

  - A negative value is a fail.

|  |  |  |  | ACLR margin |  |  |  | SEM/SE margin |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | UE CBW |  | ExtRB |  | UE CBW |  | ExtRB |  |
| Allocation | coding | type | Pout | low | high | low | high | low | high | low | high |
| 20M2C001RB000 | CP-OFDM | Edge | 24.5 | 8.4 | 27.8 | 9.9 | 28.7 | 3.1 | 21.0 | 11.4 | 21.0 |
| 20M2C001RB001 |  | Inner | 24.5 | 8.3 | 28.6 | 9.8 | 29.6 | 2.6 | 21.0 | 11.4 | 21.1 |
| 20M2C036RB000 |  | Outer | 24.5 | -5.9 | 15.6 | 12.1 | 27.0 | -5.7 | 16.9 | 13.7 | 20.3 |
| 20M2C036RB036 |  | Inner | 24.5 | 13.5 | 12.4 | 23.5 | 23.5 | 14.2 | 14.6 | 21.1 | 19.7 |
| 20M2C053RB000 |  | Outer | 24.5 | -5.8 | 11.5 | 12.7 | 21.1 | -5.8 | 13.3 | 12.3 | 18.7 |
| 20M2C053RB026 |  | Inner | 24.5 | 3.3 | 3.3 | 18.2 | 17.8 | 2.6 | 2.8 | 16.4 | 17.4 |
| 20M2C078RB000 |  | Outer | 24.5 | -8.5 | -2.8 | 5.8 | 11.3 | -4.8 | 0.3 | 6.7 | 10.6 |
| 20M2C078RB014 |  | Outer | 24.5 | -4.7 | -5.1 | 11.0 | 10.0 | -2.4 | -4.1 | 9.9 | 10.6 |
| 20M2C106RB000 |  | Outer | 24.5 | -9.2 | -8.5 | 0.5 | 0.3 | -5.4 | -5.4 | 2.1 | 1.7 |
| 20M2D001RB000 | DFT-s-OFDM | Edge | 26 | 4.7 | 28.1 | 6.8 | 30.6 | -0.9 | 19.0 | 11.2 | 18.8 |
| 20M2D001RB001 |  | Inner | 26 | 5.7 | 28.6 | 7.5 | 31.2 | -0.2 | 18.8 | 12.9 | 18.8 |
| 20M2D036RB000 |  | Outer | 26 | -5.3 | 16.1 | 12.2 | 27.2 | -4.2 | 17.1 | 14.7 | 18.7 |
| 20M2D036RB036 |  | Inner | 26 | 12.9 | 12.0 | 22.2 | 22.7 | 14.0 | 11.3 | 18.9 | 17.9 |
| 20M2D050RB000 |  | Outer | 26 | -7.6 | 11.7 | 10.8 | 20.7 | -5.8 | 11.8 | 11.3 | 16.4 |
| 20M2D050RB025 |  | Inner | 26 | 1.1 | 1.8 | 18.5 | 18.9 | -3.5 | -1.4 | 13.0 | 14.3 |
| 20M2D075RB000 |  | Outer | 26 | -7.9 | -3.2 | 4.2 | 13.3 | -6.5 | -2.8 | 1.8 | 10.1 |
| 20M2D075RB014 |  | Outer | 26 | -5.4 | -5.2 | 11.1 | 11.3 | -4.2 | -4.0 | 8.4 | 8.6 |
| 20M2D100RB000 |  | Outer | 26 | -7.1 | -6.2 | 0.6 | 2.1 | -4.3 | -4.3 | 0.9 | 0.9 |

Observations:

- All the OOBE are passed for the extended bandwidth ACLR, SEM and SE with the marginal cases being for full allocation.

- As expected, most the outer allocations fail the initial UE ACLR or SEM or SE

- The yellow allocations that can be turned into inner with ceil(NRB/2) extension on the lower side pass the initial UE ACLR/SEM/SE on the upper side and the extended ACLR/SEM/SE on the lower side. Thus, they are confirmed for MPR reduction.

- The orange allocations that can be turned into inner with ceil(NRB/2) extension on both sides fail the initial UE ACLR/SEM/SE but passes the extended ACLR/SEM/SE on both sides. Thus, they are confirmed for MPR reduction.

Conclusions:

- The MPR reduction concept is verified for both symmetric and asymmetric ceil(NRB/2) extension for PC2 on a 20MHz channel. It should also serve as a verification for PC3 as the ACLR is 1dB less and the SEN/SE requirement is 3dB less stringent.

- However, the PC1.5 case may require further verification as the SEM/SE margin is small for the fully allocated waveform and get a further 3dB tighter.

- Also the flexible extension concept will require further verification especially for ACLR.

## 2.3 Use of Flexible RB Extension

In the previous chapter, we focused our discussion on the case where the extended bandwidth is limited to RB-based extension of ceil(NRB/2) as it is the baseline and it is easier to explain the extended RB and associated guard band concept. However, we see added value in further enabling a flexible RB extension:

- First, the MPR reduction benefits the cell edge by allowing larger LCRB to transmitted with higher power. Any increase in size from the initial NRB/2 is of interest even if lower than 2xNRB or 3/2xNRB. For 20MHz for example, the inner is 53RB, the 2x extension is 106 RB and the 1.5x extension is 78RB, but increasing to 60RB is already of interest.

- Similarly, it is of interest to have more flexibility to place small RB allocation at cell edge anywhere within the channel and transmit with reduced MPR even at the channel edge.

- RedCap and eRedCap devices may be placed anywhere within a 100MHz channel and thus may have less than NRB/2 possible extension at least on one side.

- In the contiguous CA case, the two channels may be very different, (e.g., 100MHz+20MHz). In order for the 100MHz channel to benefit from the reduced MPR for more allocations, the extension needs to be 20MHz or NRB/5.

Figure 3 illustrates such cases:

- The BS NRB (NRB_NW in orange) is 32RB and the related GBNW (light orange) is higher than the maximum guard band of the next lower/next higher CBW, as related to the UEs extended bandwidths.

  - The related inner/outer/edge allocations are denoted “i”/”o”/”e” in the allocation triangle.

- UE1 has 8RB (NRB_UE1 in blue) at an offset of 4RB within the BS RBs.

- UE2 has 8RB (NRB_UE2 in brown) at an offset of 4RB within the BS RBs.

  - The UEs CBW-related inner/outer/edge allocations are in light green/yellow/red in the top triangle.

  - The UEs extended BW related inner/outer allocations are in dark green/orange in the bottom triangle.

- The required RB extension to be signalled to the UE by the BS is illustrated in the middle as follows:

  - RBextL_UE1 and RBextH_UE1 in blue for UE1.

  - RBextL_UE2 and RBextH_UE2 in brown for UE2.

Observation:

- UE1 can “turn” all its allocations to inner.

- UE2 only has several outer allocations left.

- Left and right RB extensions beyond NRB_UE/2, are not useful as all outer and edge allocations are already “turned” to inner.

- UEs in different regions of the higher BS bandwidth will have different left and right RB extensions.

- UEs placed towards the BS bandwidth edge only has an RB extension on one (opposite edge) side. This is the case for UEs that use one CC of a BS supporting intra-band contiguous CA.

- It is essential that RB or BW extension are not set to NRB/2 or CBW/2 only, as this is an exception that the UE would be positioned in the middle of the BS BW, and if this is the case, there would be other UEs on each side which would not be able to benefit from it.

Figure 3: Inner/outer allocations for two UEs within a larger BS BW before RB extension (top) and after (bottom)

However, we understand that having flexibility from 1 to NRB/2 with 1RB granularity is an unnecessary complexity. Also, to gain 1RB on the largest inner you need to extend by 2RB while the flexibility needed is proportional to the initial CBW. Thus, instead of our initial proposal where the extension would be in amount of RBs, beyond the 0 or ceil(NRB/2) baseline left and right extensions, we propose to add six other fractional NRB states ceil(N*NRB/K) that would then mean that 3 bits extension per side is sufficient.

Table 2 provides such additional fractions on top of the NRB/2 cases of Table 1. Before the last six columns under the NRB/ 3b header (standing for 3 bit NRB fractions), the columns have the same definition as in Table 1. The six additional columns provide the RB extensions based on the NRB fractions in the second row based on ceil(N*NRB/14).

Proposal on associated guard band for flexible NRB-based extension.

- Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext where NRBext=NRB_UE+ceil(KL*NRB/14)+ceil(KL*NRB_UE/14) with:

  - KL and KH being an integer from 0 to 7 coded on 3bit each

  - KL being the fractional NRB extension on the lower side of NRB_UE

  - KH being the fractional NRB extension on the upper side of NRB_UE

- This is the generalisation of the proposal for the NRB/2 extension.

Table 2: Addition of flexible NRB extension based on fractions of NRB.

| SCS [kHz] | CBW [MHz] |  |  | NRB |  |  |  | NRB/ 3b |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1x | 2x | 1.5x | NRB | NRB/2 | 2x | 1.5x | 6/14 | 5/14 | 4/14 | 3/14 | 2/14 | 1/14 |
| 15 | 3 | 6 | 4.5 | 15 | 8 | 31 | 23 | 7 | 6 | 5 | 4 | 3 | 2 |
|  | 5 | 10 | 7.5 | 25 | 13 | 51 | 38 | 11 | 9 | 8 | 6 | 4 | 2 |
|  | 7 | 14 | 10.5 | 35 | 18 | 71 | 53 | 15 | 13 | 10 | 8 | 5 | 3 |
|  | 10 | 20 | 15 | 52 | 26 | 104 | 78 | 23 | 19 | 15 | 12 | 8 | 4 |
|  | 15 | 30 | 22.5 | 79 | 40 | 159 | 119 | 34 | 29 | 23 | 17 | 12 | 6 |
|  | 20 | 40 | 30 | 106 | 53 | 212 | 159 | 46 | 38 | 31 | 23 | 16 | 8 |
|  | 25 | 50 | 37.5 | 133 | 67 | 267 | 200 | 57 | 48 | 38 | 29 | 19 | 10 |
|  | 30 | 60 | 45 | 160 | 80 | 320 | 240 | 69 | 58 | 46 | 35 | 23 | 12 |
|  | 35 | 70 | 52.5 | 188 | 94 | 376 | 282 | 81 | 68 | 54 | 41 | 27 | 14 |
|  | 40 | 80 | 60 | 216 | 108 | 432 | 324 | 93 | 78 | 62 | 47 | 31 | 16 |
|  | 45 | 90 | 67.5 | 242 | 121 | 484 | 363 | 104 | 87 | 70 | 52 | 35 | 18 |
|  | 50 | 100 | 75 | 270 | 135 | 540 | 405 | 116 | 97 | 78 | 58 | 39 | 20 |
| 30 | 5 | 10 | 7.5 | 11 | 6 | 23 | 17 | 5 | 4 | 4 | 3 | 2 | 1 |
|  | 10 | 20 | 15 | 24 | 12 | 48 | 36 | 11 | 9 | 7 | 6 | 4 | 2 |
|  | 15 | 30 | 22.5 | 38 | 19 | 76 | 57 | 17 | 14 | 11 | 9 | 6 | 3 |
|  | 20 | 40 | 30 | 51 | 26 | 103 | 77 | 22 | 19 | 15 | 11 | 8 | 4 |
|  | 25 | 50 | 37.5 | 65 | 33 | 131 | 98 | 28 | 24 | 19 | 14 | 10 | 5 |
|  | 30 | 60 | 45 | 78 | 39 | 156 | 117 | 34 | 28 | 23 | 17 | 12 | 6 |
|  | 35 | 70 | 52.5 | 92 | 46 | 184 | 138 | 40 | 33 | 27 | 20 | 14 | 7 |
|  | 40 | 80 | 60 | 106 | 53 | 212 | 159 | 46 | 38 | 31 | 23 | 16 | 8 |
|  | 45 | 90 | 67.5 | 119 | 60 | 239 | 179 | 51 | 43 | 34 | 26 | 17 | 9 |
|  | 50 | 100 | 75 | 133 | 67 | 267 | 200 | 57 | 48 | 38 | 29 | 19 | 10 |
|  | 60 | 120 | 90 | 162 | 81 | 324 | 243 | 70 | 58 | 47 | 35 | 24 | 12 |
|  | 70 | 140 | 105 | 189 | 95 | 379 | 284 | 81 | 68 | 54 | 41 | 27 | 14 |
|  | 80 | 160 | 120 | 217 | 109 | 435 | 326 | 93 | 78 | 62 | 47 | 31 | 16 |
|  | 90 | 180 | 135 | 245 | 123 | 491 | 368 | 105 | 88 | 70 | 53 | 35 | 18 |
|  | 100 | 200 | 150 | 273 | 137 | 547 | 410 | 117 | 98 | 78 | 59 | 39 | 20 |

As can be observed, these simple fractions from one to six 1/14 provide a uniform RB extension step from ceil(NRB/14) to ceil(NRB/2). All the RB extensions can be coded with 3 bits per extension side as shown in Table 3.

Table 3: coding of fractional NRB extensions

| code | BIN2DEC | RB extension |
| --- | --- | --- |
| 111 | 7 | ceil(7xNRB/14)=ceil(NRB/2) |
| 110 | 6 | ceil(6xNRB/14) |
| 101 | 5 | ceil(5xNRB/14) |
| 100 | 4 | ceil(4xNRB/14) |
| 011 | 3 | ceil(3xNRB/14) |
| 010 | 2 | ceil(2xNRB/14) |
| 001 | 1 | ceil(1xNRB/14) |
| 000 | 0 | ceil(0xNRB/14)=0 |

The above table can use the 3bit binary code directly to calculate ceil(code*NRB/14) and the NRB/2 extension or no extension correspond to all 1 or all 0 respectively.

Proposal on RB extension signalling:

- Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH in fractions of the NRB of the initial UE CBW NRB_UE using 3bits per side.

- Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW in the band.

- The height possible extensions per side are of ceil(K*NRB_UE/14) with K=0 to 7 signalled with 3bits per RBextL and RBextH

  - No extension is signalled with 000

  - Ceil(NRB/2) extensions is signalled with 111

- Inner and outer allocation can be redefined by the UE using the existing inner/outer allocation equations where:

  - NRB in the inner equations of TS 38.101-1 clause .2.2 is replaced by NRB_UE+RBextL+RBextH

  - RBstart in the inner equations of TS 38.101-1 clause .2.2 is replaced by RBextL+RBstart_UE

- A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered:

  - One where the UE is able to reassign outer/edge to inner, based on ceil(NRB_UE/14) granularity of signalled RB extension.

  - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs).

## 2.4 IBE in the Extended Region.

In our discussions in the last meeting, there was consensus that IBE must be met within the extended region. In our view, this is not an issue since IBE must be met by inner allocations. Nevertheless, while limiting the RB extension to NRB_UE/2, the extended region may not cover the entire BS RBs, but at the same time the UE with extended bandwidth will have less emissions in the extended region than a UE that would use the entire extended bandwidth.

So, it should be expected that the UE meets the IBE requirement within the extended RBs but not within the guard band at the edges of the extended bandwidth. It should be noted that the NRB extension approach does not retain the initial guard band as shown in the examples in Figure 2. Thus, an IBE requirement should apply to the extended RBs on each side of the initial UE CBW RBs. Since the RBs in the extended region are by definition not allocated, they should meet the requirement in the first row of Table 6.4.2.3-1: Requirements for in-band emissions in TS38.101-1:

In this equation, the first term depends on NRB and LCRB, while the second only depends on LCRB. For LCRB it is obviously the allocated RB within the initial UE CBW. The question is whether NBR should be the initial UE CBW NRB (NRB_UE) or the extended NRB:

NRBext=NRB_UE+RBextL+RBextH

In our view, there is no reason that the UE should have a more stringent IBE floor requirement (first term of the equation) within the extended RBs compared to the RBs within the UE initial NRB_UE. Thus, the equation applies to all RBs (initial and extended) but using NRB_UE (not NRBext) and LCRB_UE in the equations.

Proposal on IBE: The extended RBs have no allocation by definition and thus should meet the IBE requirement in the first row of Table 6.4.2.3-1 in TS38.101-1 where NRB is the initial UE CBW NRB and LCRB is the number of allocated RB within initial UE CBW NRB. The IBE requirement is not made more stringent within the extended RBs.

## 2.5 A-MPR Handling.

Because the A-MPR tables have no values when MPR>A-MPR, for outer allocations it is unclear whether the improved MPR allows to meet the additional emissions requirements especially if these are out of band.

Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction. This is because the A-MPR tables have no values when MPR>A-MPR and thus in case of an outer allocation it is unclear whether the improved MPR can apply.

# Conclusion

In this contribution, we further explained and refined our proposals from [1] based on last meeting discussions.

Proposal 1:

- To verify that all outer RBs can be turned into inner, the critical case to verify is the 15kHz SCS 1RB0 case with IMD3 < -13dBm/MHz and IMD5 <-25dBm/MHz.

- The fully allocated case may not be the worst case and, at cell edge, where MPR reduction is beneficial, full allocation is unlikely.

- PC2 and PC1.5 have more stringent SEM requirements (3dB and 6dB respectively) thus 1RB0 cases should be further verified for HPUE.

A limited set of PC2 20MHz cases were verified based on the NRB based extension proposed with the following observations and conclusions.

Observations:

- All the OOBE are passed for the extended bandwidth ACLR, SEM and SE with the marginal cases being for full allocation.

- As expected, most the outer allocations fail the initial UE ACLR or SEM or SE

- The yellow allocations that can be turned into inner with ceil(NRB/2) extension on the lower side pass the initial UE ACLR/SEM/SE on the upper side and the extended ACLR/SEM/SE on the lower side. Thus, they are confirmed for MPR reduction.

- The orange allocations that can be turned into inner with ceil(NRB/2) extension on both sides fail the initial UE ACLR/SEM/SE but passes the extended ACLR/SEM/SE on both sides. Thus, they are confirmed for MPR reduction.

Conclusions:

- The MPR reduction concept is verified for both symmetric and asymmetric ceil(NRB/2) extension for PC2 on a 20MHz channel. It should also serve as a verification for PC3 as the ACLR is 1dB less and the SEN/SE requirement is 3dB less stringent.

- However, the PC1.5 case may require further verification as the SEM/SE margin is small for the fully allocated waveform and get a further 3dB tighter.

- Also the flexible extension concept will require further verification especially for ACLR.

Proposal on NRB based extension and associated guard band:

- The maximum bandwidth extension is based on adding ceil(NRB/2) symmetrically or asymmetrically to the initial NRB

  - A symmetrically extended BW extended NRB (NRBext) = NRB+2*ceil(NRB/2)

  - An asymmetrically extended BW extended NRB (NRBext) = NRB+2*ceil(NRB/2)

- The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext.

  - Thus, the extended BW BWext=NRBext*12*SCS+2*GBext+1*SCS

- For 15kHz SCS extended NRB>270, the guard-band uses the corresponding 30KHz CBW>50MHz.

- For 30kHz SCS extended NRB>273, the 100MHz guard band is used since this is what is applied in CA.

  - It should be noted that the minimum guard-band reduces from 70MHz to 100MHz, thus the 100MHz guard band is safe for BWext>100MHz and is the guard band associated with contiguous CA using 100MHz CBW and nominal channel spacing.

  - This enables extended bandwidths for channels that are part of contiguous channels in CA or collocated channels that are SCS aligned.

- Based on this worst-case guard-band approach we propose that RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band.

- In all cases, it is the BS responsibility to ensure that the RB extension enables the use of the extended guard band to define the out of band emissions based on BWext and initial CBW based OOBE regions.

  - In cases where a nearby band needs to be protected by the UE, the BS ensures that the RB extension and corresponding guard band is set in accordance with the protected band region by making sure that the protected band is within the spurious emission range (-30dBm/MHz) of the extended BW OOBE region.

Proposal on associated guard band for flexible NRB-based extension.

- Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext where NRBext=NRB_UE+ceil(KL*NRB/14)+ceil(KL*NRB_UE/14) with:

  - KL and KH being an integer from 0 to 7 coded on 3bit each

  - KL being the fractional NRB extension on the lower side of NRB_UE

  - KH being the fractional NRB extension on the upper side of NRB_UE

- This is the generalisation of the proposal for the NRB/2 extension.

Proposal on RB extension signalling:

- Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH in fractions of the NRB of the initial UE CBW NRB_UE using 3bits per side.

- Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW in the band.

- The height possible extensions per side are of ceil(K*NRB_UE/14) with K=0 to 7 signalled with 3bits per RBextL and RBextH

  - No extension is signalled with 000

  - Ceil(NRB/2) extensions is signalled with 111

- Inner and outer allocation can be redefined by the UE using the existing inner/outer allocation equations where:

  - NRB in the inner equations of TS 38.101-1 clause .2.2 is replaced by NRB_UE+RBextL+RBextH

  - RBstart in the inner equations of TS 38.101-1 clause .2.2 is replaced by RBextL+RBstart_UE

- A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered:

  - One where the UE is able to reassign outer/edge to inner, based on ceil(NRB_UE/14) granularity of signalled RB extension.

  - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs).

Proposal on IBE: The extended RBs have no allocation by definition and thus should meet the IBE requirement in the first row of Table 6.4.2.3-1 in TS38.101-1 where NRB is the initial UE CBW NRB and LCRB is the number of allocated RB within initial UE CBW NRB. The IBE requirement is not made more stringent within the extended RBs.

Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction. This is because the A-MPR tables have no values when MPR>A-MPR and thus in case of an outer allocation it is unclear whether the improved MPR can apply.

# References

[1] R4-2501489 BW or RB extension approach for improving MPR when NW BW is higher than UE CBW, Skyworks Solutions Inc., R4#114

[2] R4-2503013 WF on Power domain enhancement, Huawei, HiSilicon, R4#114