3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503243

Wuhan, China, April 7th – 11th, 2025

# Title:	Further MSD clean-up for CA intra-band non-contiguous

# Agenda Item:	6.3.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution is a follow-up to previously agreed on clean-ups [1]. It is a proposal to remove the 0 dB REFSENS exception requirements (a.k.a. “ΔRIBNC”) for intra-band non-contiguous CA.

- Discussion

As part of the ongoing initiative to simplify RAN4 MSD requirements, some initial clean-up proposals have been agreed on to remove requirements for intra-band non-contiguous configurations with more than one secondary component carrier (SCC) [1]. We reproduce in Table 1 the status of Table 7.3A.2.2-1 “post clean-up” and observe that several REFSENS exceptions, also known as MSD or also as ΔRIBNC requirements, are specified as 0 dB. Such 0 dB MSD requirements have created confusion in RAN5 in the past [2], and several debates exist within RAN4 on how to respond.

To our understanding, ΔRIBNC specifies exceptions to the REFSENS requirements. When no REFSENS exception occurs, RAN4 does not specify 0 dB since, by definition, 0 dB means there is no exception. We believe that 0 dB was initially introduced in Table 7.3A.2.2-1 to distinguish one UL “1UL” configurations that lead to desense (like CA_n2(2A) configuration with Wgap=50 MHz) from other configurations where no desense occurs.

Table 1: Status of Table 7.3A.2.2-1 “post clean-up” and 0dB MSD requirements.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |

In our view, these requirements are not necessary and may still create confusion. Hence, we believe that the 0 dB ΔRIBNC requirements should be removed. For band combinations which enjoy no self-desense for all 1UL configurations (e.g., CA_n66(2A)), we propose a clarification in the core requirements to which REFSENS applies. For 1UL configurations where MSD may occur, a maximum of two MSD test points per band combination may be specified.

Observation 1: 1UL 0 dB MSD requirements for intra-band non-contiguous CA are not necessary and are not needed since an MSD is a REFSENS exception, and 0 dB MSD means there is no exception.

Observation 2: For band combinations that are not affected by self-desense for all supported 1UL configurations, there is no need to specify 0 dB MSD requirements. Corresponding entries in MSD tables should be removed.

Observation 3: For band combinations that may be affected by 1UL interference:

- 1UL configurations leading to no-desense are not needed and can be removed.

- For 1UL configurations that may lead to MSD, a maximum of two test points is sufficient to cover corner cases.

Proposal 1:

- Remove 0dB ΔRIBNC requirements from PC3 Table 7.3A.2.2-1 and PC2 Table 7.3A.2.2-1a.

- For band combinations with configurations that may lead to MSD, a maximum of two test points is allowed.

- For band combinations which are not affected by MSD for all supported UL-CA configurations, like CA_n66(2A):

  - Remove the MSD test point entry from Table 7.3A.2.2-1 and void Notes 1, 2, 3, and 4.

  - Remove the MSD test point entry from Table 7.3A.2.2-1a and void Notes 4, 5, 6, and 7.

  - Clarify in core requirements that REFSENS applies for band combinations not specified in Table 7.3A.2.2-1/Table 7.3A.2.2-1a.

- Consider updating the PRD rules accordingly.

These proposals are illustrated in Table 2 for PC3 requirements.

Table 2: Example of PC3 test point clean-up proposal for Table 7.3A.2.2-1

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: Void. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 NOTE 11: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

- Conclusion

This paper proposes further simplifying the 1UL intra-band non-contiguous CA MSD requirements by removing 0 dB MSD requirements.

Observation 1: 1UL 0 dB MSD requirements for intra-band non-contiguous CA are not necessary and are not needed since an MSD is a REFSENS exception, and 0 dB MSD means there is no exception.

Observation 2: For band combinations that are not affected by self-desense for all supported 1UL configurations, there is no need to specify 0 dB MSD requirements. Corresponding entries in MSD tables should be removed.

Observation 3: For band combinations that may be affected by 1UL interference:

- 1UL configurations leading to no-desense are not needed and can be removed.

- For 1UL configurations that may lead to MSD, a maximum of two test points is sufficient to cover corner cases.

Proposal 1:

- Remove 0 dB ΔRIBNC requirements from PC3 Table 7.3A.2.2-1 and PC2 Table 7.3A.2.2-1a.

- For band combinations with configurations that may lead to MSD, a maximum of two test points is allowed.

- For band combinations which are not affected by MSD for all supported UL-CA configurations, like CA_n66(2A):

  - Remove the MSD test point entry from Table 7.3A.2.2-1 and void Notes 1, 2, 3, and 4.

  - Remove the MSD test point entry from Table 7.3A.2.2-1a and void Notes 4, 5, 6, and 7.

  - Clarify in core requirements that REFSENS applies for band combinations not specified in Table 7.3A.2.2-1/Table 7.3A.2.2-1a.

- Consider updating the PRD rules accordingly.

- References

- R4-2502829 Cat F CR for TS 38.101-1-j00 Corrections to intra-band non-contiguous MSD, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, inc.

- R5-211609 Clarification on exception requirements for Intermodulation due to Dual uplink (IMD), 3GPP TSG-RAN5 Meeting #90-e, electronic meeting, Ericsson.