3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503244

Wuhan, China, April 7 – 11, 2025

# Title:	PA transients for LB-LB switching

# Agenda Item:	7.8.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution discusses if an additional transient period is needed to account for power amplifier ON to OFF / OFF to ON transients.

- Discussion

The following switching periods were agreed at meeting #114 in way forward (WF) [1]. We proposed “140us + FFS” to account for the RAN4 assumption of 140us PLL programming and settling time, and “FFS” to account for additional PA transients to account for ON / OFF transients.

It is necessary to ensure the PA is in “OFF” state when switching occurs to prevent presenting large VSWR while the PA is in an ON state, during which the PA may be transmitting at full power. Large and sudden VSWR variations while PA is transmitting at full power may generate high power spurs that are difficult to predict and control.

RAN4 specifies 10us transient period for ON/OFF time mask, which is small compared to 140us for PLL settling. PA OFF to ON may pull large amounts of current on the power supply lines and in certain cases may disturb the settling of the PLL. VCO pulling may also occur due to PA generating strong harmonics that may “fall” at the oscillating frequency, aka as injection pulling. However, considering these events are short compared to the PLL settling time, we believe it is possible to trigger these events with a timing sequence that prevents such effects. In summary, it is not necessary to consider an additional transient period to account for PA ON/OFF transients.

Proposal 1: Consider 140us for LB-LB DL switching.

- Conclusion

This paper discusses if additional transients are needed to account for ON/OFF transients.

Proposal 1: Consider 140us for LB-LB DL switching.

- References

- R4-2503016 WF on low-low band switching, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Apple.

- R4-2501988 NR Low-Band CA Switching, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks.