# 3GPP TSG RAN WG4 Meeting#114-bis	R4-2503245

# Wuhan, China, Apr 7 – Apr 11, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NR-NTN with bandwidth less than 5MHz

Agenda item:	7.10.3.6

Document for:	Discussion

# 1. Introduction

During the Rel-18 timeframe, RAN4 introduced requirements for NR with bandwidth less than 5MHz to enable use cases in spectrum-constrained scenarios. It has been noted that satellite deployments in L-band and S-band may face similar spectrum constraints. This has motivated the consideration of a Work Item (WI) in RAN4 to support bandwidth less than 5MHz for NR-NTN in FR1 [1]. RAN4 will begin discussing this work item at RAN#114-bis. In this paper, we present our perspectives on the scope of demodulation requirements for this WI.

2. Scope of Demodulation Performance Requirements

Examining the demodulation performance requirements for NR with bandwidth less than 5MHz, it is noted that requirements for 3MHz bandwidth were introduced. Consequently, both PBCH and PDCCH requirements were defined for NR operating with 3MHz bandwidth [2]. Additionally, it is recommended in the WID to develop this feature for NR-NTN based on the work done for NR support in dedicated spectrum less than 5MHz. The corresponding UE capability for this NR feature is provided below.

Based on the UE capability mentioned above, PBCH will consist of 12 PRBs, while CORESET0 will consist of 15 PRBs.

Observation 1: RAN4 has introduced PBCH and PDCCH demodulation requirements for NR with bandwidth less than 5MHz in FR1 FDD.

Observation 2: According to the UE capability for this feature, PBCH will include 12 PRBs, while CORESET0 will include 15 PRBs within a 3MHz 15 PRB BWP.

Proposal 1: Consider only 3MHz channel bandwidth with 15 PRB BWP for this WI.

Proposal 2: Discuss the feasibility of introducing PBCH requirements for NR-NTN in FR1 FDD with 12 PRBs for PBCH.

Proposal 3: Discuss the feasibility of introducing PDCCH requirements for NR-NTN in FR1 FDD with 15 PRBs for PDCCH CORESET0.

We note that extensive test cases to ensure coverage for demodulation processing may not be necessary, such as PDSCH requirements, since the demodulation algorithm for PDSCH processing remains unchanged. Additionally, the RTT in NTN transmission could be quite large. As a result, the CSI feedback may not accurately represent the channel condition due to the large propagation delay by the time it is applied for DL transmission.

Proposal 4: Don’t define PDSCH and CSI reporting requirements for NR-NTN with bandwidth less than 5MHz.

3. Conclusions

In this paper, we present our perspectives on the demodulation performance requirements for NR-NTN with bandwidth less than 5MHz in FR1. The following proposals have been made.

Observation 1: RAN4 has introduced PBCH and PDCCH demodulation requirements for NR with bandwidth less than 5MHz in FR1 FDD.

Observation 2: According to the UE capability for this feature, PBCH will include 12 PRBs, while CORESET0 will include 15 PRBs.

Proposal 1: Consider only 3MHz channel bandwidth with 15 PRB BWP for this WI.

Proposal 2: Discuss the feasibility of introducing PBCH requirements for NR-NTN in FR1 FDD with 12 PRBs for PBCH.

Proposal 3: Discuss the feasibility of introducing PDCCH requirements for NR-NTN in FR1 FDD with 15 PRBs for PDCCH CORESET0.

Proposal 4: Don’t define PDSCH and CSI reporting requirements for NR-NTN with bandwidth less than 5MHz.

4. References

[1] RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN,” 3GPP TSG RAN WG Meeting #104, Shanghai, China, June 17 – 20, 2024.

[2] TS 38.101-4, ‘User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)’, v18.6.0, Jan 2025.