# 3GPP TSG RAN WG4 Meeting#114-bis	R4-2503246

# Wuhan, China, Apr 7 – Apr 11, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NTN testing for NGSO

Agenda item:	7.10.4.2

Document for:	Discussion

# 1. Introduction

As we finalize the channel modelling for NGSO, it is crucial to initiate discussions on RAN4 requirements. It was agreed that once the channel modelling discussions are concluded, the requirements discussions will take place in their respective sessions, namely RF, RRM, and Demod sessions [1]. In this paper, we present our perspectives on approaching the RAN4 demodulation requirements for this WI.

2. Demodulation Requirements

2.1 Consideration of Fading Channel Model

During RAN#112, it was agreed to decouple the time-varying Doppler shift and propagation delay due to satellite mobility from traditional fading channel [2]. Additionally, it was decided that RAN4 would consider the Rel-17 NTN fading models. Therefore, it is crucial for RAN4 to discuss integrating the fading channel into the time-varying Doppler shift and propagation delay due to satellite mobility.

Proposal 1: Initiate discussions on the fading channel models alongside the current discussion on modelling the time-varying Doppler shift and propagation delay as part of the simulation assumptions.

Proposal 2: Consider only the Rel-17 NTN fading channel models for this WI.

2.2 Simulation Assumptions and Test Cases

We note that the scope of this WI is limited to FR1 only. To this end, our preference is to consider the Rel-17 test cases and simulation assumptions as a baseline.

Proposal 3: Adopt the Rel-17 NTN test cases and their associated simulation assumptions as the baseline for this WI.

2.3 Impact on RAN4 demodulation requirements

There were a few proposals submitted during RAN4#114 to initiate the discussions on the impact of the RAN4 demodulation requirements. The way forward is captured below.

Impact on RAN4 demodulation requirements

- Proposals

  - Option 1:

    - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

  - Option 2:

    - Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin

  - Option 3

    - Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

We note that there are still a few open issues regarding the TE-emulated channel model. Once the discussion on the channel model is concluded, RAN4 can begin simulation campaigns to analyze the impact on demodulation requirements from the TE-emulated time-varying Doppler shift and propagation delay.

Proposal 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyze the impact on demodulation requirements.

3. Conclusions

In this paper, we present our perspectives on the demodulation performance requirements for NTN testing for UEs supporting NGSO scenarios. The following proposals have been made.

Proposal 1: Initiate discussions on the fading channel models alongside the current discussion on modelling the time-varying Doppler shift and propagation delay as part of the simulation assumptions.

Proposal 2: Consider only the Rel-17 NTN fading channel models for this WI.

Proposal 3: Adopt the Rel-17 NTN test cases and their associated simulation assumptions as the baseline for this WI.

Proposal 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyze the impact on demodulation requirements.

4. References

[1] R4-2502314, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG4 Meeting #114, Athens, GR, Feb 7 – 11, 2025.

[2] R4-2413516, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG4 Meeting #112, Maastricht, NL, Aug 19 – 23, 2024.