# 3GPP TSG RAN WG4 Meeting#114-bis	R4-2503247

# Wuhan, China, Apr 7 – Apr 11, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on channel modeling for NTN testing for NGSO

Agenda item:	7.10.4.1

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to the channel modeling for NTN testing for NGSO [1]. In this paper, we share our perspectives on the remaining open issues.

2. TE Emulated Channel Model

2.1 Stopping criteria for the Newton-Raphson method to solve Kepler’s equation

Accuracy requirement to solve Kepler’s equation based on Newton-Raphson method

        - Accuracy requirement E-E0>ϵ

- Option 1: ϵ=10-8

Number of iterations to solve Kepler’s equation based on Newton-Raphson method

- Proposals

- Option 1: 4

- Option 2: Do not consider the number of iterations as a stopping criterion for Newton-Raphson

During RAN#114, it was discussed that an accuracy requirement and number of iterations can be considered to solve the Kepler’s equation based on the Newton-Raphson method. The proposed way forwards are presented below [1].

In this context, we observed during the alignment exercise that all companies required only 4 iterations to achieve ϵ=10-8 accuracy requirements. This indicates that robust accuracy can be achieved with just 4 iterations. Therefore, we propose using accuracy requirements as the stopping criterion for the Newton-Raphson method.

Observation 1: Robust accuracy can be achieved with 4 iterations.

Proposal 1: Do not consider the number of iterations as a stopping criterion for the Newton-Raphson method.

Proposal 2: Consider option 1 for the accuracy requirement (𝝐=10-8).

2.2 Interpolation method for the Doppler shift and Propagation delay

Numerical method

- Option 1: cubic spline interpolation

- Option 2: linear interpolation

- Option 3: calculate propagation delay and Doppler shift at any time instant

- Other options are not precluded

As specified in Step 3 of Issue 1-1-1 [2], the estimated Doppler shift and propagation delay at times T0 and T1 will be interpolated over the T1-T0 period to obtain the estimated Doppler shift and propagation delay at the agreed-upon update periodicity. The proposed way forward is outlined below [1].

Below, we present results using linear interpolation over Keplerian estimates for a duration of 10 seconds for the Doppler shift and propagation delay in Figs. 1 and 2, respectively. For these results, we utilized the same initial ephemeris data and UE location assumed during the alignment exercise across companies. Linear interpolation was applied to the propagation delay and Doppler shift between 1sec Keplerian propagation estimates, with a 1msec interpolation interval.

Figure 1: Linear interpolation over the Keplerian estimates for the Doppler shift

Figure 2: Linear interpolation over the Keplerian estimates for the propagation delay

We observe a strong correlation between the Keplerian estimates and the linear interpolated values over this period. Additionally, the interpolation error due to linear interpolation over the Keplerian estimates for the Doppler shift and propagation delay is plotted in Figs. 3 and 4, respectively. Please note that relatively high interpolation error is noted before 200 seconds, as the propagation delay-rate and Doppler-rate are highest during this time. Furthermore, examining the interpolation error, we find that the mean absolute error for the Doppler shift is 0.0486Hz, and for the propagation delay, it is 2.122nsec. This low interpolation error suggests that linear interpolation is a viable option. Consequently, we believe that linear interpolation is a reasonable choice to maintain manageable computational complexity while ensuring good interpolation performance.

Figure 3: Doppler shift error due to linear interpolation with 1 msec interpolation interval

Figure 4: Propagation delay error due to linear interpolation with 1 msec interpolation interval

Observation 2: Linear interpolation achieves good performance while keeping computational complexity at a manageable level.

Proposal 3: Support Option 2 (linear interpolation).

2.3 Accuracy requirements for the Doppler and Propagation Delay

Accuracy requirement for the propagation delay and Doppler shifts

- Proposals

- Option 1

  - Max |delay-error|~5e-10 s

  - Max |delay-rate error|~1e-10 s/s

- Option 2

  - RAN4 to define a simulation assumption/TE requirement on interpolation accuracy when quantifying the NGSO demodulation for dynamic conditions so all test equipment can provide equivalent results

- Option 3

  - Do not consider accuracy requirements for the estimation of propagation delay and Doppler Shift

As part of our RAN4 contribution for RAN4#113, we shared our observations on the propagation delay and Doppler shift accuracy that can be achieved via cubic spline interpolation [2]. As shown below, it appears that these observations were captured in the way forward as the propagation delay and Doppler shift accuracy requirements [1].

However, these accuracies were simply observations from our side for a given interpolation technique and the intention was not to propose them as requirements.

Proposal 4: Support Option 3 (Do not consider accuracy requirements for the estimation of propagation delay and Doppler shift.)

2.4 Consideration of Earth rotation during signal flight time

It was discussed during RAN4#114 whether earth rotation during signal flight should be considered for the calculation of propagation delay. As presented in [3], Considering the earth rotation, we provided the following derived formula for the calculation of the propagation delay.

Let us assume that tR is the signal receive time and considering the same mathematical notations as in [1], the corresponding propagation delay considering the earth rotation, DelayERtR, can be formulated as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where rtRECEF and vtRECEF are the satellite position and velocity vectors at time tR, ρtRis the difference in satellite and UE position vectors at time tR (i.e., UE position vector – satellite position vector),  K is the unit vector in standard z-axis, (a×b) is the cross-product and <a,b> is the dot product between a and b, respectively.

Fig. 3: Propagation delay error due to ignoring Earth's rotation during signal flight time

As shown in Fig. 3, at 262 seconds (corresponding to a 10-degree elevation), the delay estimation error is 0.1493 µs, while at 128 seconds (corresponding to a 30-degree initial elevation angle), the delay estimation error is 0.075 µs. The maximum delay error could reach 0.5 µs. Considering Earth's rotation during signal flight time does not require an additional step, but rather an additional factor in the propagation delay calculation. The propagation delay error, caused by not accounting for Earth's rotation during signal flight time, could be significant in certain scenarios. It may lead to UL timing issues and affect PUSCH performance, ultimately impacting the downlink demodulation performance. Therefore, we believe that Earth's rotation during signal flight time should be considered in the calculation of propagation delay.

Observation 3: A significant error in propagation delay is noted when the Earth's rotation during signal flight time is not considered in the estimation.

Proposal 5: RAN4 should account for the Earth's rotation during signal flight time for more accurate estimations of propagation delay.

Proposal 6: Considering the earth rotation, the propagation delay, DelayERtR, can be formulated as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEF and vtRECEF are the satellite position and velocity vectors at time tR, ρtRis the difference in satellite and UE position vectors at time tR (i.e., UE position vector – satellite position vector), K is the unit vector in standard z-axis, (a×b) is the cross-product and <a,b> is the dot product between a and b, respectively.

3. Conclusions

In this paper, we present our perspectives on the performance requirements for NTN testing for UEs supporting NGSO scenarios. The following proposals have been made.

Observation 1: Robust accuracy can be achieved with 4 iterations.

Proposal 1: Do not consider the number of iterations as a stopping criterion for the Newton-Raphson method.

Proposal 2: Consider option 1 for the accuracy requirement (𝝐=10-8).

Observation 2: Linear interpolation achieves good performance while keeping computational complexity at a manageable level.

Proposal 3: Support Option 2 (linear interpolation).

Proposal 4: Support Option 3 (Do not consider accuracy requirements for the estimation of propagation delay and Doppler shift.)

Observation 3: A significant error in propagation delay is noted when the Earth's rotation during signal flight time is not considered in the estimation.

Proposal 5: RAN4 should account for the Earth's rotation during signal flight time for more accurate estimations of propagation delay.

Proposal 6: Considering the earth rotation, the propagation delay, DelayERtR, can be formulated as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEF and vtRECEF are the satellite position and velocity vectors at time tR, ρtRis the difference in satellite and UE position vectors at time tR (i.e., UE position vector – satellite position vector), K is the unit vector in standard z-axis, (a×b) is the cross-product and <a,b> is the dot product between a and b, respectively.

4. References

[1] R4-2502314, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG4 Meeting #114, Athens, GR, Feb 7 – 11, 2025.

[2] R4-2418438, “Discussion on NTN testing for NGSO,” 3GPP TSG RAN WG4 Meeting #113, Orlando, USA, Nov 18 – 22, 2024.

[3] R4-2500116, “Discussion on NTN testing for NGSO,” 3GPP TSG RAN WG4 Meeting #114, Athens, GR, Feb 7 – 11, 2025.