3GPP TSG-RAN WG4 Meeting #114bis		R4-2503250

Wuhan, China 7th – 11th April, 2025

Agenda Item:	7.17.2.2

Source: 		Qualcomm Incorporated

Title: 			Enhancement on FR2 CSSF

Document for:	Discussion

# 1.	Introduction

In this paper, we present our view on the enhancement of FR2 CSSF.

# 2.  Past Agreements

Following agreements happened during the last couple of meetings [1].

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing Agreement: - RAN4 to introduce new individual capability(s) for CSSF reduction in R19. - delay the capability detail discussion to the end of the core part. ### Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable

# 3. 	Discussion

## Design of Three Searchers for CSSF

RAN4 has already agreed to support three searchers in FR1 CA and FR1 + FR2 CA (PCell FR1) scenario. The benefits of three searchers for CSSF extend to NR-DC, as well. Hence, three searchers for CSSF should be assumed for NR-DC, as well.

RAN4 also discussed whether to introduce a network signalling for specific SCC to use the 3rd searcher. The advantages of specifically indicating a specific SCC to use for the 3rd searcher are not clear. Hence, RAN4 should just stick to the already agreed default design for the 3-searcher scenario where, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the 2nd searcher sharing.

Observation 1:  RAN4 has already agreed to support three searchers in FR1 CA and FR1 + FR2 CA (PCell FR1) scenario. The benefits of three searchers for CSSF extend to NR-DC, as well.

Observation 2: The advantages of specifically indicating a specific SCC to use for the 3rd searcher of CSSF are not clear.

Proposal 1: Support three searchers for CSSF in NR-DC of FR1 and FR1 + FR2.

Proposal 2:

- RAN4 does not introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

- RAN4 just keeps the already agreed default design for the 3-searcher scenario where, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the 2nd searcher sharing.

# 4.	Conclusion

Observation 1:  RAN4 has already agreed to support three searchers in FR1 CA and FR1 + FR2 CA (PCell FR1) scenario. The benefits of three searchers for CSSF extend to NR-DC, as well.

Observation 2: The advantages of specifically indicating a specific SCC to use for the 3rd searcher of CSSF are not clear.

Proposal 1: Support three searchers for CSSF in NR-DC of FR1 and FR1 + FR2.

Proposal 2:

- RAN4 does not introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

- RAN4 just keeps the already agreed default design for the 3-searcher scenario where, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the 2nd searcher sharing.

# 5.  Reference

[1] R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN3 #113.

[2] 3GPP TS 38.331 V 18.2.0

[3] R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, 3GPP TSG-RAN WG4 #114.