3GPP TSG-RAN4 Meeting #114bis	R4-2503252

# Wuhan, China 7th – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Prediction: Simulation results and core requirements

Agenda item:	7.19.2

Document for:	Discussion

# Introduction

In this paper, we focus on CSI prediction. We include simulation results based on the agreements of last meeting and address some issues that are needed to define work item related requirements

# Prior Agreements

Following agreement happened during the last couple of meetings [1][5].

Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model. Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations Issue 1-4: Generalization Agreement: - Postpone this discussion until the requirement and test definition is clear

# Discussion

Figure: Rel-18 type II Doppler codebook

Rel-18 non-AI-ML CSI prediction tests assume Rel-18 Doppler codebook. Figure 1 shows how Rel-18 type II Doppler codebook works. RAN4 Rel-18 non-AI-ML CSI prediction tests assumed N4 = 1, i.e., UE had to report one predicted PMI in Rel-18 RAN4 tests.

Observation 1: RAN4 Rel-18 non-AI-ML CSI prediction tests assumed doppler domain basis N4 = 1.

Rel-18 also allowed UE to indicate that whether it only supports N4 = 1 or if UE can support N4 > 1 [6]. This can be seen through the UE capabilities mentioned in Table 3.

Table 1: Subset of layer-1 feature list for NR_MIMO_evo_DL_UL [6]

| Index | Feature group | Components |
| --- | --- | --- |
| 40-3-2-1 | Support of Rel-16 based Doppler CSI | ….. Support for the size of DD-basis, N4=1 … |
| 40-3-2-1a | Support of Rel-16 based doppler measurement with N4 > 1 | Support for the size of DD-basis, N4>1 … |

During the last meeting, RAN1 agreed to use Rel-18 Doppler codebooks for Rel-19 AI-ML based CSI prediction, too. Similar to Rel-18, RAN1 is expected to allow UE to indicate whether it can support N4 = 1 or N4 > 1 to report AI-ML based predicted CSI.

Hence, RAN4 should define AI-ML based CSI prediction tests for, at least, N4 = 1.

Observation 2: Rel-18 allowed UE to indicate via feature group 40-3-2-1 and 40-3-2-1a that it can only support doppler domain basis N4 = 1 in non-AI-ML CSI prediction.

- Rel-19 AI-ML CSI prediction may introduce a similar UE capability.

Proposal 1: RAN4 defines Rel-19 AI-ML CSI prediction tests for, at least, doppler domain basis N4 = 1.

# Simulation Results

## Baseline Simulation Assumptions

During the last meeting, RAN4 agreed to a diverse set of options to simulate the performance of AI-ML based CSI prediction [4]. This contribution focuses on only a subset of those options and includes simulation results with that subset.

Table 1 shows the subset of simulation assumptions that were used to generate result for this contribution.

Table 1: Simulation assumptions for AI/ML based CSI prediction of this contribution

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD |
| Carrier frequency | 2GHz |
| Bandwidth | 10MHz |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 50Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). |
| Rank per UE | Rank 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback overhead | Option4: PC7 |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback Step 2: SGCS between overall precoder vectors |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Number of samples | 235000 chunks. Channel gets refreshed at the beginning of each chunk. Each chunk consists of estimated channels of 10 CSI-RS symbols where each CSI-RS symbol is separated by CSI-RS periodicity in time domain. Since we predict CSI-RS based on 5 observations, each chunk consists of 5 samples, in total, in terms of CSI-RS prediction. 20,000 chunks get used for training. 3,500 chunks get used for testing. |

## Results

Table 2: SGCS results with sample-and-hold and AI-ML based prediction

| Structure | Layer | SVD, SGCS1 | PC7, SGCS2 |
| --- | --- | --- | --- |
| Sample and hold | 0 | 0.3127 | 0.297 |
|  | 1 | 0.1734 | 0.1662 |
| AI-ML based prediction | 0 | 0.9572 | 0.8653 |
|  | 1 | 0.9173 | 0.7788 |

The “sample and hold” based SGCS1 result of Table 2 was obtained by simply comparing the eigenvectors of the most recent channel sample with those of the next channel sample. Please note that each channel sample is spaced 5ms apart in time domain.

Ideal channel estimation was observed to obtain the SGCS of step 2 of both “sample and hold” and “AI-ML based prediction” results.

Table 2 shows that the SGCS of AI-ML based CSI prediction in step 1 and step 2 are significantly higher than those of “sample and hold” method. However, this particular AI-ML model was trained specifically for this particular channel profile (50Hz Doppler and the other properties that are mentioned in Table 1). Hence, it is possible that the trained AI-ML model is overfitting with respect to this channel profile. Further investigation will be performed to see how these results compare in a generalized setting. Besides, it remains to be seen how AI-ML based CSI prediction performs in different settings corresponding to the diverse set of assumptions of [4].

Also, the performance of AI-ML based CSI prediction depends on the complexity of the model. Further internal investigation will be performed regarding the trade-off between complexity and performance of AI-ML based CSI prediction. Hence, the SGCS results submitted to future meetings may vary from the ones shown in Table 2 based on the complexity analysis of the AI-ML model.

Observation 3: The SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 4: The AI-ML model of Table 2 was trained based on 50Hz Doppler.

- Further investigation will be performed to see how AI-ML based CSI prediction performs in different settings corresponding to the diverse set of assumptions of RAN4 114 agreements.

- More investigations are needed to see the performance of CSI prediction obtained via AI-ML model that was trained for a “generalized” setting (e.g., to tackle for 20Hz, 50Hz and 100Hz Doppler).

Observation 5: The performance of AI-ML based CSI prediction depends on the complexity of the model.

- The SGCS results submitted to future meetings may vary from the ones shown in Table 2 based on the complexity analysis of the AI-ML model.

# Conclusions

Observation 1: RAN4 Rel-18 non-AI-ML CSI prediction tests assumed doppler domain (DD) basis N4 = 1.

Observation 2: Rel-18 allowed UE to indicate via 40-3-2-1 and 40-3-2-1a that it can only support DD-basis N4 = 1 in non-AI-ML CSI prediction.

- Rel-19 AI-ML CSI prediction may introduce a similar UE capability.

Observation 3: The SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 4: The AI-ML model of Table 2 was trained based on 50Hz Doppler.

- Further investigation will be performed to see how AI-ML based CSI prediction performs in different settings corresponding to the diverse set of assumptions of RAN4 114 agreements.

- More investigations are needed to see the performance of CSI prediction obtained via AI-ML model that was trained for a “generalized” setting (e.g., to tackle for 20Hz, 50Hz and 100Hz Doppler).

Observation 5: The performance of AI-ML based CSI prediction depends on the complexity of the model.

- The SGCS results submitted to future meetings may vary from the ones shown in Table 2 based on the complexity analysis of the AI-ML model.

Proposal 1: RAN4 defines Rel-19 AI-ML CSI prediction tests for, at least, doppler domain basis N4 = 1.

# References

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- 3GPP TR 38.843 V 18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- R1 116bis Chairman’s notes, Hunan, China, April 2024.

- R4-2502970, Baseline simulation assumptions for AIML based CSI prediction, 3GPP TSG RAN WG4 #114.

- R4-2502856, WF on the requirements for AI/ML air interface, 3GPP TSG RAN WG4 #114.

- 38.822, v 18.0.0, UE feature list.