3GPP TSG-RAN4 Meeting #114bis	R4-2503254

# Wuhan China, 7th – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for Positioning Accuracy

Agenda item:	7.19.4

Document for:	Discussion

# Introduction

In this paper, we focus on RRM core requirements for positioning accuracy

# Prior Agreements

Following agreement happened during the last several meetings [1][5][8].

Issue 3-4: Report mapping for UL SRS-RSRP Agreement: - Reuse the report mapping for UL SRS-RSRP to Case 3a Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b. Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

The AI/ML WID shows the following priority for different cases of positioning [2]:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

# Discussion

## Case 2a and 2b

RAN plenary removed case 2a and 2b from the scope of AI-ML positioning in the latest WID of NR_AIML_air [9].

Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Hence, by default, RAN4 should not define any requirement for case 2a/2b.

Observation 1: RAN plenary removed case 2a and 2b from the scope of AI-ML positioning in the latest WID.

Proposal 1: RAN4 will not define any requirement for case 2a/2b.

## Case 1

UE based positioning with UE side model is a 1st priority item of AI/ML positioning. RAN4 accuracy tests were not defined for legacy (non AI-ML) UE based positioning algorithm with UE side model.

However, legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario [4]. It is expected that the same DL-TDoA will be used to report UE’s position with UE-sided AI/ML model, too.

-- ASN1START NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, ... } OPTIONAL, ..., [[ locationCoordinates-r17 LocationCoordinates OPTIONAL, -- Cond batch1 locationSource-r17 LocationSource-r13 OPTIONAL -- Cond batch2 ]] } -- ASN1STOP

Observation 2: UE based positioning with UE side model is a 1st priority item of AI/ML positioning.

Observation 3: Legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario.

RAN2 has recently agreed to LTE positioning protocol (LPP) based UE reporting of inferred location in case 1 [7]

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Step 6 of above RAN2 agreement allows network to collect UE’s inferred location in one report via LPP.

Observation 4: RAN2 has agreed to LTE positioning protocol (LPP) based UE reporting of inferred location for LCM of case 1.

Proposal 2: RAN4 defines requirements for case 1, i.e., UE-based positioning with UE-side model.

# Conclusions

Observation 1: RAN plenary removed case 2a and 2b from the scope of AI-ML positioning in the latest WID.

Observation 2: UE based positioning with UE side model is a 1st priority item of AI/ML positioning.

Observation 3: Legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario.

Observation 4: RAN2 has agreed to LTE positioning protocol (LPP) based UE reporting of inferred location for LCM of case 1.

Proposal 1: RAN4 will not define any requirement for case 2a/2b.

Proposal 2: RAN4 defines requirements for case 1, i.e., UE-based positioning with UE-side model.

# References

[1] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[2] RP-2423999, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN 105, Sep 2024.

[3] RP-242030, “Checkpoint for AI/ML PHY”, Ericsson.

[4] 37.355, “LTE Positioning Protocol”, V 18.3.0

[5] R4-2417212, “WF on requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting #112bis

[6] Chairman’s Notes, 3GPP TSG RAN WG1 #116-bis, April 2024

[7] Chairman’s Notes, 3GPP TSG RAN WG2 #127-bis, Oct 2024

[8] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 meeting #114.

[9] RP-250792, “Revised WID for NR_AIML_air”, 3GPP TSG RAN meeting #107.